3GPP TSG-RAN WG2 Meeting #129bis	R2-2502409

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.10.3

Source:	Nokia

Title:	Enhancements for network slicing (LS R3-250914)

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

In LS R3-250914 RAN3 asked RAN2 to consider if any enhancement is needed to address the following scenario:

•	A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.

This paper discusses this issue and proposes the necessary RAN2 enhancements.

# 2	Discussion

Rel-17 introduced network slice-based cell reselection, where the UE is provided with one or more network slice groups and associated frequency priorities. Based on network slice(s) the UE is interested in and on the configured prioritisation, it performs frequency selection and tries to camp on the frequency that the network prioritizes for the prioritized slice group. The goal is to avoid long delay and extensive signalling during the connection setup that may occur when redirecting the UE to the right frequency, if the one it camps on is not appropriate for the requested slice.

However, if these cell re-selection configurations are not correctly defined and optimised, this goal may not be achieved: it could happen that the UE is not able to connect to its required slice in the cell where it is camping based on the slice-specific frequency priorities provided by the network. The problems that the UE may encounter are, for example:

- Despite following the Network Slice AS Group (NSAG) priorities, the UE selects a cell where the needed slice is not supported or not allowed.

- Despite following the NSAG priorities, the UE still must be redirected at the connection setup.

- After following the frequency priorities of the highest priority NSAG of the UE, the UE is forced to revert to lower-priority NSAG configuration and repeat cell reselection process.

Observation 1: NSAG configuration is meant to decrease connection setup delay and related signalling. It may, however, be non-optimal that hinders to achieve the intended goals.

Therefore, it is important that the network gets some feedback from the UE about how well the slice-based cell reselection configurations are working and if the UE is experiencing some slice connectivity problems. More specifically, RAN3 requested RAN2 to support the scenario when “A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.”

Proposal 1: RAN2 should study how the UE should provide information that helps the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

There are two types of reports that the UE can collect in IDLE/INACTIVE mode when cell reselection is performed: Mobility History Information (MHI) and logged MDT report. The logged MDT measurements are either collected periodically or in event-based manner, but these measurements are not tightly connected to the changes of the cell where the UE camps. The UE collects mobility history information in IDLE/INACTIVE states and a new entry is created “Upon change of suitable cell” (see 5.7.9 of 38.331). This is very much suitable for the purpose of collecting information about slice-based cell reselection, which should be collected when there is a change in the cell where the UE camps on.

Proposal 2: Enhance the Mobility History Information to enable the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

The information collected in the current MHI, which is stored in the VisitedCellInfo, does not include any information how the UE selected the new cell:

VisitedCellInfo-r16 ::=  SEQUENCE {

visitedCellId-r16        CHOICE {

nr-CellId-r16            CHOICE {

cgi-Info                 CGI-Info-Logging-r16,

pci-arfcn-r16            PCI-ARFCN-NR-r16

},

eutra-CellId-r16         CHOICE {

cellGlobalId-r16         CGI-InfoEUTRA,

pci-arfcn-r16                PCI-ARFCN-EUTRA-r16

}

}                                        OPTIONAL,

timeSpent-r16            INTEGER (0..4095),

...,

[[

visitedPSCellInfoListReport-r17    VisitedPSCellInfoList-r17                   OPTIONAL

]]

}

Therefore, new enhancements in MHI are needed on how the UE used the sliced based cell reselection parameters to address the scenario that RAN3 requested.

Observation 2: New enhancements in MHI are needed on how the UE used the sliced based cell reselection parameters.

The short overview of the slice-based cell reselection is the following:

- The UE receives NSAG(s) and their priorities via NAS from the core network.

- The UE receives the cell reselection parameters for the NSAGs either in SIB16 or in dedicated RRC signalling.

- The UE tries to find a new suitable cell supporting the highest ranked NSAG(s) using the cell reselection parameters of the highest ranked NSAG(s).

- If step 3 fails, then the next highest ranked NSAG(s) are considered as the highest ranked NSAG(s), and step 3 is repeated with them.

- If there is no more NSAG to be used for cell reselection then the UE performs the normal cell reselection without considering any slice configuration.

Simply adding a flag that the highest rank NSAG ID was not used for cell reselection is not very meaningful as the NG-RAN has no information about the NSAGs and their priorities that are used by the UE, as this information is UE specific, and it is configured via NAS by the core network (see step 1 above).

Observation 3: A flag that the highest rank NSAG ID was not used for cell reselection does not help the NG-RAN to optimize the slice-based cell reselection parameters, as NG-RAN cannot know the NSAG IDs and their priorities in the UE.

If the above procedure is considered, then an important piece of information that the NG-RAN should know to be able to optimize the NSAG specific cell reselection parameters is (i) which NSAGs lead to cell reselection failure and (ii) which NSAG was the one that lead to a successful cell reselection (or that cell reselection happened without considering any NSAG). Therefore, we propose to log this information in the MHI.

Proposal 3: Enhance the Mobility History Information with all the NSAG IDs that did not lead to successful cell reselection and the NSAG ID (or the lack of any NSAG ID) that was used to select a new cell.

In the cell reselection procedure, the most time and energy consuming part is the scanning of frequencies for suitable cells. Considering a new NSAG ID (step 4 above) may not lead to the scanning of new frequencies, as the previous measurement results are supposed to be re-used. E.g., if the same frequencies are listed for two NSAGs with different priorities, then checking cells for both NSAGs does not cause a major delay or energy consumption in the UE. Therefore, including information about the scanned frequencies could also be very useful to the network for optimizing the parameters for cell reselection.

Proposal 4: Enhance the Mobility History Information with information about the scanned frequencies (e.g., a list of scanned frequencies) used during cell reselection.

Generally logging network provided parameters in UE reports is considered unnecessary, as it is assumed that the network knows what parameters have been provided to UEs. In this case this can be cumbersome for the network as (i) the dynamic slice provisioning may result in frequent changes in the slice specific network selection parameters (ii) the time between the use of the parameters and sending the report to the network can be significant (iii) the UE may receive UE specific parameters in dedicated signalling. Therefore, it is proposed that the UE can also include the used slice specific cell reselection parameters. Note that UEs are not supposed to include the full set of cell reselection parameters, as it is proposed to only send the parameters that have been used, e.g., frequency priorities for low priority NSAGs that have not been used are not proposed to be included in the report.

Proposal 5: The Mobility History Information can also include the used slice specific reselection parameters along with the NSAG IDs used during cell reselection.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: NSAG configuration is meant to decrease connection setup delay and related signalling. It may, however, be non-optimal that hinders to achieve the intended goals.

Proposal 1: RAN2 should study how the UE should provide information that helps the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

Proposal 2: Enhance the Mobility History Information to enable the network to identify the scenario when the UE did not manage to select a cell in the frequencies corresponding to the highest ranked NSAG(s).

Observation 2: New enhancements in MHI are needed on how the UE used the sliced based cell reselection parameters.

Observation 3: A flag that the highest rank NSAG ID was not used for cell reselection does not help the NG-RAN to optimize the slice-based cell reselection parameters, as NG-RAN cannot know the NSAG IDs and their priorities in the UE.

Proposal 3: Enhance the Mobility History Information with all the NSAG IDs that did not lead to successful cell reselection and the NSAG ID (or the lack of any NSAG ID) that was used to select a new cell.

Proposal 4: Enhance the Mobility History Information with information about the scanned frequencies (e.g., a list of scanned frequencies) used during cell reselection.

Proposal 5: The Mobility History Information can also include the used slice specific reselection parameters along with the NSAG IDs used during cell reselection.