3GPP TSG-RAN WG2 Meeting #129bis		R2-2502419

Wuhan, China, 7th – 11th Apr. 2025

Source:	vivo

Title:	Discussion on SRAP impact for baseline procedure

Agenda Item:		8.13.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 meeting[1], SRAP header format has been discussed and the following agreements have been reached:

Agreements:

In multi-hop L2 U2N relay, besides the agreed remote UE ID and BEARER ID, in addition, at least include D/C field in the SRAP PDU header.

In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for.

Agreement:

Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

This contribution will further discuss SRAP impact such as SRAP configuration from each UE perspective (remote UE, first/intermediate/last relay UE), and the need of additional format of SRAP header.

- Discussion

  - SRAP configuration

RAN2#128 has agreed that “Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.”[2]. It is also agreed that remote UE local ID and BEARER ID should be configured. Therefore, we think it is good to have explicit agreements on what should the gNB configure for SRAP parameters.

The below Figure 1 further shows how an end-to-end Uu radio bearer is mapped to/from the serving gNB from each UE perspective (different color represents different UE’s Uu radio bearer).

Figure 1. Example of different UE’s Uu radio bearer mapping

In accordance with the above illustration, it’s straightforward to have a bulk of proposals for each UE’s Uu radio bearer mapping configuration as follows:

Proposal 1: From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- Local identity of the Remote UE

- The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

Proposal 2: From an intermediate Relay UE perspective (including the first Relay UE), the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

- For its own Uu RRC singnalling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to Egress PC5 RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC signaling, if any

  - Local identity of each downstream intermediate Relay UE

  - Each downstream intermediate Relay UE’s SRB identity to Egress PC5 RLC channel mapping

Proposal 3: From the last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5/Uu RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC signaling

  - Local identity of each intermediate Relay UE

  - Each intermediate Relay UE’s SRB identity to Egress PC5/Uu RLC channel mapping.

  - Additional format to include remote UE L2 ID

There was an FFS in RAN2 #129 meeting about the additional format to include remote UE L2 ID:

In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for.

During the discussion, the potential use case for adding the remote UE L2 ID is to allow SRPA operation before the dedicated RRC configuration, when there may not be available bear mapping configuration and local ID configuration.

Observation 1: The potential use case for including remote UE L2 ID in SRAP header is to allow SRPA operation before dedicated RRC configuration for SRAP from gNB.

However, if one look at the SRB0 case in single-hop U2N relay, there is no SRAP header as well at the remote UE, and the SRAP header is added/removed at U2N relay UE[3]:

For data packet not corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the receiving part on the SRAP entity of PC5 interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of Uu interface. As an alternative, the receiving part may deliver SRAP SDUs to the transmitting part on the collocated SRAP entity. When passing SRAP SDUs, the receiving part removes the SRAP header and the transmitting part of the relay UE adds the SRAP header with the same SRAP header content as carried on the SRAP Data PDU header prior to removal. Passing SRAP SDUs in this manner is therefore functionally equivalent to passing SRAP Data PDUs, in implementation. The following specification therefore refers to the passing of SRAP data packets. - For UL data packet corresponding to SRB0, the receiving part on the SRAP entity of PC5 interface delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of Uu interface, and the transmitting part on the SRAP entity of Uu interface adds the SRAP header in accordance with clause 5.3.3. - For DL data packet corresponding to SRB0, the receiving part on the SRAP entity of Uu interface delivers SRAP Data PDUs to the transmitting part on the collocated SRAP entity of PC5 interface, and the transmitting part on the SRAP entity of PC5 interface removes the SRAP header in accordance with clause 5.2.2. As an alternative for handling DL data packet corresponding to SRB0 not shown in Figure 4.2.2-2 or Figure 4.2.2-3, the receiving part on the SRAP entity of Uu interface removes the SRAP header and delivers SRAP SDUs to the transmitting part on the collocated SRAP entity of PC5 interface.

Therefore, we think the legacy mechanism can be reused in the multi-hop relay case. The SRB0 traffic from remote UE can be transmitted to the first relay UE through a similar SL-RLC0, and the first relay UE is responsible for adding SRAP header for UL traffic and removing SRAP header for DL traffic. Therefore, we propose:

Proposal 4: No additional format to include remote UE L2 ID in SRAP header is needed.

Proposal 5: For SRB0 traffic at remote UE in multi-hop relay, the first relay UE is responsible for adding SRAP header for UL traffic and removing SRAP header for DL traffic.

- Conclusion

In this contribution, we discussed SRAP impact such as SRAP configuration from each UE perspective (remote UE, first/intermediate/last relay UE), and the need of additional format of SRAP header. We have the following Observations and Proposals:

For SRAP configuration:

Proposal 1: From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- Local identity of the Remote UE

- The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

Proposal 2: From an intermediate Relay UE perspective (including the first Relay UE), the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

- For its own Uu RRC singnalling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to Egress PC5 RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC signaling, if any

  - Local identity of each downstream intermediate Relay UE

  - Each downstream intermediate Relay UE’s SRB identity to Egress PC5 RLC channel mapping

Proposal 3: From the last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5/Uu RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC signaling

  - Local identity of each intermediate Relay UE

  - Each intermediate Relay UE’s SRB identity to Egress PC5/Uu RLC channel mapping.

For SRAP header format:

Observation 1: The potential use case for including remote UE L2 ID in SRAP header is to allow SRPA operation before dedicated RRC configuration for SRAP from gNB.

Proposal 4: No additional format to include remote UE L2 ID in SRAP header is needed.

Proposal 5: For SRB0 traffic at remote UE in multi-hop relay, the first relay UE is responsible for adding SRAP header for UL traffic and removing SRAP header for DL traffic.

- References

- RAN2 #129 meeting chair notes;

- RAN2 #128 meeting chair notes;

- 3GPP TS 38.351 V18.3.0 (2024-12).