3GPP TSG-RAN WG2 Meeting #129b                              R2-2502426

Wuhan, China, Apr. 7th – 11st, 2025

Agenda Item:	8.9.2

Source: 	Transsion Holdings

Title:  	Discussion on support of Store&Forward

Document for:	Discussion and Decision

Introduction

In the last RAN2#129 meeting, the support of Store&Forward for IoT-NTN was discussed, and the following agreements were made[1]:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

In this contribution, we provide our views on these issues to proceed the work item.

Discussion

Awareness of satellite Store&Forward mode change

- In last meeting it has been agreed that the time information can be broadcasted to indicate when the current satellite operation mode will transit from S&F operation mode to normal mode and the time information is provided in SIB31. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed. Whether to indicate the time information when the current satellite operation will transit from normal mode to S&F operation is discussed and no conclusion is made. From our point of view, when the satellite operation mode transit from normal mode to S&F mode, the network maybe need to release the UEs not supporting S&F mode and the service may be interrupted, from this point of view, the UEs not supporting S&F mode are not encouraged to access the network if the satellite will transit from normal node to S&F mode soon, so this time information is needed. Further more, there is no need to introduce new IE to indicate this, the time information provided in SIB31 as agreed in last meeting can achieve this. When the S&F operation indication is not broadcast, it means the satellite is work in normal mode, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from normal mode to S&F operation, when the S&F operation indication is broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from S&F operation to normal mode.

- Proposal 1: When the S&F operation indication is not broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from normal mode to S&F operation, while when the S&F operation indication is broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from S&F operation to normal mode.

- Proposal 2: If the remaining time from normal mode to S&F mode is not enough to finish the service, the UEs not support S&F mode may not triggered to access the network.

Paging enhancement in satellite Store&Forward mode

- As the satellite has already stored the downlink data for UE in advance, it means the network knows which UE needed to be paged and the paging demand is not random anymore and as the large coverage of the satellite, the number of UEs needed to be paged and the amount of downlink data needed to be send by the satellite maybe huge, which may cause paging congestion.

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 3: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- The satellite in S&F mode can distribute the paging message in time to avoid congestion from network’s perspective, however the UE has to monitor the PF/POs in every DRX cycle no matter whether there is a paging for the UE or not and there are no more paging message from the core network as there is no feeder link between the satellite and the ground, so the paging send by satellite and paging monitoring by UE can be optimized for power saving. For example there can be two DRX parameters which is different for S&F mode and normal mode, the DRX cycle or nB for S&F mode can be set larger than those in normal mode. Moreover if the UE keeps monitoring paging for some cycle or some times and no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

- Proposal 4: The paging send or monitoring can be optimized for power saving when network operates in S&F mode.

UP CIoT EPS optimisation support in satellite Store&Forward mode

For UP mode, when UE resume the RRC connection with a eNB, if the eNB is not the old eNB which suspend the connection, the eNB has to fetch the context from the old eNB. However for satellite in S&F mode, as the fast moving satellite and the no ISL between the satellite, the eNB the UE trys to resume may not the old eNB and can’t get the context from the old eNB, the UE has to fallback to the RRC connection setup procedure. To avoid this, it has to make sure that the eNB the UE resume has the UE context. To achieve this, the context can be transfer to specific satellites and UE only resume the connection to this satellites. As to how to decide these satellites, there can be two methods: one is the UE itself determines the satellites based on the satellite assistance information and the UE traffic pattern and send the satellite list to the network, then the netwrok confirms some or all of them and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered. The other method is the UE provide some assistant information such as UE location and traffic pattern to the network and the network determines the satellite list and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered.

- Proposal 5: When UP mode is used, the UE context can be transfered/distributed to specific satellites via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

When UE resume the RRC connection to a new eNB, the security key used by the new eNB is generated by the old eNB using the old key or NH、the target PCI and target EARFCN-DL. For S&F mode, if the satellite list the UE will resume are determined, the security key used by the new eNB in the satellite list in the future can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list. The generated security key are associated with each of the satellite in the satllite list and will be transfered to these satellites in advance via the ground network when the feeder link recovered.

- Proposal 6: The security key used by the new eNB can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list and transfered to these satellties via the ground network.

Conclusion

In this contribution we discussed issues related to S&F mode in IoT-NTN and made the following proposals:

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 1: When the S&F operation indication is not broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from normal mode to S&F operation, while when the S&F operation indication is broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from S&F operation to normal mode.

- Proposal 2: If the remaining time from normal mode to S&F mode is not enough to finish the service, the UEs not support S&F mode may not triggered to access the network.

- Proposal 3: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- Proposal 4: The paging monitoring can be optimized for power saving when network operates in S&F mode.

- Proposal 5: When UP mode is used, the UE context can be transfered/distributed to specific satellies via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

- Proposal 6: The security key used by the new eNB can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list and transfered to these satellties via the ground network.

Reference

- “Draft_RAN2_129_Meeting_Report_v3”