3GPP TSG-RAN WG2 Meeting #129bis	R2-2502432

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.13.3

Source:	Spreadtrum, UNISOC

Title:	Discussion on control plane for NR sidelink multi-hop relay

Document for:	Discussion and Decision

1 Introduction

In this contribution, we will continue discuss the QoS handling and notification message to support multi-hop L2 U2N relay.

2 Discussion

## 2.1 QoS aspects

1. Assistance information for QoS split

In RAN2#128 meeting, RAN2 discussed the QoS split and achieved the following agreements [1]:

QoS split

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

For remote UE end-to-end QoS split in multi-hop L2 U2N relay, the mechanism in single-hop L2 U2N relay is as baseline, i.e. gNB implementation to split PDB for each hop over multi-hop.

To help gNB to perform QoS split or adjust QoS split, UE(s) need to report measurements or other assistance information to the gNB. The existing SL measurement report and CBR measurement reports can be used by gNB to understand PC5 link conditions and determine QoS configuration.

Proposal 1: Remote UE and first relay UE report SL measurement and CBR measurement to gNB for QoS configuration determination.

2. QoS configuration

Once QoS split is done by gNB, it is necessary to convey related QoS configurations to remote UE and/or relay UE. As in single-hop L2 U2N relay, gNB directly configures remote UE and first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling. Moreover, QoS configuration for remote UE and relay UE (first and last) on PC5 hop are configured per PC5 RLC bearer.

Proposal 2: In multi-hop L2 U2N relay, gNB directly configures the remote UE, first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling.

Proposal 3: QoS configuration is configured per PC5 RLC bearer.

3. QoS flow control

One of the objective in the WID is to specific mechanism supporting QoS handling for multi-hop relaying. However, QoS is likely to be corrupted when network congestion occurs. Packet loss in that case causes TCP to slow down. Thus, some kind of flow control mechanism is required to avoid too packets to be pushed to the overloaded node. Such as, relay UE (first or last) can transmit flow control indication to remote UE for controlling traffic flow when relaying in UL, or relay UE (first or last) can transmit flow control indication to gNB for controlling traffic flow when relaying in DL.

Proposal 4: RAN2 to study flow control mechanism for multi-hop L2 U2N relaying.

4. Latency reduction

Another issue regarding QoS handling is the latency caused by multi-hop architecture. There is some concern about the overall latency of the QoS related procedures in control plane.

In IAB, the pre-emptive BSR is used by an IAB-MT to provide expected volume of data to its scheduler. In multi-hop L2 U2N relaying, similar mechanism can be introduced. The pre-emptive BSR might help to reduce UL scheduling latency by data buffering at the relay UE. First relay UE receives the data from remote UE and then sends to last relay UE. Last relay UE will triggers pre-emptive BSR.

Observation 1: Pre-emptive BSR is beneficial for multi-hop relay to reduce latency.

Proposal 5: Support pre-emptive BSR in multi-hop L2 U2N relaying.

## 2.2 Notification message

In single U2N and U2U, notification message is used by a Relay UE to send notification to the connected Remote UE on the other side link status.

Figure 1 notification message

In TS 38.331, relay UE can send a notification message if the following conditions are met.

##### 5.8.9.10.2 Initiation The Relay UE may initiate the procedure when one of the following conditions is met: 1> if the UE is acting as U2N Relay UE: 2> upon Uu RLF as specified in 5.3.10; 2> upon reception of an RRCReconfiguration including the reconfigurationWithSync; 2> upon cell reselection; 2> upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5; 1> if the UE is acting as L2 U2U Relay UE: 2> upon detection of PC5 RLF for the hop between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.3; 2> upon PC5-RRC connection release for the per-hop link between the L2 U2U Relay UE and L2 U2U Remote UE as specified in 5.8.9.5

The action upon receipting notification message at the remote UE is shown in following.

##### 5.8.9.10.4 Actions related to reception of NotificationMessageSidelink message Upon receiving the NotificationMessageSidelink, the Remote UE shall: 1> if the UE is acting as U2N Remote UE: 2> if the indicationType is included: 3> if the UE is L2 U2N Remote UE in RRC_CONNECTED: 4> if MP is configured and MCG transmission (i.e. direct path) is not suspended; 5> if the indicationType is relayUE-HO; 6> suspend indirect path transmission; 5> else: 6> initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure; 4> else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7; 3> else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE): 4> if the PC5-RRC connection with the U2N Relay UE is determined to be released: 5> indicate upper layers to trigger PC5 unicast link release; 4> else (i.e., maintain the PC5 RRC connection): 5> if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection: 6> consider cell re-selection occurs;

In multi-path U2N relay, the last relay UE should initiate the notification procedure as in single hop U2N relay. But in the first relay UE, it is a little bit different. When the first relay UE occurs relay reselection, it also should initiate the notification message to its connected remote UE with new indication type (i.e. relayUE-RelayReselection).

Proposal 6: When the first relay UE occurred relay reselection, it should send Notification message to its connected remote UE.

Moreover, the last relay UE, as illustrated in the following figure 2 for a 2-hop relay case, can directly detect the status of Uu link. While the Uu link failure (Uu RLF or RRC connection failure) or last relay UE performs HO/cell reselection, the last relay UE can send NotificationMessageSidelink to its connected first relay UE as in R17.

Figure 2 2-hop U2N relay

Upon receiving the NotificationMessageSidelink from the last relay UE, the first relay UE should initiate the RRC connection re-establishment procedure when it is in RRC_CONNECTED, then generate NotificationMessageSidelink with indicationType (e.g. relayUE-PC5-RLF, relayUE-Uu-RLF or relayUE-RRC-Restablishment) to its connected remote UE.

Proposal 7: Upon receiving NotificationMessageSidelink, the RRC_CONNECTED first relay UE will perform the RRC Reestablishment and then send NotificationMessageSidelink to its connected remote UE indicating the occurred RRC Reestablishment event.

Upon receiving the NotificationMessageSidelink from the last relay UE, the RRC_IDLE/INACTIVE first relay UE can freely decide whether to maintain or release the PC5 link with last relay UE. If first relay UE decides to release the PC5 link and perform relay reselection, then it sends NotificationMessageSidelink to its connected remote UE with relayUE-RelayReselection indication type.

If first relay UE decides to maintain the PC5 link and the indicationType indicated in the NotificationMessageSidelink is relayUE-HO or relayUE-CellReselection, first relay UE considers that a cell reselection has occurred, so the indicationType in the NotificationMessageSidelink sent to the Remote UE is relayUE-CellReselection.

Proposal 8: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE performs the PC5 link release and relay reselection, then it will send NotificationMessageSidelink to its connected remote UE indicating relay reselection.

Proposal 9: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE maintains the PC5 link, it will send NotificationMessageSidelink to its connected remote UE indicating cell reselection.

3 Conclusion

In this contribution we discuss the QoS handling and notification message for multi-hop relay, with the following proposals

Proposal 1: Remote UE and first relay UE report SL measurement and CBR measurement to gNB for QoS configuration determination.

Proposal 2: In multi-hop L2 U2N relay, gNB directly configures the remote UE, first relay UE and last relay UE for PC5 QoS configuration via Uu RRC signalling.

Proposal 3: QoS configuration is configured per PC5 RLC bearer.

Proposal 4: RAN2 to study flow control mechanism for multi-hop L2 U2N relaying.

Observation 1: Pre-emptive BSR is beneficial for multi-hop relay to reduce latency.

Proposal 5: Support pre-emptive BSR in multi-hop L2 U2N relaying.

Proposal 6: When the first relay UE occurred relay reselection, it should send Notification message to its connected remote UE.

Proposal 7: Upon receiving NotificationMessageSidelink, the RRC_CONNECTED first relay UE will perform the RRC Reestablishment and then send NotificationMessageSidelink to its connected remote UE indicating the occurred RRC Reestablishment event.

Proposal 8: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE performs the PC5 link release and relay reselection, then it will send NotificationMessageSidelink to its connected remote UE indicating relay reselection.

Proposal 9: Upon receiving NotificationMessageSidelink, if the RRC_IDLE/INACTIVE first relay UE maintains the PC5 link, it will send NotificationMessageSidelink to its connected remote UE indicating cell reselection.

- References

- Draft report of 3GPP TSG RAN WG2 #128, RAN2 chair (interDigtal), Orlando, USA, 18 - 22 November, 2024.