3GPP TSG-RAN WG2 #129bis	R2-2502444

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.6.2

Source: 	Ericsson

Title:  	Discussion on security for inter-CU LTM

Document for:	Discussion and Decision

# Introduction

Based on the reply-LS from SA3 we would like to describe our view on how the key material can be delivered to the UE, and how the basic security principles for RAN2 can look. This contribution proposes a way forward and discusses terminology for LTM.

# Discussion

## Security and delivery of security parameters

With the result of SA3 discussions as presented in the reply-LS (S3-251124) we can make the following observation:

SA3 allows for the NCC to be sent both as part of an RRC message as well as an unprotected MAC Control Element.

Given that SA3 allows for both RRC or MAC for the container of the NCC we can discuss the chain of actions that needs to happen for an inter-CU LTM Cell Switch and the similarities for the legacy procedures with L3 handover.

Let’s start off with the case in Step 1 below where the UE is in RRC_CONNECTED and has one or more inter-CU LTM candidates configured. The key in the UE and gNB1 is based on NCC=X and has been set up with legacy procedures. The NCC is an indicator known by the gNB1 in this case and the key (KgNB) has been generated previously based on the NH value that is being indicated by the NCC value X.

Step 1; UE is configured with one or more inter-CU LTM candidates

After some time, based on measurements, gNB1 decides to perform an LTM Cell Switch procedure to gNB2 which will eventually require a vertical key derivation since it is inter-CU. However, since no new key material from the AMF exist the new key has to be based on horizontal key derivation (see Step 2) which only uses local input (PCI and DL Freq).

At the time of cell switch, just like in legacy L3 HO, it is not possible to perform a vertical key derivation since the key must be retrieved from AMF. Instead, a horizontal key derivation needs to take place.

Step 2; UE performs horizontal key derivation to target cell.

Since the new KgNB is based only on local input, the UE will be in gNB2 with a horizontal key derivation-based security, and hence, just like in L3 HO it needs to perform vertical key derivation to maintain key separation between gNBs. To obtain a new NCC (to give to the UE to perform a vertical key derivation), the gNB2 performs the path switch procedure towards the AMF, see Step 3.

Step 3; Key material fetched through the path switch procedure

From the AMF, the gNB2 will receive new key material together with the NCC value which will tell the UE which NH value to use for the key generation as visualized in Step 4. Since the NCC is just a counter, for most cases it will be an increase of ‘1’ compared to the last NCC.

Step 4; New key material available at gNB2

Up until this point in the cell switch, we think there are no controversies and legacy procedures has been followed. Discussions in SA3 have earlier resulted in some solutions which prefetches security parameters to perform a vertical key change in the actual cell switch. However, these solutions are not necessary and would change security architecture in a way that is not possible to adapt to during the limited time left in release 19.

The Path Switch Procedure between the gNB and the AMF is the only existing method to fetch fresh security parameters.

Another observation we can make is that, since the key derivation at the cell switch is horizontal, from the UE perspective the actual cell switch is CU-agnostic, and the cell switch command can be used “as is” with no adaptations if the security parameters are not to be updated.

Based on the reasoning above, after the LTM Cell Switch, the vertical key derivation can take place. While there is no explicit enforcing of this according to 3GPP 33.501, it is up to the network to handle this:

3GPP TS 33.501, 6.9.2.3.2: Because the NGAP PATH SWITCH REQUEST message is transmitted after the radio link handover, it can only be used to provide keying material for the next handover procedure. Thus, for Xn-handovers key separation happens only after two hops because the source gNB/ng-eNB knows the target gNB/ng-eNB keys. The target gNB/ng-eNB can immediately initiate an intra-gNB-CU/intra-ng-eNB handover to take the new NH into use once the new NH has arrived in the PATH SWITCH REQUEST ACKNOWLEDGE message.

Since SA3 has approved both MAC CE and RRC for this RAN2 is free to go ahead with any preference here. However, regardless of what RAN2 chooses, a few things need to be considered.

In the example above, when the UE has arrived in gNB2, one of three things could normally happen from a mobility perspective:

- No new mobility procedure; The UE remains in the cell.

- The UE is subject to an intra-CU mobility procedure.

- The UE is subject to an inter-CU mobility procedure.

For the case 1 above, the new security parameters need to be applied in the current cell. For case 2 there could be room for allowing at the next mobility procedure to apply the new NH, NCC parameters, e.g. if the UE is subject to a subsequent intra-CU LTM cell switch it might be possible to include the security parameters in an LTM Cell Switch Command. This could be considered as a rare case, but it might be possible that the network wants to perform this from e.g. a load sharing perspective. For case 3, in theory it would be possible to continue executing horizontal handovers and request a new NH,NCC combination in each new source cell, but it seems not feasible from a practical perspective since a loop-around in the NCC values is at risk, and also security concerns would be hard to grasp.

For case 1 and 3 above, it would be needed to apply the security parameters immediately in the new source cell.

For case 2 above, it would be possible to apply the security parameters in an intra-CU mobility procedure.

As in legacy procedures, it is still under network implementation on when the security parameters are sent after an inter-CU LTM procedure when new security parameters are available at the (new source) gNB.

Furthermore, based on Observation 4 above, the introduction of NCC in a MAC CE that allows a mobility procedure at the same time, seems beneficial. With such a MAC CE, it would be possible to instead of doing a horizontal key derivation first, followed by a vertical key derivation in the target cell, do the vertical key derivation in the same step as the mobility procedure (if key material is present in the gNB).

However, the case where a vertical key derivation needs to be done in the target cell and the UE should remain in the same cell needs to be supported as well. This could be referred to as an intra-cell LTM cell switch procedure and could be used to trigger only the update of NCC value for vertical key derivation with the same MAC CE as for inter-cell LTM procedure.

Allowing LTM intra-cell switch can avoid multiple MAC CEs to be defined or having to use RRC to trigger the update of security parameters.

The introduction of such a MAC CE needs to cater for the case where only security parameters needs to be updated, but also the case where a cell switch takes place at the same time as vertical key derivation. For this reason, we think the MAC CE needs to contain the same information as the Rel-18 MAC CE, but with the addition of an NCC value. Also, to allow for the use of this new MAC CE also for Rel-19 UEs which should do intra-CU cell switch, there should be an indication that tells the UE if the NCC field should be applied or not, otherwise the UE doesn’t know when to use these bits.

The existing LTM cell switch MAC CE is baseline but enhanced by including the NCC value and an indication on whether the UE should perform security key change or not (this indication override the Rel-19 security IDs).

## Terminology for LTM and CLTM

According to the current running CR, there is still confusion on what terminology to use when describing things for LTM and CLTM. Since LTM and CLTM have a lot of similarities and share pretty much the same ASN.1 signaling and procedural text, the easiest way would be to use the term “LTM” by default for anything which is common for LTM and CLTM. In this case, the term “CLTM” needs to be used only for something that applies specifically for CLTM and not to LTM, such as for example the conditional execution conditions.

Use the term “LTM” by default for anything which is common for LTM and CLTM. Use the term “CLTM” for something which apply only to CLTM but not LTM.

# Conclusion

In the previous sections we made the following observations:

Observation 1	SA3 allows for the NCC to be sent both as part of an RRC message as well as an unprotected MAC Control Element.

Observation 2	At the time of cell switch, just like in legacy L3 HO, it is not possible to perform a vertical key derivation since the key must be retrieved from AMF. Instead, a horizontal key derivation needs to take place.

Observation 3	The Path Switch Procedure between the gNB and the AMF is the only existing method to fetch fresh security parameters.

Observation 4	For case 1 and 3 above, it would be needed to apply the security parameters immediately in the new source cell.

Observation 5	For case 2 above, it would be possible to apply the security parameters in an intra-CU mobility procedure.

Observation 6	Allowing LTM intra-cell switch can avoid multiple MAC CEs to be defined or having to use RRC to trigger the update of security parameters.

Based on the discussion in the previous sections we propose the following:

Proposal 1	As in legacy procedures, it is still under network implementation on when the security parameters are sent after an inter-CU LTM procedure when new security parameters are available at the (new source) gNB.

Proposal 2	The existing LTM cell switch MAC CE is baseline but enhanced by including the NCC value and an indication on whether the UE should perform security key change or not (this indication override the Rel-19 security IDs).

Proposal 3	Use the term “LTM” by default for anything which is common for LTM and CLTM. Use the term “CLTM” for something which apply only to CLTM but not LTM.

# References

R2-25xxxxx- Running RRC CR for inter-CU and conditional LTM, Ericsson, RAN2#129bis, April 2025.