3GPP TSG-RAN WG2 #129bis	R2-2502446

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.6.4

Source: 	Ericsson

Title:  	Discussion on Conditional LTM

Document for:	Discussion and Decision

# Introduction

This contribution proposes conclusion on the timing advance discussion and a problem that needs to be solved with regards to L2 reset and how the target DU can be aware of if it has been done.

# Discussion

## Timing Advance

During a previous RAN2 meeting there was an agreement taken with regards to TA values and if they should be kept or not when executing a conditional LTM cell switch:

RAN2#129: FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

On the one hand it could be argued that if the UE executes an LTM cell switch then it probably has moved since the execution conditions have been fulfilled and the UE has waited the duration of the TTT. However, on the other hand, for all TA values it is currently keeping, it might or might not move without its execution conditions being fulfilled, and the TA values are maintained by a timer. So, from this end the argument that additional TA values (for other candidate cells) should be released seems thin. Also, with regards to how LTM works for the non-conditional case (which could be seen as baseline), the network controls the validity of the TA values and they might be kept for a subsequent cell switch.

TA values for candidates that remain candidates after a conditional cell switch execution can still be used by the UE if their corresponding timers are running.

The next question that needs to be discussed is what these values can be used for. Locally in the UE they could serve a purpose of allowing the UE to perform RACH-less cell switch in a subsequent mobility procedure, for either conditional or non-conditional LTM. For conditional LTM it seems straight forward and not of much specification impact. However, for the conditional case, it is not an attractive solution to let the TA value be part of the LTM Cell Switch Command, since it would be needed to be sent to the gNB first. Instead, the LTM Cell Switch command can signal TA value ‘FFF’ and let the UE use a TA value similar to UE-based TA estimation. It is not suggested that the gNB should be aware of if the UE has a valid TA or not for this purpose.

The TA values of other candidates are not released after an LTM cell switch execution (triggered based on conditional execution conditions or not).

## L2 reset coordination

During non-conditional LTM cell switch the target DU will be aware of what the source cell is, and therefore the target DU can understand if the ltm-noResetID differs and thus if a L2 reset has been performed at the UE side.

For the conditional case however, there are no message to the DU serving the target cell since the execution is initiated by the UE. Since the source cell can be not only a certain cell, but one out of a number of candidates (and the candidate configuration does not depend on which the source cell is), the target DU needs to know if a L2 reset has been performed at the UE side to act accordingly.

When the UE has executed CLTM, the target DU needs to be aware if the UE has performed L2 reset or not.

To handle this issue, we see three possible options:

- Good Bye message; The UE tells the source DU which candidate cell it intends to switch to, before it executes the cell switch so the source DU can inform the target with the relevant information.

- Hello message; The UE tells the target in the first available grant what the reset ID is, or potentially if it has reset its MAC buffer or not, to let the target DU know the appropriate action.

- CU synchronization; The CU will know the configuration of all UEs and which have conditional configuration. It can continuously send this information to the DUs to let them know which is the current source cell for each UE. When a DU receives an incoming cell switch it will inform the CU so the CU can send an updated list to all DUs.

Option 1) above has already been discussed in RAN2 and there is no support to introduce it, mainly because the robustness aspects will be affected if the message doesn’t get through to the source. For this reason we don’t think it is a good option.

Option 2) would rely on either a new MAC CE with highest priority in the LCP procedure alternatively an indication in the UCI to let the target DU know if the buffer has been reset or not. It seems feasible but comes with some specification impact.

Option 3) sounds like a stable option since the CU will have information about current state of all candidate configurations. This falls within the RAN3 domain so we think it would be best to consult them with evaluating the feasibility for these options as RAN2s preferred choice.

Letting the CU informing the DUs pro-actively would mean the target DU has information about if a L2 reset has been performed or not.

For CLTM the CU shall update the DU(s) of other LTM candidates about the current serving cell for a UE at every LTM cell switch to allow the DU for the LTM candidate to understand whether an L2 reset needs to be done or not.

# Conclusion

In the previous sections we made the following observations:

Observation 1	TA values for candidates that remain candidates after a conditional cell switch execution can still be used by the UE if their corresponding timers are running.

Observation 2	When the UE has executed CLTM, the target DU needs to be aware if the UE has performed L2 reset or not.

Observation 3	Letting the CU informing the DUs pro-actively would mean the target DU has information about if a L2 reset has been performed or not.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The TA values of other candidates are not released after an LTM cell switch execution (triggered based on conditional execution conditions or not).

Proposal 2	For CLTM the CU shall update the DU(s) of other LTM candidates about the current serving cell for a UE at every LTM cell switch to allow the DU for the LTM candidate to understand whether an L2 reset needs to be done or not.

# References

R2-25xxxxx- Running RRC CR for inter-CU and conditional LTM, Ericsson, RAN2#129bis, April 2025.