3GPP TSG-RAN WG2 Meeting #129bis	R2-2502453

China, Wuhan, April 7th – 11th 2025

Source:                Qualcomm Incorporated

Title:	MRO enhancement for SON and MDT

Agenda Item:	8.10.4

Document for:	Discussion and Decision

# Introduction & Background

This contribution discusses NTN SON RAN2 issues based on RAN3 LS [1].

# Discussion

RAN3 sends LS[1]  to RAN2 regarding SON/MDT for NTN and ask RAN2 to consider the following two issues.

---------------------------------------------------------------------------------------------------------------------------------------

- Regarding SON/MDT for NTN:

On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not.

On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

---------------------------------------------------------------------------------------------------------------------------------------

For the second RAN3 agreement, currently MBS NTN is being discussed following agreements, the MBS service area can be presented by geographical area.

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

Since NTN MDT area So it should be assumed that the geographic area defined for MBS NTN can be used for MDT for NTN. But this needs to be further confirmed if any changes for MBS NTN. The MBS NTN area is defined for each MBS service, and the geographic area for logged MDT in case of NTN could be for non-MBS service and can be different from MBS NTN area. Therefore, a separate geographic area should be introduced for NTN MDT.

In NTN MBS, both location/radius and polygon-based methods are supported. And the encoding of Polygon in TS37.355 is reused for the geographical area of the polygon. For geographical area based logged MDT, RAN2 needs to discuss which one should be supported.

- RAN2 assumes  that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN.

- RAN2 should discuss whether to support reference location/radius or polygon-based or both.

In existing logged MDT, area scope can be provided to the UE via dedicated signaling, i.e. LoggedMeasurementConfiguration message. For NTN case, the same method can be reused, i.e. use dedicate signalling to provide area scope configuration to the UE.

- Use dedicated signalling to provide area scope configuration to the UE.

The UE may not support geographic area scope checking due to e.g. UE does not support location in IDLE/Inactive state, the a new UE capability should be introduced to indicate whether UE supports geographic area scope checking.

- Introduce a UE capability to support geographic area scope checking.

RAN3 is discussing whether both geographic area and/or the mapped cell ID are provided to the UE. If using dedicate signalling to provide area scope, then gNB can only configure one of them.

- gNB can only configure either mapped cell list or geographic area scope to the UE.

Geographic area scope checking relies on UE capability of obtaining location information. In some cases, UE may not obtain location information, e.g. UE cannot receive satellite signal, or user does not want to start location function due power saving. Only in case that the UE can obtain location information, the UE performs geographical area checking.

- Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT.

Another issue is, in case that the area scope is not configured to the UE or UE cannot obtain location information in IDLE/Inactive state if geographic area scope is configured, what UE should behave. There could be two options:

Option 1: The UE collects logged MDT data without checking area scope.

Option 2: UE does not perform logged MDT data collection.

For option 1, UE will continue to collect MDT data even though the UE moves out of the area scope, but the network does not know which measurement data are collected within the intended area scope. Then the collected data my bring wrong information to the network. For option 2, logged MDT data will not be collected by the UE. RAN2 should discuss which option should be used.

- RAN2 discusses which option is used in case that the area scope is not configured to the UE or UE cannot obtain location information in IDLE/Inactive state if geographic area scope is configured.

Option 1: The UE collects logged MDT data without checking area scope.

Option 2: UE does not perform logged MDT data collection.

# Conclusion

- RAN2 assumes  that the signaling structure agreed for encoding geographical area in case of MBS NTN can be used as a reference for encoding geographical area based logged MDT for NTN.

- RAN2 should discuss whether to support reference location/radius or polygon-based or both.

- Use dedicated signalling to provide area scope configuration to the UE.

- Introduce a UE capability to support geographic area scope checking.

- gNB can only configure either mapped cell list or geographic area scope to the UE.

- Only if the UE can obtain location information in IDLE/Inactive state, the UE performs geographical area checking for logged MDT.

- RAN2 discusses which option is used in case that the area scope is not configured to the UE or UE cannot obtain location information in IDLE/Inactive state if geographic area scope is configured.

Option 1: The UE collects logged MDT data without checking area scope.

Option 2: UE does not perform logged MDT data collection.

# Reference

[1] R2-2500026	LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT (R3-247908; contact: Samsung)	RAN3