3GPP TSG-RAN WG2 Meeting #129bis											R2-2502460

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.3.5

Source: 	Sharp

Title:  	Discussion on Specification-related Aspects for AI/ML-based Mobility

Document for: 	Discussion

# Introduction

In the past Rel-19 meetings, several aspects as described in the SID [1] related to AI/ML for Mobility have been discussed in RAN2. The focus has thus far been on the simulation assumptions and evaluation methodologies for various aspects related to RRM measurement predictions, using the scenarios as shown in the table below [2].

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Table 1: Prioritization of evaluation scenarios

Further, measurement event (e.g. for HO, RLF) predictions have also been considered with aspects related to indirect and direct predictions as shown in the figures below [2] for both study goals, i.e. reducing measurement efforts and improving handover performance.

Figure 1: Indirect measurement event prediction

Figure 2: Direct measurement event prediction

Since sufficient progress has been made with respect to simulations analyses and generalization, it was agreed during RAN2#129 that the relevant specification impacts will be considered from RAN2#129bis onwards. In this contribution, initial considerations on potential specification impacts for measurement event predictions are discussed.

# Discussion

Since it is agreed in the SID [1] that any decisions on handovers are made by the network, in case UE-sided AI/ML mobility models are used, it would be essential that the UE communicates some information on the model predictions to the network. Since predicted measurements inherently differ from legacy measurement results, it might require further considerations during the study on whether the existing measurement reporting frameworks are sufficient for UE-sided AI/ML models, even though the predicted results may be in the same format (RSRP) as the legacy measurements. This is required to enable the network to differentiate clearly between actual measurements and predicted measurements. Further, the measurement events (e.g. A3) corresponding to actual measurements and predicted measurement events (e.g. predicted A3 event) based on predicted measurement results would also require similar differentiation when being reported by the UE.

Observation 1: Measurement result predictions and measurement event predictions are inherently different from the corresponding legacy (or actual) measurement results, even though the two entities might represent the same quantities, e.g. RSRP and predicted RSRP or legacy A3 event and predicted A3 event.

Proposal 1: RAN2 to discuss the relationship between legacy measurement results and events, and the corresponding predicted measurements and events.

For legacy measurement events (e.g. A3) and corresponding reporting procedures, the measurement results obtained by the UE for its current serving cell and/or any detected neighboring cells are compared against parameters configured for the UE by the network. For example, the A3 event is specified [3] using the following measurement reporting conditions, where Mn and Mp represent the measurement results for the neighboring cell and SpCell respectively. The associated offsets (e.g. Ofn, Ocn) and hysteresis parameters are configured by the network.

Legacy A3 event (Neighbour becomes offset better than SpCell)

Inequality A3-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Inequality A3-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

With RRM measurement predictions (temporal and/or spatial domain), the UE may have predictions available for the SpCell and/or neighboring cell(s), which maybe utilized by the UE to evaluate the associated predicted measurement events. Therefore, considering legacy and predicted measurement results (and events) together for UE mobility requires precise definitions for UE measurement reporting.

Observation 2: RRM measurement predictions for SpCell and neighboring cells may be utilized by the UE to evaluate predicted measurement events.

Observation 3: Considering prediction results for UE mobility along with legacy measurement results requires clear definitions for the prediction-based measurement reporting criteria.

For example, the predicted A3 event may be formulated in relation to the legacy A3 event as follows.

Predicted A3 event (prediction of neighbour becomes offset better than prediction of SpCell), which utilizes predicted measurements, based on observed measurements.

Inequality A3-3 (Entering condition for predictions)

Pred(Mn) + Ofn + Ocn – Hys > Pred(Mp) + Ofp + Ocp + Off

Inequality A3-4 (Leaving condition for predictions)

Pred(Mn) + Ofn + Ocn + Hys < Pred(Mp) + Ofp + Ocp + Off

Similar definitions for measurement report triggering criteria related to other mobility events (A1, A2, etc.) may also be considered, whereby any prediction results available to the UE may be utilized appropriately for generating the measurement reports to be sent by the UE to the network.

Proposal 2: Predicted measurement events and respective conditions for measurement report triggering may be defined in relation to legacy measurement events (e.g. A3 event and predicted A3 event).

As agreed previously, the UE can be configured by the network to predict mobility events within a certain prediction window, based on the observed measurement results in the corresponding observation window. It was also agreed that the predicted mobility events need to follow similar rules as legacy events, such as the relevant Time-to-Trigger (TTT) to be observed. In general, the legacy measurement reporting criteria can thus be extended to also consider prediction-based information, when the necessary AI/ML functionalities are available/configured/activated to the UE.

Proposal 3: In general, the legacy measurement report events and the corresponding triggering criteria configurable to UEs can be extended to also consider prediction-based information when available, with similar parameters (TTT, hysteresis, offsets) as the legacy measurement events.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

In this contribution, we have the following observations:

Observation 1: Measurement result predictions and measurement event predictions are inherently different from the corresponding legacy (or actual) measurement results, even though the two entities might represent the same quantities, e.g. RSRP and predicted RSRP or legacy A3 event and predicted A3 event.

Observation 2: RRM measurement predictions for SpCell and neighboring cells may be utilized by the UE to evaluate predicted measurement events.

Observation 3: Considering prediction results for UE mobility along with legacy measurement results requires clear definitions for the prediction-based measurement reporting criteria.

In this contribution, we have the following proposals:

Proposal 1: RAN2 to discuss the relationship between legacy measurement results and events, and the corresponding predicted measurements and events.

Proposal 2: Predicted measurement events and respective conditions for measurement report triggering may be defined in relation to legacy measurement events (e.g. A3 event and predicted A3 event).

Proposal 3: In general, the legacy measurement report events and the corresponding triggering criteria configurable to UEs can be extended to also consider prediction-based information when available, with similar parameters (TTT, hysteresis, offsets) as the legacy measurement events.

# References

[1] RP-240082 Revised SID on AIML for mobility in NR.

[2] TR 38.744

[3] TS 38.331