3GPP TSG-RAN WG2 Meeting #129bis					           R2-2502462

Wuhan, China, 7 – 11 April, 2025

Agenda Item:		8.3.4

Source:			Huawei, HiSilicon

Title:			Discussion on specification impacts of AIML aided mobility

Document for:		Discussion and Decision

# 1   Introduction

RAN2 has performed several rounds of simulations for RRM prediction and measurement event prediction use cases and agreed on a number of observations which confirm the usefulness of AIML aided mobility, both in terms of potential measurement effort reduction and in terms of increased mobility performance. The SID objectives (see [1]) include also the following:

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: (…) - Study the need/benefits of any other UE assistance information for the network side model [RAN2] (…) - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] (…)

RAN2 discussed the priorities for further work in the remainder of Rel-19 and concluded the following:

- RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frequency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). - Continue with generalization across cell configurations for RRM prediction - Continue with measurement events - Inter-frequency measurement event prediction will be considered for the specification impact study but no explicit simulations will be required.

In this Tdoc, we provide our views on the specification impacts required to support the AIML aided mobility for the use case that were studied by RAN2 thus far. The document is divided into three parts which focus on general (model side agnostic) issues (section 2.1), UE-side model related issues (section 2.2) and NW-side model specific issues (section 2.3).

# 2   Discussion

## 2.1 General

Beam-level prediction

Beam level prediction is discussed in more detail in another paper that we co-sourced, see [2]. Here, we would like to just restate that feasibility of beam level prediction was confirmed in the past, since we already introduced sub-use case 1 for L3 cell level measurement prediction, i.e., L1 beam level measurement result(s) is predicted based on actual L1 beam level measurement result(s) and then L3 cell level measurement result is generated. This method can be reused to perform the L3 beam level measurement prediction by simply calculating L3 beam level measurement result instead of cell-level result in the post-processing.

Observation 1: The L3 beam level measurement prediction can be conducted by reusing sub-use case 1 for L3 cell level measurement prediction, i.e., L1 beam level measurement result(s) is predicted based on actual L1 beam level measurement result(s) and then L3 beam level measurement result is generated.

The motivation for using beam level predictions in the context of mobility is rather straightforward. During an existing HO procedure, the target gNB can configure dedicated RACH resources in handover command message. If the HO decision is supposed to be made based on predicted cell level measurements, in case predicted beam level measurement not provided in the measurement report at the same time, the target gNB is not able to configure the UE with optimal RACH resource for cell access. This can result in deteriorated HO performance in the end which stays in the opposition to the goal of this SI.

Observation 2: To achieve optimal HO performance, it is necessary for a target gNB to provide dedicated RACH resources in handover command for the UE to access the cell. In case of AI-aided HO decisions, L3 beam level measurement prediction results are required to achieve that.

Proposal 1: RAN2 should study an impact of L3 beam level measurement prediction on the specifications for the sake of using L3 beam level predictions to configure dedicated RACH resources for cell access by the UE.

Similarly, as for cell level prediction, all the three prediction domains, i.e. temporal, frequency and spatial domain, can be considered for L3-beam level prediction during the study.

Proposal 2: All the three prediction domains, i.e. temporal, frequency and spatial domain, can be considered for L3-beam level prediction during the study.

NR-DC scenarios

In the SID, there is a sentence that “Mobility use cases focus on standalone NR PCell change”. However, in our view, there is no need to preclude any actions to be made by the NW based on the predicted RRM measurement results. For example, after receiving some predicted measurement results, the NW may decide to configure an additional cell group or additional carrier which can be made based on gNB implementation. We also do not see the need to exclude the possibility of SCG configuring measurement predictions.

On the other hand, as the scope of Rel-19 AIML aided mobility does not include the impacts to RAN3, the MN-SN coordination should be avoided, i.e., the predicted RRM measurement results can be used independently by MN and SN.

Proposal 3: It should be possible that the configuration, triggering and reporting of predicted results are independently performed on MCG and SCG, as long as it does not require additional MN-SN coordination and does not impact RAN3, i.e. it can up to gNB implementation to use the predicted RRM measurement results in NR-DC.

## 2.2 UE-sided model

### 2.2.1 Model related issues

During the simulations, companies tested different kinds of models with different types of inputs etc. RAN2 needs to discuss whether such model related issues need to be specified or should be left up to UE implementation, taking several examples as below:

- Cluster-based vs. cell-based prediction

- L1 filtering option

- Utilized RRM sub-use case

- Direct vs. indirect prediction (only for measurement event prediction)

Companies had very differing views on which kind of model types, inputs etc. are best. From our perspective, what truly matters is the performance of the model. As long as proper prediction accuracy can be achieved, choices as above can be left to UE vendor discretion and we currently do not see the need to specify anything related to this.

Proposal 4: It is suggested that the model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, direct vs. indirect) can be up to UE implementation, as no requirement is identified to specify them for now.

### 2.2.2 Measurement framework impacts

For measurement framework, the major RAN2 impacts include measurement configuration and measurement reporting. Regarding the measurement configuration, some configuration to allow the UE to perform measurement predictions will be needed. In addition, in order to obtain signalling overhead reduction at gNB side, an impact on MG/SMTC configuration should be studied.

Regarding the measurement reporting, the gNB should be able to identify whether the measurement report is related to actual measurement or predicted measurement. This could be done based on an additional indication in the report or by specifying new measurement events and RAN2 should evaluate these different options. And further for the predicted measurement reporting, multiple measurement results (i.e., not only the measurement result at the end of TTT in legacy, taking an example that the PW length is longer than TTT length) corresponding to different timing may be needed. In general, RAN2 should study the impact of AIML aided mobility on measurement configuration and measurement reporting for both cell and beam level measurement prediction.

Proposal 5: RAN2 should study the impact of AIML aided mobility on measurement configuration and measurement reporting for both cell and beam level measurement prediction.

### 2.2.3 Functionality management

As shown in Fig.1, the LCM procedure for AIML interface was agreed to have five main steps. According to the SID, potential AI mobility specific enhancement should be based on the Rel-19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc.). Therefore, the basic framework of LCM for AIML interface should be reused.

Figure 1: LCM procedure agreed in AIML interface.

Proposal 6: The LCM framework from AIML interface should be reused for AIML aided mobility including supported functionality reporting via UE capability enquiry procedure and applicability reporting via UAI.

Regarding the detailed contents of UE capability reporting and applicable functionality reporting, they may vary for different use cases, (e.g., temporal domain, frequency domain, and spatial domain) and types of prediction (e.g., case A and case B, cell level and beam level prediction). Therefore, RAN2 should study them case by case.

Proposal 7: RAN2 to study the detailed contents of UE capability reporting and applicable functionality reporting for different use cases (e.g., temporal domain, frequency domain, and spatial domain) and prediction types (e.g., case A and case B, cell level and beam level prediction).

### 2.2.4 Performance monitoring

In RAN1#117 meeting [3], following agreements were reached for performance monitoring for BM use case.

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

In our view, both of the above options can be reused for AIML aided mobility. Regarding option 1, the reported measurement result can be L3 cell level measurement result or L3 beam level measurement result. Regarding option 2, RAN2 needs to discuss the performance metric(s) calculated at UE side, e.g., RSRP difference for RRM measurement prediction or F1 score for indirect measurement event prediction.

Proposal 8: The two options for performance monitoring agreed in AIML air interface can be reused for AIML aided mobility, i.e. NW-side and UE-assisted performance monitoring:

- For NW-side monitoring, the reported measurement result can be L3 cell/beam level measurement result.

- For UE-assisted monitoring, RAN2 needs to discuss the performance metric(s) calculated at UE side, e.g., RSRP difference for RRM measurement prediction, F1 score for indirect measurement event prediction.

### 2.2.5 UE-sided training data collection

There are three aspects that are discussed for UE-side data collection in RAN2 for AIML-aided air interface:

- Data transfer:  this aspect is still under discussion as part of the study item, i.e. see the bullet "CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:" in [5]. We should wait for the progress and conclusion before discussing the data collection of UE sided model for AIML aided mobility.

- UE-side data collection configuration (including UE request): the basic procedure is already agreed and once its available, the same principles can be used for mobility, e.g. messages to be used. However, what needs to be discussed specifically for mobility use case is the contents of the UE request and the details of data collection configuration.

- Associated ID: although it was introduced in AIML air interface, in our view, the need of associated ID for AIML aided mobility should be discussed, considering that AIML based mobility addresses another use case and relies on different assumptions.

Proposal 9: RAN2 should wait for the conclusion of UE-sided training data transfer solution which is being discussed as part of AIML for air interface study item.

Proposal 10: UE-side data collection configuration (including UE request) procedure from AIML for air interface WI can be reused as a baseline and RAN2 should discuss the details in WI phase, e.g., what needs to be requested by the UE and what the data collection configuration should contain.

Proposal 11: RAN2 should discuss whether associated ID is needed for AIML aided mobility.

## 2.3 NW-sided model

### 2.3.1 Assistance information for NW-sided inference

For the network to be able to perform inference and predict measurements, some information needs to be provided from the UE. The basic input that is required are RRM measurement results reported by the UE. For RRM measurement prediction, we have three sub-use cases, and depending on which one is used, either L1 beam level measurement results or L3 beam/cell level measurement results are needed to derive the predicted L3 beam/cell level measurement result. However, in current mobility measurement framework, only L3 cell/beam level measurement results can be included in the measurement report, while L1 beam level measurement result cannot be covered by current mobility measurement framework. Therefore, RAN2 should discuss what basic information is needed for NW-sided model inference, and further study whether current measurement reporting framework can be reused.

Proposal 12: RAN2 should analyse an impact of L1/L3 measurement results reporting from the UE to the network for the sake of NW-sided model inference.

When it comes to other potential assistance information, RAN2 considered mainly two things:

- UE speed: based on the model generalization study, we can conclude that in case the model is trained using UE’s with different speeds, the it generalizes well across UE speeds. Hence, knowledge about the UE speed does not seem useful.

- UE location information: Based on simulation results we provided to the previous meeting, see [4], UE location information does not help to improve prediction accuracy. Thus, we believe such information is also not useful and can be excluded.

Proposal 13: Based on the simulation results, it can be concluded that UE speed and UE location do not have to be provided to the network for NW-sided model inference.

### 2.3.2 NW-sided data collection

For the beam management use case, RAN2 agreed the following:

For NW-side data collection related to BM, RAN2 agreed on gNB-centric and OAM-centric approaches, and the same measurement framework is applied to both approaches. Immediate MDT is the baseline framework for OAM-centric data collection.

Even though an existing L1 beam level reporting mechanism could be reused to a large extent for NW-side data collection, RAN2 decided to develop a solution where the measurements are first logged and hen reported based on the NW request. This was justified by the fact that continuous measurement reporting is inefficient both in terms of signalling overhead and UE power consumption, as well as it would not support OAM-centric data collection without some further enhancements. We believe the same motivation holds for the AIML based mobility enhancement, so we believe the enhanced immediate MDT framework as developed within AIML for air interface WI should be used a baseline for NW-sided data collection for AIML aided mobility.

Proposal 14: The enhanced immediate MDT framework as developed within AIML for air interface WI should be used a baseline for NW-sided data collection for AIML aided mobility for both gNB-centric and OAM-centric data collection approaches, e.g. configuration and reporting procedures.

### 2.3.3 Model details

In general, the NW-sided model details can be up to gNB implementation, so this aspect does not have to discussed by RAN2. However, we would like to note that according to the current SID, only measurement event prediction using UE-sided model is studied. On the other hand, in the evaluations RAN2 focused on indirect measurement event prediction where the mobility event occurrence is determined based on the predicted RRM measurement results. Hence, there is nothing that prevents the gNB to perform measurement event prediction based on the predicted RRM measurements via implementation, without an additional specification work. Therefore, we suggest to capture in TR that measurement event prediction via indirect method can be performed via gNB implementation.

Proposal 15: Capture in the TR that measurement event prediction via indirect method can be performed via gNB based on implementation.

# 3   Summary

In the paper, the specification impacts on AIML aided mobility are discussed, and the following observations and proposals are provided:

UE-sided models – model implementation issues

Observation 1: The L3 beam level measurement prediction can be conducted by reusing sub-use case 1 for L3 cell level measurement prediction, i.e., L1 beam level measurement result(s) is predicted based on actual L1 beam level measurement result(s) and then L3 beam level measurement result is generated.

Observation 2: To achieve optimal HO performance, it is necessary for a target gNB to provide dedicated RACH resources in handover command for the UE to access the cell. In case of AI-aided HO decisions, L3 beam level measurement prediction results are required to achieve that.

Proposal 1: RAN2 should study an impact of L3 beam level measurement prediction on the specifications for the sake of using L3 beam level predictions to configure dedicated RACH resources for cell access by the UE.

Proposal 2: All the three prediction domains, i.e. temporal, frequency and spatial domain, can be considered for L3-beam level prediction during the study.

Proposal 3: It should be possible that the configuration, triggering and reporting of predicted results are independently performed on MCG and SCG, as long as it does not require additional MN-SN coordination and does not impact RAN3, i.e. it can up to gNB implementation to use the predicted RRM measurement results in NR-DC.

Proposal 4: It is suggested that the model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, direct vs. indirect) can be up to UE implementation, as no requirement is identified to specify them for now.

UE-sided models – measurement framework impacts

Proposal 5: RAN2 should study the impact of AIML aided mobility on measurement configuration and measurement reporting for both cell and beam level measurement prediction.

UE-sided models – functionality management

Proposal 6: The LCM framework from AIML interface should be reused for AIML aided mobility including supported functionality reporting via UE capability enquiry procedure and applicability reporting via UAI.

Proposal 7: RAN2 to study the detailed contents of UE capability reporting and applicable functionality reporting for different use cases (e.g., temporal domain, frequency domain, and spatial domain) and prediction types (e.g., case A and case B, cell level and beam level prediction).

UE-sided models – performance monitoring

Proposal 8: The two options for performance monitoring agreed in AIML air interface can be reused for AIML aided mobility, i.e. NW-side and UE-assisted performance monitoring:

- For NW-side monitoring, the reported measurement result can be L3 cell/beam level measurement result.

- For UE-assisted monitoring, RAN2 needs to discuss the performance metric(s) calculated at UE side, e.g., RSRP difference for RRM measurement prediction, F1 score for indirect measurement event prediction.

UE-sided models – training data collection

Proposal 9: RAN2 should wait for the conclusion of UE-sided training data transfer solution which is being discussed as part of AIML for air interface study item.

Proposal 10: UE-side data collection configuration (including UE request) procedure from AIML for air interface WI can be reused as a baseline and RAN2 should discuss the details in WI phase, e.g., what needs to be requested by the UE and what the data collection configuration should contain.

Proposal 11: RAN2 should discuss whether associated ID is needed for AIML aided mobility.

NW-sided models – assistance information

Proposal 12: RAN2 should analyse an impact of L1/L3 measurement results reporting from the UE to the network for the sake of NW-sided model inference.

Proposal 13: Based on the simulation results, it can be concluded that UE speed and UE location do not have to be provided to the network for NW-sided model inference.

NW-sided models – training data collection

Proposal 14: The enhanced immediate MDT framework as developed within AIML for air interface WI should be used a baseline for NW-sided data collection for AIML aided mobility for both gNB-centric and OAM-centric data collection approaches, e.g. configuration and reporting procedures.

NW-sided models – model implementation issues

Proposal 15: Capture in the TR that measurement event prediction via indirect method can be performed via gNB based on implementation.

# 4   References

- RP-240082 Revised SID on AIML for mobility in NR

- R2-2501789	Way forward on study of beam-level prediciton	vivo, Ericsson, ZTE, Huawei, HiSilicon, CATT, NTT DOCOMO, CMCC, Samsung, Interdigital

- Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0 (Fukuoka, Japan, May 20th – 24th, 2024)

- R2-2500890	Additional simulation results for RRM measurement prediction	Huawei, HiSilicon

- RP-243245	New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML)	Qualcomm