3GPP TSG-RAN WG2 Meeting #129bis	R2-2502471

Wuhan, China, April 07th – 11th, 2025

Agenda item:	8.4.4

Source:	Huawei, HiSilicon

Title:	Further discussion on LP-WUS for RRC_CONNECTED mode

Document for:	Discussion and Decision

- Introduction

In this contribution, we further discuss higher layer design for CONNECTED mode.

- Discussion

  - Configuration for LP-WUS

Regarding the issue of how the NW provides the configuration to the UE, it can rely on the existing RRC measurement report. For example, the NW configures the UE to report measurement results based on some events and periodicity, then the NW can know whether it is suitable for the UE to start using LP-WUS functionality, e.g. the NW finds that the measurement result is good enough, or the change of the measurement result is small enough, the NW can configure the UE to use LP-WUS. However, the NW only knows the measurement results from MR. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR.

Observation 1: The NW only knows the measurement results from MR based on existing RRC measurement report. Sometimes even when the measurement result from MR is good, the link quality of LR can be bad due to the weaker tolerance for adjacent-channel interference for LR.

In this case, the UE can inform the NW to stop using LP-WUS since the UE knows the link quality of LR, and also indicate whether the LP-WUS can be used again if it finds that the LR link quality is good enough. For example, the UE is monitoring LP-WUS and measures LP-SS/SSB by LR, once it finds that the LR link quality is not good enough, the UE switches to MR and sends assistance information to the NW to disable the LP-WUS. How the UE determines the LR link quality is not good enough and sends the assistance information, it can be up to UE implementation without configuring additional conditions/thresholds.

Proposal 1: UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: When to send the assistance information is up to UE implementation, without additional configuration to the UE.

After the NW provides the LP-WUS configuration to the UE by RRC signaling, the UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality. Separate activation/deactivation signaling, e.g., L1/L2 signaling is not needed, since it won’t change dynamically.

Proposal 3: The UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality after receiving LP-WUS configuration by RRC signaling. Separate activation/deactivation signaling is not supported.

We understand enabling LP-WUS functionality does not mean UE monitors LP-WUS. Once the LP-WUS functionality is enabled, UE can monitor LP-WUS (i.e., outside C-DRX active time), or UE does not monitor LP-WUS but monitors PDCCH (i.e., during C-DRX active time). Thus, one issue that needs discussion is whether the UE starts LP-WUS monitoring once it receives the LP-WUS configuration.

Observation 2: Enabling LP-WUS functionality does not imply UE monitors LP-WUS. Once the LP-WUS functionality is enabled, UE can monitor LP-WUS (i.e., outside C-DRX active time), or UE does not monitor LP-WUS but monitors PDCCH (i.e., during C-DRX active time).

One option is to start LP-WUS monitoring immediately after receiving the LP-WUS configuration. It seems like LP-WUS configuration can trigger the UE enter LP-WUS monitoring status. However, if the UE applies LP-WUS monitoring ahead of the NW, ongoing services from the NW may be affected which causes data loss for the UE. On another hand, if the LP-WUS functionality takes effect on the NW before the UE, UE may miss the LP-WUS which leads to LR/MR state mismatch between the UE and the NW. Thus, it is beneficial that the UE and NW can align the starting point for monitoring LP-WUS, e.g. the UE and the NW start LP-WUS monitoring after a time period after the UE sends RRC reconfiguration complete message.

Another option is to keep the UE stay in C-DRX Active Time (i.e., using MR) after it receives the LP-WUS configuration. As the UE receives RRC reconfiguration message and needs to receive PDCCH scheduling PUSCH for sending RRC reconfiguration complete message, the UE can naturally stay in C-DRX Active Time. And there is no interruption for ongoing service if any. After the data transmission is finished and C-DRX Active Time ends, the UE goes back to LP-WUS monitoring according to the agreed UE’s behavior.

Proposal 4: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring immediately after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

  - LP-WUS with DC/CA

In RAN1#120 meeting, LP-WUS with DC/CA was discussed with the following agreements.

- For the case when UE is configured with NR-DC with CA without dual DRX groups or without CA in RRC CONNECTED mode, - LP-WUS can be configured on a serving cell per cell-group - This does not imply that LP-WUS has to be configured on both cell groups - LP-WUS indication is applicable to all serving cells in the cell-group - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - FFS whether to support this case as separate UE capability from CA case - FFS UE capability for monitoring LP-WUS on one cell group or both cell groups - FFS: Whether the serving cell on which the LP-WUS is configured is only PCell/PSCell or can be RRC configured cell other than Pcell/PSCell

Based on RAN1 agreements, in CA scenario, LP-WUS indication is applicable to all serving cells in the cell-group if secondary DRX is not configured. The next question is that whether LP-WUS can be configured with secondary DRX at the same time and its impacts on secondary DRX if configured. Currently, legacy DCP and secondary DRX cannot be configured simultaneously due to complexity. In addition, the additional power saving gain was not justified. Similarly for LP-WUS, it is preferred to not support finer control for different DRX groups or different serving cells. Thus, the following two options are suggested:

- Option 1: LP-WUS cannot be configured with secondary DRX at the same time (same as DCP).

- Option 2: LP-WUS can be configured with secondary DRX at the same time, but LP-WUS indication is still applicable to all serving cells in the cell-group without separate handling for different DRX groups/serving cells. In this case, if LP-WUS is received, UE starts the new timer and monitors PDCCH for all activated serving cells. If the PDCCH indicating a new transmission is further received in a DRX groups, the associated drx-InactivityTimer will be started or restarted as the legacy operation.

Proposal 5: For both Option 1-1 and Option 1-2, the following two options are suggested:

- Option 1: LP-WUS cannot be configured with secondary DRX at the same time (same as DCP);

- Option 2: LP-WUS can be configured with secondary DRX at the same time, but LP-WUS indication is still applicable to all serving cells in the cell-group without separate handling for different DRX groups/serving cells.

  - Coexistence with other features

    - Combination with Rel-16 DCP

In RAN2#126 meeting, we had the following agreement:

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The same principle can be applied to Option 1-2, since Option 1-2 can technically provide better performance for power saving and latency compared with DCP. We don’t see the motivation to configure both legacy DCP and Option 1-2, besides, such combination gives additional complexity for the specification.

Proposal 6: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

    - Combination with short DRX cycle

In RAN2#129 meeting, we had the following agreements [1]:

- For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used.

- RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

- RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

Legacy DCP and short DRX cycle are not used simultaneously. During the Rel-16 discussion, some companies thought that if short DRX cycle is configured, it means there is delay sensitive service ongoing, and the performance for the service is more important than power saving. However, it is no harm if power saving can be obtained without negative impact on data transmission. LP-WUS configured and used with short DRX cycle simultaneously can be considered, since it can be supported easily without impacts on performance and standardization work.

Proposal 7: LP-WUS can be configured and used with short DRX cycle simultaneously for the same UE, no further specification impacts are expected.

    - Impacts on SPS and CG

LP-WUS indication in CONNECTED mode is to trigger MR PDCCH monitoring, the LP-WUS only impacts MR PDCCH monitoring. For DL configured resources, e.g. DL SPS, as the gNB allocates DL SPS resources despite knowing the LP-WUS is enabled, gNB expects the UE to perform legacy operation on DL SPS resources, and there is no mismatch between UE and gNB on LP-WUR/MR monitoring. From UE perspective, UE should monitor DL SPS resources by MR to avoid missing any DL data.

Proposal 8: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

However, for UL configured resources, e.g. configured grant Type 1 or configured grant Type 2 (UL SPS), sometimes there is a UL configured resource but UE does not have the UL data for transmission. If UL skipping is not enabled, the UE should still transmit padding bits in the UL configured resource which leads to additional power consumption. If the UE is operating with LP-WUR, it is beneficial to skip UL configured resource if no UL data available so the UE does not need to turn on the MR and keeps monitoring LP-WUS.

Proposal 9: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

- Conclusion

In this paper, we discuss higher layer design for CONNECTED mode, and have the following proposals.

Configuration for LP-WUS

Proposal 1: UE can send assistance information to the network indicating to disable the LP-WUS functionality or whether the LP-WUS can be enabled again.

Proposal 2: When to send the assistance information is up to UE implementation, without additional configuration to the UE.

Proposal 3: The UE considers the LP-WUS is enabled and starts to use the LP-WUS functionality after receiving LP-WUS configuration by RRC signaling. Separate activation/deactivation signaling is not supported.

Proposal 4: RAN2 to further discuss the following alternatives:

- Alt1: UE starts LP-WUS monitoring immediately after it receives the LP-WUS configuration, alignment of the starting point for LP-WUS monitoring needs to be further studied;

- Alt2: UE stays in MR (C-DRX Active Time) after it receives the LP-WUS configuration, UE goes back to LP-WUS monitoring after C-DRX Active Time ends according to the agreed UE’s behavior.

LP-WUS with DC/CA

Proposal 5: For both Option 1-1 and Option 1-2, the following two options are suggested:

- Option 1: LP-WUS cannot be configured with secondary DRX at the same time (same as DCP);

- Option 2: LP-WUS can be configured with secondary DRX at the same time, but LP-WUS indication is still applicable to all serving cells in the cell-group without separate handling for different DRX groups/serving cells.

Coexistence with other features

Proposal 6: Legacy DCP and Option 1-2 are not configured simultaneously for the same UE.

Proposal 7: LP-WUS can be configured and used with short DRX cycle simultaneously for the same UE, no further specification impacts are expected.

Proposal 8: DL SPS is not impacted by LP-WUS, i.e. UE still needs to wake up MR for reception on DL SPS resources.

Proposal 9: For UL configured resources, RAN2 to discuss whether UE can skip UL configured resource if the UE is operating with LP-WUR and does not have the UL data for transmission.

- References

R2-2501332 Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD	Vice Chairman (CATT) report.

3GPP TS 37.340, Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2.