3GPP TSG-RAN WG2 Meeting #129bis	R2-2502482

Wuhan, China, 7th – 11th Feb 2025

Agenda item:	8.10.5

Source:	Samsung

Title:	SON/MDT for Leftovers from Rel-18

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the remaining issues for the SON/MDT for Leftovers from Rel-18.

# Discussion

## RACH optimization for SDT

In running RRC CR for SON/MDT, there is an Editor’s Note as below:

[[

sdt-FailureCause-r19                 ENUMERATED {t319a-expiry, maxRetxThreshold, preambleTransMax, configuredGrantTimer, cg-SDT-TimeAlignmentTimer, cellReselection, spare2, spare1}                                       OPTIONAL,

sdt-DL-RsrpInfo-r19                  RSRP-Range                                       OPTIONAL,

sdt-UL-DataVolume-r19                INTEGER (0..xxxx)                                OPTIONAL,

timeSinceSdtExecution-r19            TimeSinceFailure-r16                             OPTIONAL

]]

Editor's note: The value range for sdt-UL-DataVolume-r19 is FFS.

Sdt-DataVolumeThreshold, the Data volume threshold used to determine whether SDT can be initiated, is broadcasted in SIB. It is defined as follows.

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range                                                            OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000,

ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

}

As the broadcasted values are not changed frequently, the network can know the value of the threshold used. In our view, the simplest way is to report the actual UL data volume used. Since RAN2 discussed to log the data volume when UE has evaluated but the evaluation failed, it is possible that the UL data volume will be bigger than the largest value in sdt-DataVolumeThreshold-r17. However, it would be enough to report the maximum value ‘byte96000’ in such cases.

We understand that further optimisations are possible, for e.g. what the network requires for optimisations is whether the data volume is between two possible threshold values when there is a failed SDT, as the network can adjust the threshold only between the enumerated values. So the UE can report its UL data volume in relation to its nearest lower threshold. For e.g. if the data volume at the time of evaluation is 50 bytes, UE may report 0 indicating that it is near to 32. If the data volume is 120, UE may report 2. If the data volume is 1500 UE may report 7 to inform that it is closer to 1000. Nevertheless, such optimisations may not be really needed, considering the added complexity.

Proposal 1: Value range of SDT-UL-DataVolume is 0 to 96000. UE reports 96000 when the data volume is larger than 96000.

In the running CR, it is specified that UE reports the UL data volume as the the UL data volume at the time of SDT evaluation. We think that it is only required to report the UL data volume of radio bearers (DRB or SRB2) configured for SDT. It is also required to report UL data volume only when the SDT evaluated is MO-SDT.

Proposal 2: UL data volume is reported only for MO SDT and only radio bearers (DRB and SRB2) configured for SDT are considered.

## MRO for MR-DC SCG failure

RAN2 has agreed to log the  perRA-InfoList for EN-DC in case of SCG failure.

Add reporting of the following parameters for SCG failure report in EN-DC scenario:

⁻	For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

⁻	For timeSCGFailure: value range 0-1023;

⁻	For failureType: Reuse the legacy field.

-	perRA-InfoList

For NR-DC, the reported values in case of SCG failure includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800. Our understanding is all of these are useful for EN-DC/NG-EN-DC also. There is no difference in the maximum number of CSI indices between NR-DC and EN-DC. Similarly, the SCG can be NR-U for EN-DC also. Therefore, it is required to include the same information as in SCGFailureInformation in the SCGFailureInformationNR also. This is captured in the LTE running CR and this can be kept.

Proposal 3: perRA-InforLIST reported in SCGFailureInformationNR includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 as in SCGFailureInformation (same as in running LTE CR).

## MHI enhancement for SCG Activation/Deactivation

In the RAN2#129 meeting, it was agreed to report the time the UE has spent in PSCell with SCG activated, while the details were FFS.

UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

As we discussed in [1], the SCG activation information has limited use, especially when there is a network based solution available. So it is enough to report the required information in a simple way. It will be ok to report absolute time as other time information in MHI.

Proposal 4: UE reports the absolute time it has spend in PSCell with SCG activated in MHI.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: Value range of SDT-UL-DataVolume is 0 to 96000. UE reports 96000 when the data volume is larger than 96000.

Proposal 2: UL data volume is reported only for MO SDT and only radio bearers (DRB and SRB2) configured for SDT are considered.

Proposal 3: perRA-InforList reported in SCGFailureInformationNR includes PerRAInfoList-r16, PerRAInfoList-v1660 and PerRAInfoList-v1800 as in SCGFailureInformation (same as in running LTE CR).

Proposal 4: UE reports the absolute time it has spend in PSCell with SCG activated in MHI.

# References

[1] R2-2500867: MHI Enhancement for SCG Deactivation/Activation