3GPP TSG-RAN WG2 Meeting #129bis		R2-2502483

Wuhan, China, April 7 – 11th 2025

Agenda Item: 		8.1.2.2

Source:			Sony

Title:	Some aspects for model monitoring on UE side

Document for:		Discussion & Decision

# Introduction

In this contribution we provide our views on model monitoring aspects for beam management. We also discuss the quality of input data for inference.

# Discussions

## 2.1 Model monitoring subset

For model monitoring, the NW does not always need to indicate the full set to the UE, especially considering the potential for larger-scale beams in the future. Therefore, we fully support the use of subsets of beams for monitoring. However, there are several challenges that require further study, such as how to select an appropriate subset from the full set and the mapping rules between the full set and the subset.

Whether the subset used for monitoring measurements should include the Top K predicted beams depends on the performance metric being used. If the metric focuses on the RSRP difference between the predicted and measured RSRP of the corresponding beams, the subset has to involve the Top K predicted beams. However, if the performance metric is based on Top 1 or Top K beam prediction accuracy, the subset does not necessarily need to cover the Top 1 or Top K predicted beams. For example, if the highest RSRP in the measured set exceeds the predicted highest RSRP in the set of predicted beams, the subset can still be considered valid even if it does not include the Top 1 or Top K predicted beam. In this case, such an event can be counted as a prediction failure, leading to two statistical methods: one is when the number of such failures exceeds a predefined threshold, the model can be considered to have failed; another is that the failure probability can be calculated based on these failures, and the beam prediction accuracy equals 1 minus the failure probability. The advantage of this approach is that the NW does not need to wait for the UE to report its prediction results. Instead, after the monitoring mechanism is activated, the NW can directly allocate a monitoring set to the UE for measurement, thereby reducing latency. Additionally, the NW does not need to reconfigure/update the monitoring set due to the absence of predicted beams in the initial configuration.

Certainly, if a subset does not contain the top-1 or top-K predicted beams and the measured RSRP values are lower than those of the predicted beams, this subset can be considered invalid. Reconfiguration of the subset in such case is worthy of further research and analysis.

Proposal 1: Support the configuration of a subset from Set A for monitoring and further investigate the mapping rules between the subset and the full set.

Proposal 2: Support further investigation into the scenario where the subset does not include the Top K predicted beams during monitoring resource configuration.

## 2.2 Model monitoring resource optimization

For the UE-side model, further study is needed to determine how to trigger reporting based on events for Option 1 and/or Option 2 during monitoring. These options are defined in RAN1 as below:

Option 1 (NW-side performance monitoring): · UE sends a report to NW (for the calculation of performance metric at NW) o Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report o FFS on other contents · The report is at least configured/triggered by NW · Note: this may or may not have additional spec impact Option 2 (UE-assisted performance monitoring): · UE calculates performance metric(s) o FFS how to report and what to report

Regarding option 2, UE-assisted performance monitoring, it is necessary to define specific events that trigger reporting for model monitoring. Considering the scenario, in BM-Case 1, when predicted candidate beams are also measured in Set C or Set B, a direct comparison between the predicted and measured results can be performed to determine whether the inference is successful or not. For example, this involves checking if the predicted RSRP differs from the measured RSRP. When such a failure is detected, the UE can report it to the network to initiate further monitoring procedures. This process can be defined as an event.

For BM-Case 2, performing model monitoring needs to measure the full resource set configured by NW which results in significant measurement overhead and latency. Especially, to avoid the "ping-pong" effect, the performance metric must be calculated across multiple time instances rather than relying on a single measurement. Hence, to reduce this overhead, we propose reusing part of the input data from the previous inference instance for each new inference instance. This approach can be illustrated as following picture: for each inference instance, the model's input is based on measurements from five consecutive slots and predict for future four consecutive slots. In the first inference instance, the input data is derived from the measurements of slots one through five. In the second inference instance, the input data is collected from two parts: the new measurement from slot six and the reused measurement data from slots two through five, which were measured in the first inference instance.

This approach doesn’t need NW to configure the full set of resources according to the model's input requirements. Instead, only a portion of the resources needs to be configured, while the other part of the input data is derived from the measurements obtained in the previous instance. This approach can significantly reduce the measurement overhead.

Figure1. schematic diagram of multiple inferences instances with data reuse in BM-Cases 2

In this figure, the candidate beam in slot eight has been predicted in multiple inference instances. If the predicted results in each instance are the same or similar, this indicates that the model's performance is stable and suitable for the current scenario. Conversely, if the predicted results vary significantly, it may suggest that the model is failing to infer accurately.

Proposal 3: Support for defining event(s) to trigger reporting for monitoring,

  - when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1.

  - when the multiple prediction results for the same future time instance vary significantly in BM-Case2.

Proposal 4: For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

## 2.3 UE side model update with NW assistance

For UE-side models, updates can be assisted by the NW. The need for NW assistance in updating UE-side models arises in the following scenarios:

- Model Initial Training on the NW Side: The initial training of the model is performed on the NW side and then distributed to the UE. Given that the NW has comprehensive knowledge of the model’s structure and parameters, it is better positioned to update the model based on measurement reports or prediction outcomes reported by the UE.

- Limited UE Capabilities: The UE may lack sufficient computational power, memory, or battery life to perform model updates. In such cases, the UE might rely on the NW to assist with model updates. However, this approach could potentially lead to some privacy-related parameters being disclosed to the NW, which is unavoidable in these situations.

NW assists model updating that ensures a comprehensive updating mechanism that leverages external resources for enhanced efficiency and accuracy. In the AI-based beam management case, this approach requires the UE to report  model difference.  The model difference can be one of the following:  difference between the required value and the monitored value of model performance metrics, difference between expected results and actual predicted results (model output) of the current model to the NW.  By observing the difference in model performance metrics, the NW can determine how well the model is performing against the optimal/required level and may initiate a model update before its performance degrades to an unacceptable level. The expected results represent actual optimal beam based on UE measurements, which can be obtained through traditional beam sweeping, as illustrated in Figure 1.

Figure 1. Traditional beam sweeping

Proposal 5: For the UE-side models, UE can report the following types of model difference to the NW when requesting assistance in model updates:  difference between the required value and the monitored value of model performance metrics, or difference between the predicted results and the traditional beam measurement-based results. .

## 2.4 Quality of collected data

gNB will configure a set of beams for UE to measure to collect input data. Hence, Set C [1] has been introduced. Set B as the model input can be chosen from Set C. We understand that model input data can be filtered based on specific rules, such as ensuring that the RSRP obtained through measurement is higher than a predefined threshold.

However, a challenge arises when there may not be enough high-quality input data available for the model. This can lead to two possible outcomes: firstly, inputting inadequate data into the model, resulting in a decline in predicted accuracy. If the predicted accuracy decreases significantly, the predicted beams are unsuitable for link communication. Alternatively, even if there is a decline in predicted accuracy, the results may still satisfy the link quality requirements. No matter whichever is the outcome, the gNB needs to understand the reason of the decline in model performance.

Hence, when the UE does not collect enough high-quality data for Set B from Set C, it should report to the gNB. Subsequently, the gNB can configure a new set for collecting input data. If the UE still cannot obtain high-quality data in the new set, it indicates that the scenario has changed and is no longer applicable for the current serving model. Model switching or fallback can be considered afterwards. We think this should in general apply to all use cases.

Proposal 6: At the inference stage, the UE report that the quality of input data has gone bad, to the gNB when the measurement results fail to meet the model's input requirements (applies to both beam management and positioning).

# Summary

In this contribution, we have the following proposal for RAN2 to agree:

Proposal 1: Support the configuration of a subset from Set A for monitoring and further investigate the mapping rules between the subset and the full set.

Proposal 2: Support further investigation into the scenario where the subset does not include the Top K predicted beams during monitoring resource configuration.

Proposal 3: Support for defining event(s) to trigger reporting for monitoring,

- when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1.

- when the multiple prediction results for the same future time instance vary significantly in BM-Case2.

Proposal 4: For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

Proposal 5: For the UE-side models, UE can report the following types of model difference to the NW when requesting assistance in model updates:  difference between the required value and the monitored value of model performance metrics, or difference between the predicted results and the traditional beam measurement-based results.

Proposal 6: At the inference stage, the UE report that the quality of input data has gone bad, to the gNB when the measurement results fail to meet the model's input requirements (applies to both beam management and positioning).

# References

- RP-240774: AI/ML for NR air interface