3GPP TSG RAN WG2 Meeting #129bis			R2-2502484

Wuhan, China, April 7th – 11th 2025

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on paging for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new WID for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

At RAN2#129 [3], the following agreements were made related to Paging.

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

Agreements The A-IoT paging message can include a number of msg1 resources From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Also the following Post email discussion was agreed [4]

[POST129][035][AIoT] Paging (Qualcomm)

The summary from the email discussion included the following proposals.

Proposal 1:	Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/7/5)

Proposal 2:	Discuss and decide how a device determines the procedure is ‘ongoing’ or not.

Proposal 3:	Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2).

Proposal 4:	Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3).

Proposal 5:	Discuss and decide whether Reader ID information should be provided to the device (yes/no/FFS = 11/17/1).

Proposal 6:	Wait for SA2/RAN3 progress before further discussing on how the reader calculates transaction ID.

Proposal 7:	Discuss whether to send an LS: To SA2 asking whether correlation ID is expected to be same or different when the same service is requested from different readers, and to RAN3 asking whether coordination between readers is expected.

Proposal 8:	Postpone discussion on exact size of transaction ID until further progress on other issues captured in previous proposals.

Proposal 9:	Discuss whether and how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.)

In this contribution, we discuss our view on some RAN2 related aspects related to subsequent paging, a new R2D trigger signal for D2R transmission and views on a number of the proposals from the email discussion.

# Discussions on paging for Ambient IoT

In legacy cellular systems, for the Network to reach the UE normally paging is used, for which a UE-ID is needed. The UE-ID is not only used for identification, but also to group the UEs in different wake-up groups for the system to page the UE without having to wake-up all UEs and for the UEs to save power, not having to listen for paging continuously.

In the case of Ambient IoT, the traffic assumptions have been defined as Device Terminated (DT), and Device Originated (DO). In this release, DO traffic only considers DO-DTT, Device Originated, Device Terminated Trigger, which means that the network wakes up the device but for expected or required uplink traffic. This could e.g., be sensor data that should be reported.

## Subsequent paging

One issue that has been identified in the study item [3GPP TR 38.769 V2.0.0 (2024-12)] [2], is when the reader sends multiple subsequent paging messages associated with the same service request [ chapter 6.3.3].

The duplicated response from a device for the same service request should be avoided, this to reduce the power consumption at both device and the reader as well as reduction of spectral usage. To address this, the A-IoT paging message can include information to avoid this duplicated response from the device to a reader.

The overall procedure between the A-IoT device and reader is according to Figure 1 where overall AS procedures between A-IoT device and reader, [2] is presented.

Figure 1, Overall AS procedures between A-IoT device and reader (Figure 6.3.1-1 in [2])

In this paper we discuss and list different alternatives on the type of information that can be included in the Paging response message to reduce and/or minimize the impact of duplicated responses from the AIoT device.

Uplink Response:

One option is when the A-IoT device receives a subsequent paging message and detects it is for the same service request and is a repeated paging message. The AIoT device responds again, but it does not include any data or trigger any subsequent action if same as in the initial paging message, but it only replies with a simple indication, that the A-IoT device has already responded.

The indication could be as simple as a one bit:

-Already responded [1 bit]. TRUE Conditional

Proposal 1: The paging message includes information, that the AIoT device can respond with a one bit indication.

The indication should be included already in message 1 or message 3.

Proposal 2:  The device responds with a one-bit indication, that it has already responded.

Alternatively the Reader can include a repetition number in the A-IoT Paging message, and when the A.IoT device receives a subsequent paging message for the same service, (service request) the device responds with a repetition number, and does not include the same data again. Hence, the subsequent response message will be smaller in size. In case the reader successfully completed the initial paging attempt, the Reader will understand that since there is not data or data field is empty the device already has responded. This alternative would be for the case when there is no indication during the inventory steps, up to message 3, but in the subsequent command procedure, where data is appended after message 4.

Proposal 3: The paging message includes information such a repetition number, that the AIoT device can associate its response with.

Downlink Ack:

As a third option, the subsequent paging message includes either those devices that have responded, or alternatively those devices that have not responded.

This would not be as part of any RACH or STEP B procedure but included in the subsequent A-IoT paging message. For the case where the device -ID is included in the subsequent A-IoT paging message, the corresponding devices can simply ignore to respond again. The behavior can be indicated either that service, transaction or reader id is repeated, or a flag is set indicating that those A-IoT devices whether the device id is included can refrain from responding.

For the other case, the reader has prior information on what devices are to be paged and can then include the ID of those devices that have not responded to the initial A-IoT paging message. For the initial paging message, only group or type ID is included, to limit the message size.

Proposal 4: The subsequent paging message for the same service includes information, on which devices that has either responded or which have not responded.

Time window:

Additionally, a time window may be used as a fourth option or in conjunction with any of the alternatives above.

When the UE receives an A-IoT message is starts a timer, and if the device gets subsequent A.-IoT paging messages for the same service, while the timer is running, the A-IoT device can ignore responding to those subsequent A-IoT paging messages for the same service request.

Proposal 5: During a configured time window, the A-IoT device can ignore responding to subsequent paging transmission for the same service request.

## R2D trigger signal for D2R synchronization

Due to the simple design of the Ambient IoT devices, the quality and reliability of the clock accuracy may vary, and it may be difficult to maintain sync due to clock inaccuracy or clock drift.

During the study phase timing and clock information has been discussed and it has been captured in [TR 38.769], the usage of clock acquisition signals. An R2D timing acquisition signal (R-TAS), immediately preceding the transmission of PRDCH, is included at least for timing acquisition and indicating the start of R2D transmission, and a D2R timing acquisition signal (D-TAS), preceding each PDRCH, is included at least for timing acquisition, indicating the start of the D2R transmission.

The pilot signals R-TAS and D-TAS is used by the receiver in both the device and Reader. However, the timing of the device transmitter is based on an internal device clock which may use the R-TAS and in particular the CAP (Clock-acquisition part) of the R-TAS. For device type 1, the most constrained Ambient IoT device, it may not be able to keep a clock running with an accuracy assumed for good slot alignment. Furthermore, when the device is assigned a specific slot to transmit in, it needs to calculate when in time this will occur and set a timer accordingly.

To handle this timing issue for the D2R transmission, the Reader could transmit a specific slot indication, to indicate for the A-IoT device, the start of the slot to be used for the D2R transmission.

This would in particular useful for transmission of message 1 for slotted Aloha, where the device needs to wait for a random slot to transmit in. see Figure 2.

Also for the case for triggering the transmission of message 3 it would be beneficial to introduce a slot or start of resource indication for that message 3. This would be even more useful if there are multiple devices included in the same RAR, message 2, and they have individual slots or resources assigned, then the message 2 would include a relation to what slot or resource number the device should use.

This number or id is included in the slot indication, in order for the device to know which slot or resource that is the intended slot or resource for the device to respond in.

Figure 2, Slot indication

Proposal 6: Introduce a new trigger signal, slot indication, indicating the timing or start of the slot when the device should respond in.

## Open issues related to some of the proposed email discussion:

Proposal 1:	Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/7/5)

Generally, we think the device should only handle one procedure at the time, and that the device should not monitor or react to any paging while a previous paging procedure is ongoing.

Observation 1: Generally, we think the device should only handle one procedure at the time, and that the device should not monitor or react to any paging while a previous paging procedure is ongoing.

Proposal 2:	Discuss and decide how a device determines the procedure is ‘ongoing’ or not.

If the service request is inventory only, agree on that the procedure is terminated after message 3 transmission, or after acknowledgement of message 3 reception.

Though, in case the reader transmits repeated paging messages for the same service request, then the behaviour should be according to alternatives discussed for minimizing the impact of repeated and subsequent paging for the same service.

Proposal 7: Further discussion is needed to conclude when the procedure can be regarded as terminated.

Proposal 3:	Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2).

If location scenario is considered, then the device needs to distinguish whether the same service request is received from the same reader or different reader, but likely it would be enough with only a Transaction ID.

Proposal 8: If location scenario is considered, then the device needs to distinguish whether the same service request is received from the same reader or different reader. It would likely be enough with only a Transaction ID.

Proposal 4:	Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3).

For this use case, the command part of the service request could include information to the device that the service is for locations purpose, implying that if the same service request is received from a different reader, and the device should respond to it.

Proposal 9: The command part of the service request could include information that the service is for locations purpose, and the device should respond to each of the different readers initiating the same service request.

Proposal 5:	Discuss and decide whether Reader ID information should be provided to the device (yes/no/FFS = 11/17/1).

Except for the Location case,the reader ID would not be needed, if the service request is the same, and also the transaction ID. Anyway, the device would act as if the repeated service request would be coming from the same Reader.

Proposal 10: Basically, the reader ID would not be needed. If the Service request is the same, and the also the transaction ID, the device would anyway act as if the repeated service request would be coming from the same Reader.

Proposal 7:	Discuss whether to send an LS: To SA2 asking whether correlation ID is expected to be same or different when the same service is requested from different readers, and to RAN3 asking whether coordination between readers is expected.

We think the same correlation ID is to be used for the same service request triggering multiple Readers.

Proposal 11: We think the same correlation ID is to be used for the same service request triggering multiple Readers.

Proposal 9:	Discuss whether and how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.)

The paging identifier is a Temporary ID according to conclusions in SA3. The Temporary ID will be updated over to time, how often is FFS in SA3. If the Temporary ID is used in MAC layer, then upper layer must update MAC layer when the Temporary ID is updated. As there is no need (as of now) for MAC layer to have any awareness of the paging identifier, we believe it would add unnecessary complexity and interlayer dependency.

Proposal 12: There should be no need for the reader to know such association.

# Summary

In this contribution, we have discussed our views on aspects related to subsequent paging, a new R2D trigger signal for D2R transmission and views on a number of the proposals from the email discussion

Proposal 1: The paging message includes information, that the AIoT device can respond with a one-bit indication.

Proposal 2:  The device responds with a one-bit indication, that it has already responded.

Proposal 3: The paging message includes information such a repetition number, that the AIoT device can associate its response with.

Proposal 4: The paging message includes information, on which devices that has either responded or which have not responded.

Proposal 5: During a configured time window, the A-IoT device can ignore responding to subsequent paging transmission for the same service request.

Proposal 6: Introduce a new trigger signal, slot indication, indicating the timing or start of the slot when the device should respond in

Observation 1: Generally, we think the device should only handle one procedure at the time, and that the device should not monitor or react to any paging while a previous paging procedure is ongoing.

Proposal 7: Further discussion is needed to conclude when the procedure can be regarded as terminated.

Proposal 8: If location scenario is considered, then the device needs to distinguish whether the same service request is received from the same reader or different reader. It would likely be enough with only a Transaction ID.

Proposal 9: The command part of the service request could include information that the service is for locations purpose, and the device should respond to each of the different readers initiating the same service request.

Proposal 10: Basically, the reader ID would not be needed. If the Service request is the same, and the also the transaction ID, the device would anyway act as if the repeated service request would be coming from the same Reader.

Proposal 11: We think the same correlation ID is to be used for the same service request triggering multiple Readers.

Proposal 12: No. There should be no need for the reader to know such association.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- 3GPP TR 38.769 V2.0.0 (2024-12)]

- Minutes RAN2#129 meeting, Athens

- [POST129][035][AIoT] Paging (Qualcomm)