3GPP TSG-RAN WG2 Meeting # 129bis 	                R2-2502495

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.11.2

Source: 	Sony

Title:	Random Access for SBFD Operation

Document for:	Discussion and Decision

# Introduction

3GPP has been working for enhancing duplexing operation for TDD based on subband duplex (SBFD) operation at gNB side within a TDD carrier [1-2]. The motivations of enhancing duplexing operation at least for TDD are for improving system capacity, reducing delays, and improving uplink coverage.

A Rel-19 Work Item (WI) on Duplex Evolution is therefore agreed to specify the requirements for FD-TDD where FD-TDD is performed at the gNB, i.e., the gNB can transmit and receive data/signals to/from the UEs at the same time on the same frequency band, whilst the UE is maintained as HD-TDD. That is, full duplex TDD is achieved at the gNB by scheduling a UE in the DL and scheduling another UE in the UL within the same OFDM symbol.

RAN1#116 Agreement:

- For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier.​

RAN1#117 Agreement:

- For RRC connected mode UEs, SBFD subband time period:

  - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

  - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- For cell-specific indication of SBFD subband frequency location, adopt the following option:

  - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1#116bis Agreement:

- Confirm the working assumption:​

  - Working assumption​

    - For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.​

RAN1#117 Agreement:

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

RAN1#117 Agreement:

- Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

RAN1#118 Working Assumption:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

RAN2#127 Agreements:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

FFS whether/how early indication is used during a SBFD RA procedure.

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

RAN2#127bis Agreements:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration.

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2#128 Aagreements:

The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

RAN2#129 Aagreements:

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell.

When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

FFS whether RO type selection is performed before or after the RA type selection.

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In this contribution, we discuss some details of random access in SBFD operation and provide some proposals at the end.

## Random Access for SBFD Subband

Work Item (WI) captures to specify SBFD operation to support random access in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs. The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage. RAN2 agreed that 4-step RACH is supported while 2-step RACH is FFS. Based on the above justification of reducing the delays and increasing the capacity, it is reasonable to support 2-step RACH in UL SBFD symbols as well.

Observation 1. The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Proposal 1. Support 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Early Indication:

In RAN2#127bis meeting, RAN2 discussed whether/how early indication is used during a SBFD RA procedure. For one case, it is agreed that RAN2 understands that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

However, for the other case, when SBFD-aware UE selects non-SBFD ROs, then the network can not identify the SBFD-aware UE, hence the network should schedule message 3 to non-SBFD PUSCH resources. Propopents for early idication think that the early indication is needed in order the network to be able to schedule message 3 of SBFD-aware UE to non-SBFD PUSCH resources. However, most of the companies questioned the merit of this, because for at least two reasons : a) MAC CE would increase unnecessary design efforts for RAN2, and b) early indication via message 3 would provide marginal gain given that the UE will anyway provide its capability report to gNB.

Proposal 2. Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure.

The initial RO type indication for CFRA:

For CFRA, RAN2 agreed that the RO type is indicated by NW for CFRA. For PDCCH-ordered CFRA, obviously the indication should be included in the PDCCH-order whether to use additional RO (SBFD RO) or legacy RO. The motivation is that the network has an overall picture of the resource usage and what service triggered the PDCCH order. In our view, a 1-bit can be included in the DCI carrying the PDCCH-order to indicate whether to use additional RO (SBFD RO) or legacy RO.

Proposal 3. For CFRA with PDCCH-order, the RO type is indicated in the PDCCH-order. 1-bit is included in the DCI carrying the PDCCH-order to indicate whether to use additional RO (SBFD RO) or legacy RO.

For other cases (like SI request if needed) where CFRA is not triggered by PDCCH-order, if there are two different resources for CFRA locating in the additional RO (SBFD RO) and legacy RO, then the network should inform UEs which one to apply. Obviously different UEs have different location within the cell as well as channel and interference conditions, so the indication has to be per UE. Alternatively, an additional selection condition can be configured for all UEs, like SSB RSRP threshold and load balancing criteria.

Observation 2: For CFRA cases (like SI request if needed) where CFRA is not triggered by PDCCH-order, UEs have different location within the cell as well as channel and interference conditions, so the network indication has to be per UE. Alternatively, an additional selection conditions is configured for all UEs, like SSB RSRP threshold and load balancing criteria.

Proposal 4. For CFRA (like SI request) that is not triggered by PDCCH-order, network should indicate whether to use additional RO (SBFD RO) or legacy RO and the indication could be per UE.

The initial RO Type selection for CBRA:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

- The network may indicate the prioritization of the additional ROs over legacy RO for load balancing. For example, in the early deployment of the SBFD feature where number of SBFD-aware UEs are small, the network can mandate all SBFD-aware UEs to select the additional ROs more often compared to legacy non-SBFD ROs, at least in the first attempt. It means that after certain (configured) number of times of RACH attempt in SBFD RACH occasions, the UE is allowed to switch to legacy RACH occasions.

- In the last meeting, it is agreed that for initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS.

- In terms of signalling to indicate the RO type from the network could be a cell specific signalling as for example load balancing is for all SBFD-aware UEs. The question is do we specify a binary indication like ON (all SBFD UEs to use additional RO) or OFF (all SBFD UEs to use legacy RO). It seems this is not desirable as there is no case where UE can select either additional RO or legacy RO with equal chance/probability. So, we think multilevel indication is needed. For example:

- In the early deployment of the SBFD feature where number of SBFD-aware UEs are small, the network can mandate for all SBFD-aware UEs to select the additional ROs, i.e., additional RO 100% and legacy RO 0%.

- When the deployment of the SBFD feature is popular (i.e., a significant number of UEs supports the feature), the network can signal equal selection probability of additional RO and legacy RO for all SBFD-aware UEs, i.e., additional RO 50% and legacy RO 50%.

- The network can signal unequal selection probability additional RO and legacy RO for all SBFD-aware UEs, i.e., additional RO 80% and legacy RO 20%.

- In some cases, the network can mandate all SBFD-aware UEs to select the legacy RO, i.e., additional RO 0% and legacy RO 100%.

Based on the above deign principles, we think the indication should be semi-static signalling as dynamic change of this partitioning is not changed in a short period. Hence, signalling should be cell specific with four different values, indicated by 2-bits representing four different values.

Proposal 5: For initial RA transmission, the network indicates in SIB1 the RO type (legacy RO or additional RO) to the SBFD-aware UEs for the case of CBRA, where indication is 2-bits representing four different values.

RNTI collision/confusion:

For configuration 2, how to handle the case where the legacy ROs (e.g. in Flexible Symbols) overlap with additional ROs needs to be discussed in RAN1 or RAN2. As the possible impact relates to RNTI collision/confusion, it should be discussed RAN2.

The legacy RA-RNTI/MsgB-RNTI are calculated as follows (TS 38.321):

The RA-RNTI/ MSGB-RNTI associated with the PRACH occasion in which the Random-Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

In our view, the above issue can be simply resolved by introducing a configurable offset for the symbol index (s_id) in the legacy RNTI equation. The offset will shift the s_id into another unused symbol index (0 ≤ s_id < 14). This will increase the number of available RNTIs as well as avoiding RNTI collisions, without increasing RNTI space. The offset should be signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

Proposal 6: Simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

## Conclusions

In this contribution, we have discussed some details of random access in SBFD operation. We have the following observations and proposals:

Observation 1. The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Proposal 1. Support 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Proposal 2. Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure.

Proposal 3. For CFRA with PDCCH-order, the RO type is indicated in the PDCCH-order. 1-bit is included in the DCI carrying the PDCCH-order to indicate whether to use additional RO (SBFD RO) or legacy RO.

Observation 2: For CFRA cases (like SI request if needed) where CFRA is not triggered by PDCCH-order, UEs have different location within the cell as well as channel and interference conditions, so the network indication has to be per UE. Alternatively, an additional selection conditions is configured for all UEs, like SSB RSRP threshold and load balancing criteria.

Proposal 4. For CFRA (like SI request) that is not triggered by PDCCH-order, network should indicate whether to use additional RO (SBFD RO) or legacy RO and the indication could be per UE.

Proposal 5: For initial RA transmission, the network indicates in SIB1 the RO type (legacy RO or additional RO) to the SBFD-aware UEs for the case of CBRA, where indication is 2-bits representing four different values.

Proposal 6: Simply introduce a configurable offset for the symbol index (s_id) in the RNTI equation, and the offset is signalled in SIB1 as a part of PRACH resource configuration for SBFD-aware UEs.

## References

# [1] RP-232763, 3GPP TR 38.858 V2.0.0, Study on Evolution of NR Duplex Operation.

# [2] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104.