3GPP TSG-RAN WG2 #129bis	R2-2502532

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.0.2.1

Source:	Ericsson

Title:	Support of inter-gNB scenarios for RACH-less handover

Document for:	Discussion

# 1	Introduction

In the last RAN2 meeting, the following agreement was made in the context of NTN:

Agreements:

1.	RAN2 assumes that, in NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch. We send a LS to RAN3 (cc RAN1) asking if there is any problem with this

This contribution is to clarify whether this agreement applies to all Rel-18 UEs or if is only relevant for NTN.

# 2	Discussion

According to what has been discussed in the last RAN2 meeting, in the NTN session it has been agreed that that the assumption for RACH-less handover for the case of NTN is that this can be used for both intra-gNB and inter-gNB scenarios. However, to confirm this an LS has been sent to RAN3 in order to check if everything is in place of whether some additional work needs to be done. This is an extract from the text of the LS that was sent in [1]:

RAN2 has assumed that in NTN RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch. However, RAN2 would like to confirm with RAN3 that the assumption made at RAN2#128 meeting for inter-gNB scenario in NTN is feasible from RAN3 point of view.

For NTN, RAN2 assumes that RACH-less handover for an inter-gNB scenario is possible. However, this needs to be confirmed with RAN3.

According to the LS sent by RAN2, RAN3 has discussed the matter in Athens that the outcome has been that “there is no issue to support inter-gNB RACH-less HO in NTN”, as stated in [2].

The applicability of the RACH-less handover for an inter-gNB scenario is something that has been already discussed in RAN2 (when RAN2 has decided to extend the RACH-less feature to all Rel-18 UEs), but at the end the outcome has been that only the intra-gNB scenario for the RACH-less handover is supported. This basically happen for two main motivations:

- RAN2 did not want to trigger any additional work on RAN3 (since RAN3 did not have any formal allocation for this)

- The applicability of the RACH-less feature to all Rel-18 UEs was decided during the ASN.1 review and thus when the functional freeze already happened.

This was also confirmed by the agreement taken at the RAN2#125 meeting:

Agreements on RACH-less HO

1.	We will generalize RACH-less HO without impact to RAN3 in Rel-18

When RAN2 has decided to generalize the RACH-less feature to all Rel-18 UEs, the intention was to support this only for intra-gNB scenarios.

Therefore, since now Rel-18 has been frozen our intention is to avoid re-opening this discussion and, eventually, triggering any additional work on RAN3. For this reason, we would like RAN2 to confirm that the RACH-less handover feature will not be extended to the inter-gNB scenario, except for the case of NTN. Thus, we propose:

RAN2 to confirm that the RACH-less handover feature will not be extended to the inter-gNB case, except for the case of NTN (if this will be eventually supported).

# 3	Conclusion

According to the discussions in Section 2, we have the following observations:

Observation 1	For NTN, RAN2 assumes that RACH-less handover for an inter-gNB scenario is possible. However, this needs to be confirmed with RAN3.

Observation 2	When RAN2 has decided to generalize the RACH-less feature to all Rel-18 UEs, the intention was to support this only for intra-gNB scenarios.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that the RACH-less handover feature will not be extended to the inter-gNB case, except for the case of NTN (if this will be eventually supported).

# References

R2-2411191, LS on inter-gNB RACH-less HO in NTN, RAN2, RAN2#128 meeting, US, November 2024.

R3-250762, Reply LS on inter-gNB RACH-less HO in NTN, RAN3, RAN3#127 meeting, Athens, February 2025.