3GPP TSG-RAN WG2 Meeting #129-bis	Tdoc R2-2502547

Wuhan, China, April 7 – 11, 2025

Agenda Item:	8.12.3

Source:	Ericsson

Title:	Impacts from other NR MIMO Phase 5 objectives

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the impacts from NR MIMO phase 5 including any of the following objectives:

- Specify enhancement to facilitate UE-initiated/event-driven beam management

- Specify CSI support for up to 128 CSI-RS ports

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions

# 2	Discussion

## 2.1	UE-initiated beam management in RAN2

RAN1 has made some progress in their discussion for UE initiated beam reporting. Currently the functionality includes Mode A and Mode B, which represents dynamically scheduling UCI by gNB or UCI in pre-configured resource(s) for second UL channel. Some progress also in UL signalling content, beam reporting events, RS measurement for current and new beam (Event-2) etc.

Concerning the Beam trigger events for Event-2, there are still apparently a few options under discussion, to determine details on which measurement can trigger an event.

Notable is that the discussion has still not converged in a conclusion whether PHY or MAC handles the event evaluation and related specification of that.

In RAN2 we have:

RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

If we look at the basic agreement from RAN1#117:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

We can note that like what is specified for beam failure detection or LTM event evaluation, MAC layer currently have a framework in place in handling events using counters or windows based on L1 measurement (e.g., counting indicated instances within a window) and may upon a triggering event, such as an TTT, activate a report, indication or similar. For UE initiated beam management and Event-2, this means that the framework within LTM or BFR could easily be reused with very little specification effort to handle the event evaluation based on L1 measurements.

Also to note, the current new UCI for the first PUCCH channel for both Mode-A and Mode-B seems to at length mimic the legacy procedures for SR, i.e also here several procedural characteristics in MAC can be reused, such as prioritization, prohibit timers, etc if needed.

For example, by looking at LTM (RAN2 running CR, Disc #109 MOB), section 5.x.1, the whole section and subsection could be used as baseline in that we have:

Config:

- The network may configure the UE to perform L1 event triggered beam level measurement..

- The network may configure the UE to indicate to lower layers … measurement information .. event trigger(s).. based on..RSRP Threshold..

- … and so on

Measurement eval:

An RRC_CONNECTED UE obtains L1 RSRP beam level measurement results by measuring one or multiple beams as configured by the network as specified in [RAN1 REF]. For each L1 beam level measurement result in RRC_CONNECTED, the UE … before using the measured results for evaluation of reporting criteria and ….

The MAC entity performs the evaluation of reporting criteria as specified in … based on the L1 filtered measurement results from lower layer.

For L1 beam level event triggered measurements report, the network can configure … as trigger quantity.

Report Triggering:

...

As a result, RAN2 can initiate in the running CR draft text that includes the current status of UE initiated Beam Reporting using the LTM framework as high level specification template and indicate the rationale and progress in doing so to RAN1.

RAN2 start work in specifying event evaluation for UE-initiated beam reporting in the running 38.321 CR and inform RAN1 on its progress.

Moreover, in [Post129][208], it was left as FFS whether UEI BM parameters should be moved to CSI-MeasConfig. This should be discussed later once most of the UEI BM parameters are resolved, and thus can be postponed for now.

No need to address the FFS on “whether UEI BM parameters should be moved to CSI-MeasConfig” in RAN2-129-bis.

## 2.2	CSI support for up to 128 CSI-RS ports

For CSI-Type-I/II, it was discussed in [Post129][208] whether to use option 2 or 3 as below for group based CBSR (with the corresponding ASN-1 example):

Option 2: (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2;

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-codebookSubsetRestriction-r19		typeI-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeI-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (384)), thirtytwo BIT STRING (SIZE (192)), sixtyfour BIT STRING (SIZE (1024))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (192)), thirtytwo BIT STRING (SIZE (256)), sixtyfour BIT STRING (SIZE (512))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (96)), thirtytwo BIT STRING (SIZE (128)), sixtyfour BIT STRING (SIZE (256))},

four-two-r19           CHOICE {twentyfour BIT STRING (SIZE (48)), thirtytwo BIT STRING (SIZE (64)), sixtyfour BIT STRING (SIZE (128))},

four-four-r19          CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))}

}

typeII-codebookSubsetRestriction-r19		typeII-X1-X2-CBSR-r19							 OPTIONAL    -- Need R

typeII-X1-X2-CBSR-r19 ::=     CHOICE {

one-one-r19            CHOICE {twentyfour BIT STRING (SIZE (24)), thirtytwo BIT STRING (SIZE (32)), sixtyfour BIT STRING (SIZE (64))},

two-one-r19            CHOICE {twentyfour BIT STRING (SIZE (12)), thirtytwo BIT STRING (SIZE (16)), sixtyfour BIT STRING (SIZE (32))},

two-two-r19            CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-one-r19           CHOICE {twentyfour BIT STRING (SIZE (6)), thirtytwo BIT STRING (SIZE (8)), sixtyfour BIT STRING (SIZE (16))},

four-two-r19           CHOICE {thirtytwo BIT STRING (SIZE (4)), sixtyfour BIT STRING (SIZE (8))}

}

Option 3: (N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING.

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two, four-four}	OPTIONAL    -- Need R

typeII-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two}	OPTIONAL    -- Need R

typeI-codebookSubsetRestriction-r19		BIT STRING (SIZE (24, 32, 48, 64, 96, 128, 192, 256, 384,   512, 1024))							 OPTIONAL    -- Need R

typeII-codebookSubsetRestriction-r19		BIT STRING (SIZE (4, 6, 8, 12, 16, 24, 32, 64))		OPTIONAL    -- Need R

The structure in option 3 is more intuitive to follow and with minimized design. It was raised in the email discussion, however, whether this type of variable bit string length is feasible in ASN.1. After running syntax check on the option 3 above, we can confirm this option is not feasible since the SIZE structure should have a range rather than fixed values. Therefore, option 2 is preferred.

CSI-typeI/II group based CBSR is defined as option 2 i.e. (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2.

It should be noted, however, that an FFS was added to typeII-CBSR-r19 on whether additional bits are needed, so the group-based CBSR parameters for type II can only be implemented once the FFS is resolved. For typeI-softScalingRank, since the structure is similar, this field can follow the same as agreed for CSI-typeI/II.

The same structure defined for CBSR CSI-typeI/II is also used for typeI-softScalingRank.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	No need to address the FFS on “whether UEI BM parameters should be moved to CSI-MeasConfig” in RAN2-129-bis.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 start work in specifying event evaluation for UE-initiated beam reporting in the running 38.321 CR and inform RAN1 on its progress.

Proposal 2	CSI-typeI/II group based CBSR is defined as option 2 i.e. (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2.

Proposal 3	The same structure defined for CBSR CSI-typeI/II is also used for typeI-softScalingRank.