3GPP TSG-RAN WG2 Meeting #129bis	R2-2502562

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.7.4.1

Source:	China Telecom

Title:	Considerations on LCP Enhancements

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129 meeting, the agreements on LCP enhancement were made as follow.

Only one additional priority is configured to an LCH for LCP enhancement. We keep an existing agreement (remaining time threshold is configured per LCH) There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination No additional PBR is needed for priority adjusted data FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

The paper aims to discuss the remaining issues on LCP enhancements and provide some considerations.

2 Discussion

In the last meeting, it has agreed that the additional LCP priority will be used for UL grant priority determination in intra-UE prioritization. But it is undetermined whether the additional LCP priority should be used for SR priority determination in intra-UE prioritization. Per TS38.321, the mechanism of intra-UE prioritization procedure is shown as follow.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

It can be observed that the priority of SR is determined by the priority of logical channel that triggers SR. If the LCH priority-adjusted data is in the corresponding logical channel, it is more reasonable to apply the additional priority for the SR. When the SR overlaps with other UL grant, additional priority will be used for SR priority determination in intra-UE prioritization.

Proposal 1: Additional priority is applied for SR in intra-UE prioritization.

In the current LCP mechanism, resource allocation is based on Bj, i.e. tokens in the bucket. Specifically, the UE maintains a variable Bj for each logical channel j. During the first round of resource allocation, UL grant resources are assigned to logical channels in decreasing order of Bj, where Bj >0. Subsequently, Bj is reduced by the total size of the MAC SDUs used in the MAC PDU for logical channel j. In the second round of resource allocation, the remaining UL resources are allocated to logical channels in strict decreasing order, without considering the size of Bj. In this way, the UE can maximize data transmission for high-priority logical channels.

Due to the Bj limitation, some LCH priority-adjusted data may not be allocated enough resources in the first round of resources. As a result, the LCH priority-adjusted data may be delayed or even be discarded. To address this issue, some companies proposed to allow LCH priority-adjusted data to be transmitted irrespective of whether Bj is negative or not.

However, there may exist some concerns if LCH with upgraded priority is allowed to transmitted if Bj < 0. For instance, it is assumed that both LCH1 and LCH2 have LCH priority-adjusted data, and the priority of LCH1 is higher than LCH2. If the LCH1 is allowed to be transmitted if Bj is negative, the UL grant may be allocated to LCH1 until all data in LCH1 is transmitted. If LCH1 also has non LCH priority-adjusted data, non LCH priority-adjusted data in LCH1 may be transmitted before the LCH priority-adjusted data in LCH2, which is unfair for LCH2 with LCH priority-adjusted data. Even the worse, the remaining UL grants may be insufficient for the LCH priority-adjusted data in LCH2.

Proposal 2: LCH with an upgraded priority is not allowed to be transmitted if Bj is negative.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: Additional priority is applied for SR in intra-UE prioritization.

Proposal 2: LCH with an upgraded priority is not allowed to be transmitted if Bj is negative.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece