3GPP TSG-RAN WG2 Meeting #129bis	R2-2502580
Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.5.3

Source:	InterDigital

Title:	On-demand SIB1 request and reception

Document for:	Decision

# Introduction

RAN approved a new work item on network energy saving (NES) [1], which was revised as [2]. The revised WID includes the following objective where study item objective was converted to the normative phase, as in the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN2 made the following agreements in RAN2 #129 meeting [3]

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

This contribution discusses various aspects to support on-demand SIB1 for UEs in idle/inactive mode.

# UL-WUS configuration for On-demand SIB1

Triggering for on-demand SIB1 is based on use of UL WUS signal of an existing signal/channel. One important step to enable on-demand SIB1 transmissions is the provisioning of UL WUS configuration for the UEs which are currently in idle/inactive mode.

When Cell A is providing the UL-WUS configuration for one or more NES cells, e.g., to support the scenario 1a, the Cell A may transmit UL-WUS configuration as part of the system information.

RAN2 #127Bis [4] agreed to provide UL-WUS configuration through a new SIB.

UL-WUS configuration will allow the UE to identify the NES cell, as agreed in RAN2#126 [6] through inclusion of PCI and frequency as part of the UL-WUS configuration. Based on the received UL-WUS configuration, the UE can transmit the UL WUS to the NES cell. Thus, at minimum, the UL-WUS configuration provides the RACH configuration for the NES cell. The RACH configuration should provide the suitable RO and preamble for requesting the OD-SIB1 based on the SSBs being transmitted by the NES cell, as agreed in RAN2#125Bis [7].

RAN2 agreed to provide UL WUS configuration through a new SIB. RAN2 #128 discussed the topic of providing UL WUS configuration as part of RRC release message, and following working assumption was made: UL-WUS configuration in RRC release is not supported.

In our understanding, if the network has the mechanism to provide UL WUS configuration through SIB, it does not need additional handling of providing the same configuration as part of RRC release message. This may require additional handling of timing and validity for the configurations provided through release message as well. Based upon this, we have the following proposal:

Proposal 1:	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

# RA procedure for WUS transmission

In the discussion for the MAC running CR [2], some companies discussed the preamble backoff behavior, whether the UE should apply preamble backoff or not, and whether the network can signal the preamble backoff value to a non-zero value. The following editor’s note was thus added:

Editor’s note: FFS whether any changes are required for handling preamble backoff during SIB1 request procedure.

It is worth noting that section 5.1.1 already specifies that the default backoff value for a RA procedure is already set to 9:

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall: […] 1> set the PREAMBLE_BACKOFF to 0 ms;

Only if the NW signals a different backoff value, the UE uses the signalled value in the backoff indication sub-PDU. if the NW doesn’t indicate any backoff in the RAR, the following text is applied and 0 backoff is applied:

1> else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded: 2> if the Random Access Response contains a MAC subPDU with Backoff Indicator: 3> set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI. 2> else: 3> set the PREAMBLE_BACKOFF to 0 ms.

the current text can therefore be kept as is, and we can rely on network implementation w.r.t. whether to signal a backoff or not.

Proposal 2:	No changes are required for handling preamble backoff during a RA procedure for SIB1 request.

Another issue that was raised during the running CR discussion, is whether RO for OD-SIB1 request can be shared with other usages of random access [2]. It was discussed that in case the RO is shared among WUS and other RA preambles, Msg2 transmitted by the NES cell may include SIB1 request ACK for the NES UE and SI request ACK and/or RAR for other UEs in the NES cell. If the SIB1 request ACK for the NES UE is after an SI request ACK (1-byte long) or RAR (1+7=8 byte long) for other UEs in Msg2 (Random access response MAC PDU), the NES UE might not know the start position of its SIB1 request ACK in the MAC PDU. The following editor’s note was thus added:

Editor’s note: whether any changes are required for when the UE can select any SSB for SIB1 request in case there are no SSBs measured above rsrp-SIB1ThresholdSSB is available, dependent on RAN1.

In our understanding, shared ROs between OD-SIB1 and other RACH use cases was an option studied by RAN1 during the SI phase. Therefore, we can rely on the network to configure OD-SIB1 RO/preamble is separate from other RACH use cases. If the RO is shared, the SIB1 ACK can be known to the UE when it’s in front.

Proposal 3:	Rely on network to handle separation of RO/preamble used for SIB1 request from other RACH use cases.

Another issue that was raised during the running CR discussion, is whether any changes are required for when the UE can select any SSB for SIB1 request in case there are no SSBs measured above rsrp-SIB1ThresholdSSB is available [2]. It was mentioned that it is still open in RAN1 whether all the SSBs will be configured with OD-SIB1 RA resources, and thus this editor’s note was added:

Editor’s note: whether any changes are required for when the UE can select any SSB for SIB1 request in case there are no SSBs measured above rsrp- SIB1ThresholdSSB is available, dependent on RAN1.

However, the OD-SIB1 request is triggered by cell reselection, and thus the quality of the cell has been measured by the UE prior to the RA initiation. Similar to RA for Other SI request, the UE should select any SSB if no SSB is above the configured threshold, and this can be left to UE implementation.

Proposal 4:	Similar to RA for Other SI request and other RA procedures, the UE selects any SSB if no SSB is above the configured RSRP threshold (which SSB the UE selects is left to UE implementation).

Another issue that was raised during the running CR discussion, is whether there is a need to clarify that a preamble can be transmitted and RAR received on “SpCell or a cell supporting SIB1 request” instead of just “SpCell”. It is worth noting that RA for initial access (attach) is already supported with this text, in IDLE mode. There is no strong view, but it can be understood that SpCell can include a cell on which the UE is requesting OD-SIB1. This avoids the need to define what is a “cell that supports OD-SIB1” in the MAC specification.

Proposal 5:	From a MAC specifications perspective, RAN2 confirms the understanding that RA on an SpCell can include a cell on which the UE is requesting OD-SIB1. No specification changes needed.

# Conclusions

The following proposals are made in this document for on demand SIB1 acquisition in IDLE/Inactive states:

Proposal 1:	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

Proposal 2:	No changes are required for handling preamble backoff during a RA procedure for SIB1 request.

Proposal 3:	Rely on network to handle separation of RO/preamble used for SIB1 request from other RACH use cases.

Proposal 4:	Similar to RA for Other SI request and other RA procedures, the UE selects any SSB if no SSB is above the configured RSRP threshold (which SSB the UE selects is left to UE implementation).

Proposal 5:	From a MAC specifications perspective, RAN2 confirms the understanding that RA on an SpCell can include a cell on which the UE is requesting OD-SIB1. No specification changes needed.

# References

RP-242353, Revised WID: Enhancements of network energy savings for NR, September 2024

R2-2502584, “Summary of Comments to 38.321 CR for NES”, InterDigital