3GPP TSG-RAN WG2 Meeting #129bis	R2-2502596

Wuhan, China, April 07th – 11th, 2025

Agenda item:	8.5.3

Source:		Lenovo

Title:			Remaining OD SIB1 issues

Document for:	Discussion and Decision

# Introduction

This document examines the remaining SIB1 OD parts.

# Discussion

Following agreements were reached in the last RAN2 meeting (#129):

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

As one remaining part, RAN2 needs to discuss how a R19 NES UE acquires SIB1 from a NES Cell. UE needs to check if SIB1 is currently broadcasted before it could go ahead and make SIB1 Request from the NES cell, as agreed in one previous meeting:

8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Therefore, if the ssb-SubcarrierOffset (as part of kSSB), is used for special purpose and can’t really provide a valid SSB Subcarrier offset, the kSSB should be provided along with the WUS configuration by the Cell A. We assume here that the pdcch-ConfigSIB1 can be used directly from the MIB of the NES cell i.e., RAN1 has no need to repurpose this IE.

Proposal 1: The kSSB value is provided in the new agreed SIB of Cell A along with already agreed WUS Configuration.

## SIB1 reception after making Request

RAN1 has agreed to the following with regards to on-demand SIB1 transmission:

Agreement The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR. - FFS: Use starting or ending slot of the RAR window as reference time Agreement The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration. - Unit of the duration is in slot - FFS: Starting time

After having made the SIB1 request, UE just attempts receiving SIB1 broadcast as in the previous releases i.e., the UE can use SI-RNTI to decode PDCCH and use kSSB and pdcch-ConfigSIB1 received in MIB of the NES Cell.

Proposal 2: UE uses SI-RNTI to decode PDCCH in the time window signalled in the RAR for UL WUS to receive on-demand SIB1 of the NES Cell.

Proposal 3: KSSB value from Cell-A and the pdcch-ConfigSIB1 received in MIB of a NES Cell is used to receive on-demand SIB1 of the NES Cell.

Finally, since some of the proposals affect RAN1 progress, if agreed an LS to RAN1 should be sent to inform them of our progress.

Proposal 4: An LS to RAN1 should be sent to inform them of our progress.

# Conclusion

Remaining essential issues were discussed in the paper for the on-demand SIB1 topic. Following proposals are made:

Proposal 1: The kSSB value is provided in the new agreed SIB of Cell A along with already agreed WUS Configuration.

Proposal 2: UE uses SI-RNTI to decode PDCCH in the time window signalled in the RAR for UL WUS to receive on-demand SIB1 of the NES Cell.

Proposal 3: KSSB value from Cell-A and the pdcch-ConfigSIB1 received in MIB of a NES Cell is used to receive on-demand SIB1 of the NES Cell.

Proposal 4: An LS to RAN1 should be sent to inform them of our progress.

# References

[1]: RAN1 meeting minutes from RAN2#129