3GPP TSG-RAN2 #129bis	R2-2502603

Wuhan, China, Apr.  7th – 11th, 2025

Agenda Item:		8.6.4

Source:		Ofinno

Title:	Discussion on open issues of CLTM

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss open issues about CLTM.

# L1 execution condition and L3 execution condition

In RAN2 #129 meeting, RAN2 agreed either L1 execution condition or L3 execution condition is configured for a CLTM candidate cell

Agreements on C-LTM: 1. From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

A network may configure a L1 execution condition for an CLTM candidate cell A and a L3 execution condition for an LTM candidate cell B. A UE evaluates both execution conditions. the UE may determine both execution conditions being fulfilled. The L3 execution condition is more reliable than the L1 execution condition. The UE would be better to select the CLTM candidate cell B.

Proposal 1: When more than one CLTM triggered candidate cells exist, a UE selects a candidate cell associated with L3 execution condition.

# CG resource for CLTM

A network may allocate a configured grant (CG) resource when transmitting to a UE an LTM configuration. The network may need to keep allocating the CG resource of a candidate cell until the UE execute an LTM cell switch procedure for the candidate cell. The UE may not execute the LTM cell switch procedure for the candidate cell. The CG resource is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

Observation 1: a CG resource of a candidate cell for CLTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

The network may need to release or update the CG resource (e.g., to avoid the waste of the CG resource). For releasing or updating the CG resource, the network needs to transmit an RRC reconfiguration message comprising an LTM configuration. It causes signal overheads and is not desired for subsequent CLTM without the RRC reconfiguration.

Observation 2: it causes signal overheads and is not desired for subsequent CLTM without the RRC reconfiguration for an RRC reconfiguration message to release or update a CG resource of the candidate cell.

Proposal 2: RAN2 discuss dynamic management of CG resource of candidate cell for CLTM.

# MAC CE for Early TA acquisition

In RAN2 #128, RAN2 agreed that a network may transmit new MAC CE to inform the candidate cell’s TA information to UE via the new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. The network can transmit the new MAC CE in response to receiving a RA preamble from a UE. The network may need to include more than one TA values. The network may transmit the new MAC CE to manage the TA of a candidate cell (e.g., for restarting TA timer or updating TA value). To avoid signal overheads, it would be better to allow the new MAC CE to include more than one TA values.

Proposal 3: RAN2 to agree new MAC CE to include more than one TA values.

In RAN2 #129, RAN2 agreed that the UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition. It is also understood that the UE completes an RA procedure after transmitting an RA preamble for early uplink sync, similar to Rel-18 LTM. If at least one execution condition is fulfilled for a candidate cell and if the UE has completed RA procedure for the candidate cell but has not yet received the MAC CE informing the candidate cell’s TA, then the UE has two options:

Option 1: UE does not have a valid TA for the candidate cell, since it has not received the MAC CE informing the candidate cell’s TA. So, the UE initiates a RACH-based CLTM cell switch.

Option 2: UE waits for a duration of time to receive the MAC CE informing the candidate cell’s TA. If the duration of time expires and the UE still does not receive the MAC CE, UE initiates RACH-based cell switch. If the UE receives the MAC CE within the duration of time, UE initiates a RACH-less cell switch.

Option 1 has the benefit of slightly simpler implementation, but it also adds overhead in terms of performing two RA procedures (one during early uplink sync and one for RACH-based cell switch) to obtain TA for the candidate cell. It also increases cell switch interruption, which is a drawback, since one of the main goals of LTM is to reduce this interruption. Option 2 has the benefit of using RACH-less cell switch by waiting for the MAC CE. During this waiting time, the UE is still connected to the source cell and is therefore not wasting time. Also, the duration of time can be controlled by the network through RRC signaling, so that network can ensure UE does not wait for an unnecessarily long time. For instance, this duration can also be set to 0, in which case, an identical behavior to Option 1 can also be achieved.

Proposal 4: After transmitting RA preamble for early uplink sync, UE waits for a duration of time to receive the MAC CE. The duration of time can be RRC configured.

# ID of LTM candidate configuration

In Rel-18, RAN2 agreed that a UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message. For CLTM, a UE needs to include the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

Proposal 5: RAN2 to confirm that for CLTM, a UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

A candidate cell may be configured for a normal LTM and a conditional LTM. The same LTM configuration identifier of the candidate cell is configured for the normal LTM and the conditional LTM. When a network receives an RRCReconfigurationComplete message including the applied LTM candidate configuration identifier, the network may not be able to know which LTM between the normal LTM and the CLTM the UE executed. For example, the network may transmit an LTM cell switch command for the normal LTM to a UE, but the UE executes the CLTM (e.g., without receiving the LTM cell switch command). Based on executing the CLTM, the UE transmits an RRC reconfiguration complete message comprising the applied LTM candidate configuration identifier. The network receiving the applied LTM candidate configuration identifier may consider that the UE executed the normal LTM while the UE executed the CLTM. The network may update/release LTM configuration parameters. The network may update/release wrong LTM configuration parameters.

Proposal 6: RAN2 to discuss whether a UE executing CLTM includes a CLTM indication within the RRCReconfigurationComplete message.

# Fast MCG failure recovery

In Rel-18, a UE that is configured with LTM configuration for intra-CU LTM cells can also be configured with fast MCG Recovery based on T316 timer. If RLF is detected on a UE capable of supporting fast MCG recovery, and if T316 is configured, the UE initiates fast MCG recovery procedure. Otherwise, the UE initiates re-establishment procedure. Furthermore, if fast MCG recovery fails (i.e., T316 expires), the UE initiates re-establishment procedure.

According to current specification, TS 38.331, the UE continues measuring candidate cells and neighbor cells when MCG failure recovery is ongoing. If the UE is configured with conditional LTM, this can lead to a case where UE satisfies an execution condition corresponding to a condition LTM candidate cell and trigger LTM cell switch. This leads to a misalignment in status of current PCell between the UE and network. The network assumes the PCell has RLF and attempts to recover from the RLF while the UE has changed to a different PCell (i.e., based on conditional LTM execution). To avoid this problem, the UE has to stop conditional LTM evaluation during fast MCG recovery.

The conditional LTM evaluation may be performed by RRC (i.e., if execution condition is based on L3 measurements) or by MAC (i.e., if execution condition is based on L1 measurements). Therefore, stopping the conditional LTM evaluation may be done separately by RRC or MAC based on the configured execution conditions.

Proposal 7: Stop conditional LTM evaluation when fast MCG recovery is ongoing. Based on the execution conditions configured for the candidate cells, the LTM evaluation is stopped by RRC and/ or MAC layer.

# Observations and Proposals

L1 execution condition and L3 execution condition

- Proposal 1: When more than one CLTM triggered candidate cells exist, for selection, a UE prioritizes a candidate cell associated with L3 execution condition over a candidate cell associated with L1 execution condition.

CG resource for CLTM

- Observation 1: a CG resource of a candidate cell for CLTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

- Observation 2: it causes signal overheads and is not desired for subsequent CLTM without the RRC reconfiguration for an RRC reconfiguration message to release or update a CG resource of the candidate cell.

- Proposal 2: RAN2 discuss dynamic management of CG resource of candidate cell for CLTM.

MAC CE for Early TA acquisition

- Proposal 3: RAN2 to agree new MAC CE to include more than one TA values.

- Proposal 4: After transmitting RA preamble for early uplink sync, UE waits for a duration of time to receive the MAC CE. The duration of time can be RRC configured.

ID of LTM candidate configuration

- Proposal 5: RAN2 confirm that for CLTM, a UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

- Proposal 6: RAN2 discuss whether a UE executing CLTM includes a CLTM indication within the RRCReconfigurationComplete message.

Fast MCG failure recovery

- Proposal 7: Stop conditional LTM evaluation when fast MCG recovery is ongoing. Based on the execution conditions configured for the candidate cells, the LTM evaluation is stopped by RRC and/ or MAC layer.