3GPP TSG-RAN WG2 Meeting #129bis		R2-2502606

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item	: 8.6.3

Source	: Fraunhofer HHI, Fraunhofer IIS

Title	: LTM event-triggered measurement reporting

Document for	: Discussion

# Introduction

- RAN2#129 agreed the following [1]:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". - Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). - Do not create a new resource configuration just for the event triggered measurements. - Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused". - Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report. - Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue. - TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval. - Cell specific offset is not supported for LTM L1 event evaluation. - The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity. - Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. - Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam. - R18 differential RSRP encoding is used as baseline. - LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant. - Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)) - Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP - Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur. - For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used. - S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

RAN2#128 agreed the following [2]:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Support the truncated measurement report MAC CE.

RAN2#127bis agreed the following [3]:

- MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3. - For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed. - For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed. - The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. - TTT operates only based on a timer (like TTT used in L3 event triggered MR). - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. - Additional information included in MR MAC CE: The information and quantity of current beam, based on NW configuration. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission.

RAN2#127 agreed the following [4]:

- Event LTM1 is not defined. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - R19 LTM event triggered measurement configuration can be taken as baseline: LTM measurement resource configuration is provided in LTM-config. Event triggered report config is provided in serving cell config. - MAC layer handles the event evaluation and measurement report triggering. - Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2#126 agreed the following [5]:

1. Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. 2. For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. 3. Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

This paper discusses further aspects related to measurement enhancement for LTM.

# LTM Event Reporting Procedures

## Reported Beams

During RAN2#129, the following agreements were made:

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam

According to the agreement, explicit indication in MAC CE is required for the network to distinguish which beams fulfill the event condition and which do not. There are two possible approaches. The first approach allows the UE to explicitly indicate the number of beams that meet the event condition (X). Since the total configured number of beams (N) includes all beams in the MR MAC CE for event-triggered L1 LTM measurement reporting, the remaining beams (N-X) are those that meet the leaving condition. In this approach, beams fulfilling the event condition are reported first, followed by those that do not. The second approach introduces a flag for each reported beam, enabling the network to differentiate between those that meet the event condition and those that do not. Since the first approach is more efficient in reducing overhead, we support this method.

Proposal 1: the UE includes the number of beams that fulfill the event condition (X) in MAC CE. Given that the total number of beams is N, the remaining beams (N-X) represent those that meet the leaving condition. Beams fulfilling the event condition are reported first.

## Report Configuration

RAN2#128 agreed that a single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. This decision can be evaluated by considering scenarios where the UE is configured to measure both SSB and CSI-RS, as these reference signals serve distinct purposes. For example, SSB is primarily used for evaluating cell quality over wider beams, making it suitable for initial cell search and broad coverage assessments. CSI-RS, on the other hand, provides higher resolution measurements due to its use of narrower beams, making it more effective for precise channel state information.

Given these differences, we propose that the gNB can configure the UE to report both SSB and CSI-RS in a single measurement report, ensuring a more comprehensive network evaluation while significantly reducing overhead. Instead of requiring separate reports for different reference signals, consolidating them into a single report minimizes signaling overhead, improving overall network efficiency. This approach also optimizes resource utilization. Additionally, a unified reporting structure simplifies processing for both the UE and gNB, reducing complexity and enhancing system performance.

Proposal 2: UE is configured to report both CSI-RS and SSB beams within a single measurement report.

According to the agreement in the previous RAN2 meeting, for event-triggered L1 LTM measurement reporting, the network controls whether beams that do not satisfy the event criteria can still be reported, following the N-beam configuration in MR MAC CE. Different scenarios and options should be considered for these N beams to ensure flexibility and efficiency in reporting.

In the existing LTM reporting framework, the following constraints apply:

- A maximum of four reference signals can be reported per cell.

- Up to four cells may be reported in a single L1 measurement report instance.

These specifications are outlined in TS 38.212 v18.2.0:

The number of reported cells and beams per cell is determined by configuration, meaning the UE always reports a fixed number of beams, four, for each configured cell. However, this approach is inefficient for several reasons.

First, some cells have stronger signals than others, making equal reporting unnecessary. Second, RAN2 has agreed that the NW controls if the beam(s) not satisfying the event can still be reported. This may necessitate reporting more than four beams per cell in certain scenarios to ensure a more comprehensive measurement.

Therefore, we propose allowing the number of beams per cell to vary. The UE should have the capability to report a different number of beams for different cells, improving flexibility and efficiency.

Proposal 3: For event-triggered L1 LTM measurement reporting, the number of beams (M) reported can vary for each cell, where N = M × L gives the total number of reported beams.

Similarly, this approach can also be applied to the number of reported cells (L).

Proposal 4: For event-triggered L1 LTM measurement reporting, the total number of reported cells (L) can vary, maintaining the total reported beams as N = M × L.

## Differential RSRP Reporting

RAN2#129 agreed that R18 differential RSRP encoding is used as baseline. For measurement metric representation, a differential format approach can be used to minimize the overall report message size. Reporting one absolute value for the largest RSRP of SSB or CSI-RS should be considered to ensure an accurate reference point for differential reporting. One absolute value for the largest RSRP of the SSB or CSI-RS measured beam and the remaining measured beams using differential L1-RSRP values are reported.

Proposal 5: Configure the UE to report measurements in a differential L1-RSRP format to reduce overhead.

- A single absolute value is reported for the highest RSRP of the SSB and/or CSI-RS beam.

- Use differential L1-RSRP values for the remaining measured beams,

- FFS further quantization details, e.g., step size, range, bit width.

## Prohibit timer

In RAN2#128, the following was agreed:

TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

However, there is a possibility that another beam may meet the event conditions shortly after the measurement report for the first event has been sent. If a new measurement report is triggered immediately whenever an event condition is satisfied, the overhead could increase significantly due to the high frequency of event-triggered measurement reports.

To mitigate this issue, prohibit timer should be introduced. This timer would begin once the first measurement report for a triggered event has been transmitted. By implementing this mechanism, the event-triggered measurement report mechanism can operate more efficiently by controlling the frequency of report transmissions.

Figure 1: Prohibit Timer

Proposal 6: Configure the UE with a prohibit timer that regulates the transmission of measurement reports with the following behavior:

- The prohibit timer defines the minimum duration that must pass before a new measurement report can be sent after the previous one.

- The timer starts when the first measurement report is triggered, preventing any subsequent event-triggered measurement reports from being transmitted until the timer expires.

## RSRP Measurement Reporting

The reporting range for SS-RSRP and CSI-RSRP in L3 reporting, along with the mapping of measured quantities, is defined in Table 10.1.6.1-1 of TS 38.133. Since this table is already used for the R18 LTM measurement report by CSI, it can be reused for LTM event-triggered measurement reporting.

Currently, LTM event-triggered measurements require an efficient and standardized way to report SS-RSRP and CSI-RSRP values. Reusing the existing reporting range and mapping definitions from TS 38.133 ensures alignment with current measurement frameworks, reduces implementation complexity, and maintains compatibility across different reporting layers. This approach also minimizes the need for additional specifications or modifications in reporting methodologies.

Proposal 7: The L1 reporting measured quantity values for SS-RSRP and CSI-RSRP should be utilized for LTM event-triggered measurement reporting at higher layers, e.g., L2.

# Conclusion

Proposal 1: the UE includes the number of beams that fulfill the event condition (X) in MAC CE. Given that the total number of beams is N, the remaining beams (N-X) represent those that meet the leaving condition. Beams fulfilling the event condition are reported first.

Proposal 2: UE is configured to report both CSI-RS and SSB beams within a single measurement report.

Proposal 3: For event-triggered L1 LTM measurement reporting, the number of beams (M) reported can vary for each cell, where N = M × L gives the total number of reported beams.

Proposal 4: For event-triggered L1 LTM measurement reporting, the total number of reported cells (L) can vary, maintaining the total reported beams as N = M × L.

Proposal 5: Configure the UE to report measurements in a differential L1-RSRP format to reduce overhead.

- A single absolute value is reported for the highest RSRP of the SSB and/or CSI-RS beam.

- Use differential L1-RSRP values for the remaining measured beams,

- FFS further quantization details, e.g., step size, range, bit width.

Proposal 6: Configure the UE with a prohibit timer that regulates the transmission of measurement reports with the following behavior:

- The prohibit timer defines the minimum duration that must pass before a new measurement report can be sent after the previous one.

- The timer starts when the first measurement report is triggered, preventing any subsequent event-triggered measurement reports from being transmitted until the timer expires.

Proposal 7: The L1 reporting measured quantity values for SS-RSRP and CSI-RSRP should be utilized for LTM event-triggered measurement reporting at higher layers, e.g., L2.

# References

[1] Chair notes, 3GPP RAN2#129, Athens, Greece,17th – 21st, 2025

[2] Chair notes, 3GPP RAN2#128, Orlando, USA,18th – 22nd, 2024

[3] Chair notes, 3GPP RAN2#127bis, Hefei, China, Oct 14th – 18th, 2024.

[4] Chair notes, 3GPP RAN2#127, Maastricht, NL, August 19th – 23rd, 2024.

[5] Chair notes, 3GPP RAN2#126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.