3GPP TSG-RAN WG2 Meeting #129bis	R2-2502617

Wuhan, China, April 7 - 11, 2025

Agenda item:	8.1.2.3

Source: 	Qualcomm Incorporated

Title: 	LCM for positioning use case

Document for:		Discussion and Decision

# 1.	Introduction

At RAN2#129, the following agreements were made on "LCM for Positioning use case" [1]:

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extension can be discussed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

In this contribution, we discuss the remaining aspects from the agreements above.

-	"Switching" and "fallback" between AI/ML and non-AI/ML methods.

-	LMF control for "applicable functionality reporting"/LPP provide capabilities.

# 2.	Discussion

## 2.1	Switching and fallback between AI/ML and non-AI/ML methods

Per agreement from RAN2#129 summarized in section 1 above, "AI/ML Positioning Case 1" is introduced as a new positioning method. This agreement essentially determines the specification work required for both, TS 38.305 (Stage 2) and TS 37.355 (LPP). Upon receiving a location request from an AMF, the LMF may perform the necessary operations to obtain the location of the target UE as described in clause 5.2 of TS 38.305 [2].  The LMF then decides on the positioning method(s) to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, etc. as described in clause 5.4.4 of TS 38.305 [2]. Similar, per clause 4.3.8 of TS 23.273, the functions of an LMF include [3]:

"Determine type and number of position methods and procedures based on UE and PLMN capabilities, QoS, UE connectivity state per access type, LCS Client type, co-ordinate type and optionally service type and indication of requiring reliable UE location information."

Therefore:

Observation 1:	The positioning method(s) used to determine the location of a target UE are always selected by an LMF.

Note, Observation 1 also holds for an MO-LR for basic self-location [3].

TR 38.843 [4] distinguishes between "model switching" and "fallback operation". For example:

Model switching: Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific AI/ML-enabled feature. Management instruction: Information needed to ensure proper inference operation. This information may include selection/(de)activation/switching of AI/ML models or AI/ML functionalities, fallback to non-AI/ML operation, etc.

"Model switching" for positioning may be needed dependent on the scenario and UE implementation, e.g., in the case of a change of area where a trained model is applicable. In certain implementations, an AI/ML model may have been trained for different scenarios and may then rely on model switching based on the applicable scenario where the UE is currently located, e.g., different area validity of the assistance data, etc. For the positioning use case, this "model switching" (if applicable in an implementation) however, is transparent to the network and does not imply a change of selected positioning method(s).

Observation 2:	"Model switching" for the positioning use case depends on UE implementation.

Observation 3:	The "switching" terminology in TS 38.843 usually refers to a change of "AI/ML models.

The "switching between AI/ML and non-AI/ML methods" is usually referred to as "fallback operation". If the LMF selects one or more LPP based positioning methods, the LMF would send an LPP Request Location Information message to the target UE indicating the required positioning method or methods. If the UE is requested/allowed to use "AI/ML positioning" in a LPP Request Location Information message, the UE cannot autonomously select a non-AI/ML positioning method to return a location result to the LMF. To not change the existing LCS framework (which would have multiple implications incl. SA2 impacts), it is proposed to remove the "FFS" from RAN2 #129 in the agreement and clarify that "switching" in this context is synonymous to "fallback":

Proposal 1:	UE autonomous switching/fallback between AI/ML and non-AI/ML methods is not allowed.

However, Proposal 1 does not mean that "switching/fallback between AI/ML and non-AI/ML methods" is never allowed/applicable. Rather, the "switching/fallback" is controlled by the LMF. All the currently defined LPP-based positioning methods can be used individually or in combination (the latter is also referred to as "hybrid positioning") as defined in e.g., TS 38.305 clause 4.3 [2].

In LPP, the Request Location Information message is a SEQUENCE of requested positioning method(s). For example, the UE may receive a location request for GNSS and DL-TDOA positioning. The UE may then combine, all the GNSS and DL-TDOA results, if possible, and determine a single location estimate for the target UE. However, it may also be possible that a UE is in a blocked environment (e.g., indoors) and may not receive GNSS signals in this example but may be able to receive sufficient DL-PRS signals from base stations. The UE would then determine the location estimate via DL-TDOA only. That is, a UE may "switch" between allowed/requested positioning methods to report a location estimate best fulfilling the requested QoS, however, this "switching" is always among the positioning methods requested.

Proposal 2: 	Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

With respect to the "FFS" in the agreement from RAN2#129 (see section 1, agreement #3)

"FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,"

we understand that the "UE autonomous switching between AI/ML and non-AI/ML methods" is unconditional, since when multiple positioning methods are included in a LPP Request Location Information message, any "switching/fallback" is not considered "UE autonomous". I.e., the UE is still required to determine a location with the requested positioning method or methods. Therefore, we believe no further agreements are needed on this aspect if Proposals 1/2 above are agreeable.

## 2.2	LMF control for "applicable functionality reporting"/LPP provide capabilities

For agreement #6 in section 1:

"UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality."

there is still the question whether LMF control is needed for this functionality or not:

"FFS if any additional LMF control is needed."

The discussions on this aspect essentially started at RAN2#126, where the following initial agreement was made [5]:

"The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality."

and at RAN2#127bis [6]:

"FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively)."

This issue was then discussed during "[POST128][026][AIML]" email discussion [7], where the vast majority believes that such an LMF control is not needed. The motivation for an "LMF control" seems primarily to avoid that a UE is sending unsolicited LPP Provide Capabilities messages frequently.

In general, providing updated positioning capabilities to an LMF is an existing LPP/LCS feature. Once an LPP session has been instigated by an LMF (i.e., a Correlation/Session ID is available), a UE may at any time send an appropriate UL LPP message to the LMF (i.e., LPP Provide Capabilities message in this case) [2], [8]. Examples of UE capability changes were previously given by RAN2 in [9] and are repeated below:

-	Radio configuration dependency: Positioning capabilities based on current/active radio configuration are not static (e.g., the srs-PosResourceConfigCA-BandList in TS 37.355 [8] is provided for the current configured CA band combination).

-	Power Savings: A (e.g., IoT) UE whose battery level is low may switch off positioning support in order to conserve battery power for more important tasks such as communicating with an external server or may report lower processing capabilities.

-	Processing Resources Constraints: The available processing resources (processors, memory, etc.) may be shared between "communication" and "positioning operations".  If the "communication operation" requires increased processing resources, the resources allocated to the "positioning operation" may temporarily be reduced.

-	Operation as a U2N Remote UE: A UE may transmit a ProvideCapabilities message when it starts or stops operation as a U2N Remote UE [8].

Further, as agreed by RAN2 [9]:

"RAN2 would like to point out that the above are examples and there may be other situations in which the UE positioning capability may change."

Observation 4:	A change in UE positioning capabilities can happen at any time during an LPP session. A UE can provide its updated capabilities via unsolicited LPP Provide Capabilities message.

Therefore, with respect to unsolicited LPP Provide Capabilities messages, no changes should be required, given that this is an existing functionality.

Specifically for AI/ML positioning, a UE may generally support AI/ML positioning, however, a UE may not know a-priori whether AI/ML positioning is indeed possible/applicable in the current situation (location session). One of the factors that influence the support of AI/ML positioning is the current UE location, which however, may only be known after the UE has obtained some location information or after some assistance data indicating an indirect UE location (e.g., TRP locations in assistance data) have been received. The UE must have a trained and valid model available for the area where the UE is currently located and the "training condition" must match the current "network condition". This is generally also captured by the following agreement from RAN2#126 [5]:

"The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it."

But even if the UE has a trained and valid model available for the applicable area, the model performance may not meet the requested QoS in a LPP Request Location Information message (e.g., requested positioning accuracy). Therefore, based on the available model performance monitoring result, the "applicable functionality" may also change once a LPP Request Location Information message has been received.

Observation 5: 	Factors that influence the support of AI/ML positioning ("applicable functionality") in an ongoing LPP positioning session include the general availability of AI/ML models, the current UE location area, matching training and inference conditions, and the performance of the available models.

In the baseline positioning procedure agreed at RAN2#127bis and illustrated in Figure 1 below, it is assumed that a UE can infer from the assistance data whether AI/ML positioning is available/applicable in the area where the UE is currently located and whether the "training conditions" are consistent with the current "network condition". However, this may also happen after step 5 when the UE is aware that AI/ML positioning is indeed requested and additional information on the location request is available (e.g., requested QoS). There seems no case/situation where the available/applicable functionality (or capability) changes back and forth, such that a UE repeats sending unsolicited LPP Provide Capabilities messages "endlessly" after assistance data/location request has been received. Similar for periodic services, a UE may not be able to continue supporting AI/ML positioning when moving into an area where no AI/ML model is available (which is in principle not different to any other positioning method; e.g., GNSS and UE is moving deep into a concrete tunnel). In that case, a UE may send an Event Report with error indication and could include updated LPP Provide Capabilities on "available/applicable functionality" (i.e., the reporting follows in any case the Event Report configuration, since a UE cannot send an LPP message outside an Event Report in this case). The LMF may then take appropriate actions (e.g., change of positioning method or abort the session, etc.).

Observation 6: 	There seems no case/situation where the available/applicable functionality (or capability) changes back and forth, such that a UE repeats sending unsolicited LPP Provide Capabilities messages "endlessly" after assistance data/location request has been received.

However, an unknown factor in this discussion is still a missing and precise definition of what the LPP Provide Capabilities message for AI/ML positioning will include (i.e., what the "functionality" terminology comprises in terms of signalling). Therefore, it is proposed to "close" this issue from RAN2 perspective but possibly revisit this issue when the details of the LPP Provide Capabilities message for AI/ML positioning become clear (i.e., when the RAN1 capability/parameter list ("functionality") has been received). If there is a case where certain parameters in a LPP Provide Capabilities message for AI/ML positioning are expected to change rather frequently during an LPP session, some LMF control for "applicable functionality" reporting could be considered. Therefore, we propose a small modification to the agreement #6 in section 1:

Proposal 3:	The UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of "applicable functionality". Additional LMF control is not needed but can be revisited when the RAN1 feature/parameter list has been received (e.g., if it turns out that parameters are expected to change rather frequently during an ongoing LPP session).

Figure 3: Generic LPP positioning procedure.

# 3.	Summary

In this contribution, we discussed some remaining LCM aspects for AI/ML positioning accuracy enhancements. The following observations and proposals were made.

"Switching" and "fallback" between AI/ML and non-AI/ML methods:

Observation 1:	The positioning method(s) used to determine the location of a target UE are always selected by an LMF.

Observation 2:	"Model switching" for the positioning use case depends on UE implementation.

Observation 3:	The "switching" terminology in TS 38.843 usually refers to a change of "AI/ML models".

Proposal 1:	UE autonomous switching/fallback between AI/ML and non-AI/ML methods is not allowed.

Proposal 2: 	Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation".

LMF control for "applicable functionality reporting"/LPP provide capabilities:

Observation 4:	A change in UE positioning capabilities can happen at any time during an LPP session. A UE can provide its updated capabilities via unsolicited LPP Provide Capabilities message.

Observation 5: 	Factors that influence the support of AI/ML positioning ("applicable functionality") in an ongoing LPP positioning session include the general availability of AI/ML models, the current UE location area, matching training and inference conditions, and the performance of the available models.

Observation 6: 	There seems no case/situation where the available/applicable functionality (or capability) changes back and forth, such that a UE repeats sending unsolicited LPP Provide Capabilities messages "endlessly" after assistance data/location request has been received.

Proposal 3:	The UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of "applicable functionality". Additional LMF control is not needed but can be revisited when the RAN1 feature/parameter list has been received (e.g., if it turns out that parameters are expected to change rather frequently during an ongoing LPP session).

# References

[1]	RP-250310, "Status Report to TSG", RAN#107.

[2]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[4]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[5]	RP-241153, "Status Report to TSG", RAN#104.

[6]	RP-242559, "Status Report to TSG", RAN#106.

[7]	R2-2500968, " Report of [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)".

[8]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[9]	R2-2108960, "Response LS on storage of UE Positioning Capabilities", RAN2#115.