3GPP TSG-RAN WG2 Meeting #129-bis					R2-2502623

Wuhan, China, April. 7–11, 2025

Source:	Continental Automotive

Title:                       Discussion on re-access for A-IoT

Agenda Item:         8.2.3

Document for:	Discussion and Decision

# Introduction

In the RAN2#129 meeting, the following agreements were made for success/failure indication of D2R messages [1].

Agreement on success/failure indication

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

In this document, we will further discuss whether a timer is needed or an explicit message from the reader after the device sends Msg3 to the reader.

# Discussion

According to the RAN2#129 agreement, if the reader does not successfully receive Msg3 from the device, it will send a NACK to the device, helping the device determine whether to re-attempt sending Msg3. However, if the reader successfully receives Msg3 from the device, it will not send an explicit ACK. This is to avoid sending too many ACKs to the devices, as the majority of D2R transmissions are successful. This approach helps to reduce ACK signalling overhead from the reader.

Additionally, after the device sends Msg3, it does not know whether it needs to monitor follow up R2D message (e.g., a command). If the reader does not send any information to the device, the device will continue monitoring transmission from the reader. To prevent the device from continuously monitoring unnecessarily, we believe that a timer or an explicit message from the reader would be useful.

If we choose the timer option, the UE will start a timer after sending the Msg3 transmission to the reader. While the timer is running, the reader can send a follow-up message to the device. If the UE does not receive a message from the reader within this period, it will implicitly assume that reader has no further message to send. Furthermore, if the UE does not receive a NACK for the D2R message corresponding to Msg3 within time, it will implicitly assume an ACK. In cases where the reader has sent a NACK, but the device does not receive it due to poor coverage, the device will still assume an ACK. Later, the reader will send a subsequent paging message to trigger re-access.

Figure 1: Timer for monitoring follow-up R2D data

Similarly, if we choose the explicit indication option, the reader sends an explicit indication to the device if it has a follow-up message to transmit. This indication will be sent after the reader successfully receives Msg3 from the device.

However, in this option, the reader must always send an explicit indication, regardless of whether it has a follow-up message or not. The reason is that if the reader only sends an indication when it has a follow-up message, the device will not know how long it should continue monitoring for a message from the reader. For instance, if the reader has no follow-up message and does not send an explicit indication, the device will require an additional mechanism, such as a timer, to determine when to stop monitoring for messages. Therefore, in our opinion, an explicit indication alone, without a timer, would not work. For this reason, we propose that the timer option is more beneficial for the device to monitor follow-up messages from the reader.

Proposal 1: The device will implicitly assume an ACK if it does not receive a NACK while the timer is running. If the reader sends a NACK but the device misses it due to poor coverage, the reader will send a subsequent paging message to trigger re-access.

Proposal 2: A timer is beneficial for the device to determine whether the reader has follow-up R2D data to send or not.

# Conclusions

In this document, we will further discuss whether a timer is needed or an explicit message from the reader after the device sends Msg3 to the reader. Additionally, we request RAN2 to discuss the following proposals:

Proposal 1: The device will implicitly assume an ACK if it does not receive a NACK while the timer is running. If the reader sends a NACK but the device misses it due to poor coverage, the reader will send a subsequent paging message to trigger re-access.

Proposal 2: A timer is beneficial for the device to determine whether the reader has follow-up R2D data to send or not.

# References

[1] RAN2#129 meeting chair note