3GPP TSG-RAN WG2 #129bis	R2-2502627

Wuhan, China, 07th – 11st April 2025

Agenda Item:	8.3.2.2 Model Generalization

Source:	Ericsson

Title:	Generalization of the AI/ML models for RRM prediction

Document for:	Discussion

# Introduction

The study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 [1]. In this contribution, generalization aspects of the RRM measurement prediction models are discussed and the relevant simulation results are included.

The generalization of an AI/ML model refers to how well such model can adapt what it has learned from the training data to make accurate predictions using new data.

In RAN2#128 meeting, the following agreements have been taken about model generalization aspects [2]:

Agreements on generalization - Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized. 4 Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can chose to study frequency domain prediction cases and report what they have simulated. 5 Study generalization over UE speeds 6 The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h. FFS on combinations 7 The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h. FFS on combinations

Besides the generalization aspects related to UE speed for in temporal domain prediction, as agreed in RAN2#128, the generalization over the frequency domain predictions and network deployment are also discussed in this contribution.

# Discussion

## Generalization over deployment (UMa - UMi)

It was agreed in the RAN2#129 meeting that the two cell configurations below can be considered for evaluating the generalization over cell configurations for both FR1 and FR2 scenarios.

Table 1: generalization parameters for both FR1 and FR2

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

In the results provided below, we used the configurations from Table 1.

### Temporal prediction case B, RRM sub case 2

The following combinations were evaluated for FR1 temporal prediction case B.

Table 2 Combinations for FR1 temporal prediction case B simulation

|  | Training @Dataset: UMi(FR1) | Training @Dataset: UMa(FR1) | Inference @UMi(FR1) | Inference @UMa(FR1) |
| --- | --- | --- | --- | --- |
| Configuration#1 | Yes |  | Yes |  |
| GC#1 |  | Yes | Yes |  |
| GC#2 | Yes | Yes | Yes |  |
| Configuration#2 |  | Yes |  | Yes |
| GC#1 | Yes |  |  | Yes |
| GC#2 | Yes | Yes |  | Yes |

In the simulation, the following prediction patterns were used and different UE moving speed, e.g., 30km/h, 60km/h, 90km/h were considered.

Figure 1 Prediction patterns with MRRT 50% and 80% used in simulation for generalization over Cell configuration in FR1

MRRT 50% and 80% were simulated, and simulation results are shown in below tables.

Table 3. MRRT 50%, averaged RSRP difference for simulation combinations listed in Table 2

| Case@(train dataset/test dataset) | MRRT 50% |  |  |
| --- | --- | --- | --- |
|  | Averaged RSRP difference @30km/h | Averaged RSRP difference @60km/h | Averaged RSRP difference @90km/h |
| Configuration#1@(UMi/UMi) | 0.144 | 0.192 | 0.268 |
| GC#1@(UMa/UMi) | 0.254 | 0.225 | 0.220 |
| GC#2@(MixUMiUMa/UMi) | 0.149 | 0.152 | 0.204 |
| Configuration#2@(UMa/UMa) | 0.143 | 0.148 | 0.155 |
| GC#1@(UMi/UMa) | 0.367 | 0.302 | 0.222 |
| GC#2@(MixUMiUMa/UMa) | 0.101 | 0.111 | 0.154 |

Table 4．MRRT 80%, averaged RSRP difference for simulation combinations listed in Table 2

| Case@(train dataset/test dataset) | MRRT 80% |  |  |
| --- | --- | --- | --- |
|  | Averaged RSRP difference @30km/h | Averaged RSRP difference @60km/h | Averaged RSRP difference @90km/h |
| Configuration#1@(UMi/UMi) | 0.405 [0.159, 0.301, 0.480, 0.680] | 0.539 [0.200, 0.400, 0.644, 0.913] | 0.618 [0.221, 0.463, 0.742, 1.045] |
| GC#1@(UMa/UMi) | 0.418 [0.157, 0.310, 0.498, 0.707] | 0.550 [0.208, 0.410, 0.656, 0.927] | 0.644 [0.259, 0.485, 0.762, 1.070] |
| GC#2@(MixUMiUMa/UMi) | 0.402 [0.151, 0.300, 0.480, 0.679] | 0.545 [0.198, 0.407, 0.653, 0.921] | 0.627 [0.229, 0.468, 0.752, 1.059] |
| Configuration#2@(UMa/UMa) | 0.227 [0.086, 0.167, 0.271, 0.384] | 0.316 [0.121, 0.234, 0.375, 0.532] | 0.38 [0.147, 0.283, 0.452, 0.637] |
| GC#1@(UMi/UMa) | 0.381 [0.355, 0.348, 0.377, 0.445] | 0.662 [0.538, 0.591, 0.687, 0.832] | 0.709 [0.474, 0.592, 0.770, 1.000] |
| GC#2@(MixUMiUMa/UMa) | 0.227 [0.102, 0.170, 0.266, 0.373] | 0.316 [0.113, 0.232, 0.379, 0.540] | 0.397 [0.173, 0.296, 0.465, 0.655] |

Note: Averaged RSRP difference for different prediction steps is demonstrated in the square brackets.

Observation on GC#1

For study goal 1, a model trained with FR1-UMa data generalizes well in a FR1-UMi scenario with RSRP difference lower than 1dB.

For study goal 1, a model trained with FR1-UMi data generalizes well in a FR1-UMa scenario with RSRP difference lower than 1dB.

Observation on GC#2

For study goal 1, comparable performance is obtained for a model trained with a dataset mixed from FR1-UMa and FR1-UMi and the baseline where the model is trained with a dataset of FR1_UMa and inference on a dataset of FR1-UMa.

For study goal 1, comparable performance is obtained for a model trained with a dataset mixed from FR1-UMa and FR1-UMi and the baseline where the model is trained with a dataset of FR1_UMi and inference on a dataset of FR1-UMi.

### Temporal prediction case A, RRM sub case 2

The following combinations were evaluated for FR2 temporal prediction case A:

Table 5. Combinations for FR2 temporal prediction case A simulation

|  | Training @Dataset: UMi(FR2) | Training @Dataset: UMa(FR2) | Inference @UMi(FR2) | Inference @UMa(FR2) |
| --- | --- | --- | --- | --- |
| Configuration#1 | Yes |  | Yes |  |
| GC#1 |  | Yes | Yes |  |
| GC#2 | Yes | Yes | Yes |  |
| Configuration#2 |  | Yes |  | Yes |
| GC#1 | Yes |  |  | Yes |
| GC#2 | Yes | Yes |  | Yes |

In the simulation, the following prediction patterns were used and different UE moving speed, e.g., 60km/h, 90km/h, 120km/h were considered.

Figure 2. Prediction patterns with ratio 1 used in a simulation for generalization over Cell configuration in FR2

Table 6. Ratio 1, averaged RSRP difference for simulation combinations listed Table 5

| Case@(train dataset/test dataset) | Ratio 1 |  |  |
| --- | --- | --- | --- |
|  | Averaged RSRP difference @60km/h | Averaged RSRP difference @90km/h | Averaged RSRP difference @120km/h |
| Configuration#1@(UMi/UMi) | 0.87 [0.259, 0.521, 0.838, 1.138, 1.550] | 0.991 [0.273, 0.594, 0.967, 1.363, 1.757] | 1.072 [0.312, 0.657, 1.055, 1.462, 1.876] |
| GC#1@(UMa/UMi) | 0.983 [0.260, 0.562, 0.935, 1.364, 1.793] | 1.162 [0.367, 0.712, 1.125, 1.578, 2.030] | 1.200 [0.336, 0.723, 1.169, 1.639, 2.133] |
| GC#2@(MixUMiUMa/UMi) | 0.845 [0.220, 0.475, 0.805, 1.169, 1.554] | 1.001 [0.258, 0.581, 0.969, 1.384, 1.814] | 1.093 [0.328, 0.661, 1.067, 1.49, 1.917] |
| Configuration#2@(UMa/UMa) | 1.082 [0.323, 0.659, 1.059, 1.478, 1.893] | 1.266 [0.382, 0.790, 1.254, 1.723, 2.180] | 1.379 [0.418, 0.879, 1.378, 1.875, 2.345] |
| GC#1@(UMi/UMa) | 1.593 [0.744, 1.123, 1.569, 2.023, 2.506] | 1.736 [0.633, 1.138, 1.717, 2.284, 2.909] | 2.022 [0.880, 1.371, 1.958, 2.617, 3.280] |
| GC#2@(MixUMiUMa/UMa) | 1.068 [0.313, 0.643, 1.042, 1.462, 1.879] | 1.245 [0.363, 0.771, 1.231, 1.701, 2.158] | 1.378 [0.419, 0.874, 1.373, 1.874, 2.353] |

Note: Averaged RSRP difference for different prediction steps is demonstrated in the square brackets.

Observation on GC#1

For study goal 2, a model trained with FR2-UMa data generalizes well in FR2-UMi scenario with RSRP difference lower than or around 1 dB.

For study goal 2, a model trained with FR2-UMi data generalizes in FR2-UMa scenario with a cost of additional RSRP difference of more than 0.5 dB.

Observation on GC#2

Comparable performance is obtained for a model trained with dataset mixed from FR2-UMi and FR2-UMa and the baseline where the model is trained with dataset of FR2_UMi and inference on dataset of FR2_UMi.

Comparable performance is obtained for a model trained with dataset mixed from FR2-UMi and FR2-UMa and the baseline where the model is trained with dataset of FR2_UMa and inference on dataset of FR2_UMa.

### Frequency prediction generalization over UMa/UMi configurations

In this subsection, we investigate frequency prediction (2 GHz – to – 4 GHz) generalization when training on UMi and infer on UMa, versus training on UMa and infer on UMi. Here are some definitions for the results in Table 7:

- Baseline: train on 2 GHz to predict 4 GHz @UMa or UMi  2 GHz to predict 4 GHz@UMa or Umi.

- GC#1: train on 2 GHz to predict 4 GHz @UMi  Generalize over 2 GHz to predict 4 GHz@UMa

- GC#2: train on 2 GHz to predict 4 GHz @UMi & UMa Generalize over 2 GHz to predict 4 GHz@UMi

It is worth noting that if it not mentioned otherwise, the bandwidth (BW) of the 2GHz carrier is 20Mhz, whereas the BW of 4GHz is 100Mhz. Also, if the generalization case is not mentioned, then it is considered to be GC#2. Because in previous submission we have established already that GC#2 outperformed GC#1, so we did not repeat comparison (between GC#2 and GC#1) for all the following insightful empirical experiments.

Table 7. Comparison of model capability of UMI Validation results vs UMA Generalization results.

| Row | Task | Model | Input@2GHz | Output @4GHz | RSRP difference (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | Baseline | xgboost | 9 Cells | 9 Cells | 0.12 |
| 2 | Baseline | Conv2D | 9 Cells | 9 Cells | 2.17 |
| 3 | GC#2 | Conv2D | 9 Cells | 9 Cells | 2.66 |
| 4 | GC#2 | xgboost | 9 Cells | 9 Cells | 3.15 |

Table 7 compares the frequency generalization capabilities of two models—XGBoost and Conv2D CNN—under both baseline (same configuration for training and inference) and generalization conditions (training on UMi and testing on UMa). The key performance metric is the RSRP difference (dB), which reflects the degradation when transitioning from validation (UMi) to inference (UMa).

In Table 7, we observe that row-1 and row-4 show the training and generalization performance of xgboost model, whereas row-2 and row-3 represent the training and generalization performance of Conv2D model. The analysis in Table 5 highlights contrasting strengths between XGBoost and Conv2D. While XGBoost excels in the baseline setting (RSRP error = 0.12 dB), it exhibits substantial degradation under generalization (GC#2), reaching an RSRP error of 3.15 dB when inferring on UMa after training on UMi. Conversely, Conv2D, though suboptimal in the baseline (2.17 dB), demonstrates better cross-environment generalization with a smaller degradation (2.66 dB). This suggests that Conv2D captures more transferable features across urban deployment scenarios, whereas XGBoost may be overfitting to in-domain patterns, limiting its real-world applicability in unseen environments.

The model which predicts the best may not necessarily generalize the best.

Table 8. Comparing variable input/output size for generalization on UMI (while trained on UMA)

| Row | Task | ModelStrType | InputCells@2GHz | Output-Cells@4GHz | RSRP difference (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | GC#2 | Conv2D | 9 Cells | 1 Cells | 1.00 |
| 2 | GC#2 | Conv2D | 9 Cells | 3 Cells | 1.03 |
| 3 | GC#2 | Conv2D | 9 Cells | 9 Cells | 1.15 |
| 4 | GC#2 | Conv2D | 21 Cells | 21 Cells | 1.17 |
| 5 | GC#2 | Conv2D | 9 Cells | 21 Cells | 7.43 |
| 6 | GC#2 | Conv2D | 1 Cells | 9 Cells | 8.98 |

Table 8 illustrates the sensitivity of the Conv2D model to the spatial configuration of input and output cells under the GC#2 generalization scenario, trained on UMa and tested on UMi. Results indicate that performance remains robust when input and output cell sizes are matched or input size is higher than output, with output has corresponding collocated cells at input, (e.g., 9→1 or 9→9 or 21→21 cells), with RSRP differences under 1.2 dB. However, significant degradation occurs when there is a strong mismatch between input and output cell dimensions, with RSRP errors exceeding 7 dB. These findings underscore the importance of spatial alignment in designing model input-output configurations and highlight the limited extrapolation capability of Conv2D architectures when presented with spatially mismatched scenarios.

Given a fixed input (number of cells), the larger the cluster of output the more difficult to generalize.

A larger number of input cells always improves the generalization.

Table 9. Comparing generalization results of UMi-to-UMa vs UMa-to-UMi

| Row | Task | Model | Generalization | InputCells@2GHz | OutputCells@4GHz | RSRP difference (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | GC#2 | xgboost | UMa UMi | 9 Cells | 3 Cells | 2.4 |
| 4 | GC#2 | xgboost | UMa UMi | 9 Cells | 9 Cells | 2.4 |
| 5 | GC#2 | xgboost | UMi UMa | 9 Cells | 3 Cell | 2.7 |
| 6 | GC#2 | xgboost | UMi UMa | 9 Cells | 9 Cells | 2.7 |

Table 9 provides a comparative view of XGBoost's frequency generalization performance between UMa-to-UMi and UMi-to-UMa transfer scenarios. The results suggest a slight but consistent edge for UMa-to-UMi generalization, with RSRP degradation of 2.4 dB compared to 2.7 dB in the reverse direction. This may be attributed to the richer and more heterogeneous propagation conditions present in UMa environments, enabling models to learn more generalizable patterns. Furthermore, the output cell granularity (3 vs. 9 cells) shows minimal effect on performance, underscoring the resilience of XGBoost to moderate variations in output spatial resolution under fixed input conditions.

UMa to UMi achieves slightly better generalization performance in comparison with UMi to UMa.

Table 10. Comparison of GC#1 to GC#2 generalization in case of UMa to UMi.

| Row | Task | env | InCell | OutCell | RSRP difference (dB) @xgboost | RSRP difference (dB) @Conv2D |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | GC#2 | UMaUMi | 9 Cells | 3 Cell | 2.56 | 2.49 |
| 2 | GC#1 | UMaUMi | 9 Cells | 3 Cell | 19 | 19.7 |
| 3 | GC#2 | UMiUMA | 9 Cells | 3 Cell | 3.16 | 2.75 |
| 4 | GC#1 | UMiUMA | 9 Cells | 3 Cell | 20 | 20 |

Table 10 provides a comprehensive comparison of GC#1 and GC#2 generalization strategies across both model types (XGBoost and Conv2D) and directionality (UMa→UMi vs. UMi→UMa). Results confirm that GC#2 consistently enables effective frequency generalization with RSRP degradation below 3.2 dB, while GC#1 fails catastrophically across the board, exceeding 19 dB. Conv2D slightly outperforms XGBoost under GC#2, indicating its advantage in capturing transferable spatial-frequency features. Additionally, UMa-to-UMi generalization proves marginally easier than the reverse, suggesting that training in macro environments imparts broader generalization capacity.

When generalizing for frequency prediction from UMa to UMi (or vice-versa), GC#2 achieves reasonable performance. However, GC#1 has highly unsatisfactory performance.

Focus on GC#2 when generalizing over UMi and UMa in frequency predictions.

We would also like to indicate that depending on several factors, including UE capabilities, network states, and traffic heterogeneity, it is highly likely that UE operates on different BWs. Reusing frequency prediction model across different bandwidth configurations is a realistic scenario. However, as shown in Table 11, the model’s generalizability performance degrades when the bandwidth is different between the input frequency and the predicted frequency. How the network’s operating status impact the predicted RSRP values can be further studied.

The model’s generalizability performance degrades when the bandwidth is different between the input frequency and the predicted frequency.

Table 11. Comparison of fixed bandwidth vs different bandwidth for 2GHz (BW=20MHz) 4GHz (BW=100MHz)

| Row | Task | Model | Env | Input@2GHz Output@4GHz | BW@2GHz BW@4GHz | RSRP difference (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Baseline | Xgboost | UMa | 9 9 Cells | Fix-BW 2020MHz | 0.08 |
| 2 | Baseline | Xgboost | UMa | 9 9 Cells | Variable-BW 20100MHz | 0.14 |
| 3 | GC#2 | Xgboost | UMaUMi | 9 9 Cells | Fix-BW 2020MHz | 1.6 |
| 4 | GC#2 | Xgboost | UMaUMi | 9 9 Cells | Variable-BW 20100MHz | 8.71 |

## Temporal prediction on L3 filtered beam-level measurements

In contribution R2-2501789, the way forward on the study of beam-level predictions is discussed. Below, some simulation results for L3 filtered beam-level predictions are shown.

Simulation results

We use L3 filtered beam-level measurement results for both model input and the output. Specifically, we use 8 beam-level results sampled every 80ms at each time instance depicted with { …, T0-1, T0} in Figure 2. As shown, the observation window varies from 1x80ms to 10x80ms to predict the L3 filtered beam-level results which is (N x 80 ms), wherein N ∈{1, 2, 3, 4, 5}, starting right after the observation window T0. For example, as the case of prediction ratio 1/5 illustrated in Figure 2, we predict L3 filtered beam-level results of 8 beams at multiple time instances in the period of T0+1 ~T0+5 by using L3-filtered beam-level RSRP measurements of 8 beams sampled at time instance T0 as model input. For the case of ratio 2 illustrated in Figure 2, we predict L3 filtered beam-level results of 8 beams at multiple time instances in the period of T0+1 ~T0+5 by using L3-filtered beam-level RSRP measurements of 8 beams sampled at time instance T0~T0-4 as model input.

Figure 2: L3 filtered beam level measurement predictions from L3 filtered beam level measurement results; Case 2 and Case A. L3 filtered beam level measurement sampling rate is 80ms.

We evaluate the prediction accuracy (measured as RSRP difference) with different prediction window size that is a multitude of N, which leads to prediction window size between 80ms to 400ms. The evaluation results are provided in the following.

Table 1. Case A, sub-case 2: RSRP difference (prediction error) for L3 filtered beam-level measurement prediction from L3-filtered beam level measurement results.

| Cases | Averaged RSRP difference @60km/h | Averaged RSRP difference @90km/h | Averaged RSRP difference @120km/h |
| --- | --- | --- | --- |
| Sample AndHold | 2.179 [0.805, 1.556, 2.243, 2.866, 3.428] | 2.425 [0.911, 1.749, 2.505, 3.180, 3.782] | 2.582 [0.978, 1.871, 2.670, 3.380, 4.009] |
| AI/ML (ratio 1/5) | 1.633 [0.824, 0.945, 1.512, 2.140, 2.742] | 1.824 [0.861, 1.071, 1.721, 2.411, 3.054] | 1.931 [0.798, 1.176, 1.873, 2.577, 3.228] |
| AI/ML (ratio 3/5) | 1.423 [0.362, 0.808, 1.384, 1.988, 2.573] | 1.634 [0.409, 0.947, 1.613, 2.285, 2.916] | 1.776 [0.425, 1.047, 1.774, 2.489, 3.146] |
| AI/ML (ratio 5/5) | 1.423 [0.367, 0.813, 1.385, 1.984, 2.565] | 1.650 [0.449, 0.970, 1.625, 2.290, 2.917] | 1.807 [0.461, 1.080, 1.804, 2.515, 3.173] |
| AI/ML (ratio 10/5) | 1.443 [0.412, 0.838, 1.399, 1.993, 2.572] | 1.669 [0.443, 0.986, 1.646, 2.319, 2.948] | 1.801 [0.482, 1.081, 1.793, 2.499, 3.148] |

Note: Model structure (i.e., 1 LSTM layer+2 linear layer) used for this L3 filtered beam-level measurement prediction simulation is similar as we used in RRM measurement temporal prediction Case A-Case 2.

The evaluation results captured in Table 1, shows that the sample and hold on baseline can predict the RSRP value with up to 1dB difference compared to the ground truth value, when prediction window size is less than 160ms. For ratio 1/5 case which has the same OW length as the baseline, it shows that in a scenario with UE movability set to 60km/h AI/ML can predict the RSRP value with up to 1 dB difference when prediction is less than 320ms. In a higher mobility scenario (i.e., when UE speed is 90km/h, 120km/h), AI/ML can predict the RSRP value with up to 1 db difference when prediction is less than 160ms.

When OW length is increased, it shows that in a scenario with UE mobility set to 60 km/h or 90km/h, AI/ML can predict the RSRP value with up to 1dB difference when PW is less than 320ms. In a higher mobility scenario (i.e., when UE speed is 120 km/h) the RSRP difference/error is slightly higher.

In addition, increasing OW length can improve improving prediction accuracy when OW length is small. When OW length exceeds a certain value (i.e., 3x80ms), increasing OW length has minor impact on prediction accuracy.

And comparing to the L3 cell-level measurement prediction in FR2 Case A-Case2[4], there is ~0.1 db degradation on prediction accuracy for L3 filtered beam-level measurement prediction.

Therefore, we have the following observations for L3 filtered beam predictions:

To some extent, increasing the OW length helps improving the prediction accuracy.

Maximum prediction window is below 240ms for UE moving speed 60km/h&90km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Maximum prediction window is below 160ms for UE moving speed 120km/h if we consider 1 db as acceptable threshold for RSRP difference.

L3 filtered beam-level measurement prediction performance is comparable to L3 filtered cell-level measurement prediction

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	For study goal 1, a model trained with FR1-UMa data generalizes well in a FR1-UMi scenario with RSRP difference lower than 1dB.

Observation 2	For study goal 1, a model trained with FR1-UMi data generalizes well in a FR1-UMa scenario with RSRP difference lower than 1dB.

Observation 3	For study goal 1, comparable performance is obtained for a model trained with a dataset mixed from FR1-UMa and FR1-UMi and the baseline where the model is trained with a dataset of FR1_UMa and inference on a dataset of FR1-UMa.

Observation 4	For study goal 1, comparable performance is obtained for a model trained with a dataset mixed from FR1-UMa and FR1-UMi and the baseline where the model is trained with a dataset of FR1_UMi and inference on a dataset of FR1-UMi.

Observation 5	For study goal 2, a model trained with FR2-UMa data generalizes well in FR2-UMi scenario with RSRP difference lower than or around 1 dB.

Observation 6	For study goal 2, a model trained with FR2-UMi data generalizes in FR2-UMa scenario with a cost of additional RSRP difference of more than 0.5 dB.

Observation 7	Comparable performance is obtained for a model trained with dataset mixed from FR2-UMi and FR2-UMa and the baseline where the model is trained with dataset of FR2_UMi and inference on dataset of FR2_UMi.

Observation 8	Comparable performance is obtained for a model trained with dataset mixed from FR2-UMi and FR2-UMa and the baseline where the model is trained with dataset of FR2_UMa and inference on dataset of FR2_UMa.

Observation 9	The model which predicts the best may not necessarily generalize the best.

Observation 10	Given a fixed input (number of cells), the larger the cluster of output the more difficult to generalize.

Observation 11	A larger number of input cells always improves the generalization.

Observation 12	UMa to UMi achieves slightly better generalization performance in comparison with UMi to UMa.

Observation 13	When generalizing for frequency prediction from UMa to UMi (or vice-versa), GC#2 achieves reasonable performance. However, GC#1 has highly unsatisfactory performance.

Observation 14	The model’s generalizability performance degrades when the bandwidth is different between the input frequency and the predicted frequency.

Observation 15	To some extent, increasing the OW length helps improving the prediction accuracy.

Observation 16	Maximum prediction window is below 240ms for UE moving speed 60km/h&90km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Observation 17	Maximum prediction window is below 160ms for UE moving speed 120km/h if we consider 1 db as acceptable threshold for RSRP difference.

Observation 18	L3 filtered beam-level measurement prediction performance is comparable to L3 filtered cell-level measurement prediction

Based on the discussion in the previous sections we propose the following:

Proposal 1	Focus on GC#2 when generalizing over UMi and UMa in frequency predictions.

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

RAN2 Chairman’s Notes, RAN2#128