3GPP TSG RAN WG2 #129bis	R2-2502629

Wuhan, China, April 07-14, 2025

Agenda Item:	8.8.2
Source:	Toyota ITC
Title:	Downlink coverage enhancements and different SMTCs

Document for:	Discussion and Decision

# 1	Introduction

The Work Item on NR for NTN Ph3 was initially approved in December 2023 and then revised lastly in RAN#106 in [1]. It includes the objective on downlink coverage enhancements as follows:

The objectives of the work item are the following: - Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

The latest update of the WID is in line with the RAN1#118bis decision to extend the default SSB periodicity to 160 ms compared with the existing 20 ms for NTN DL_CE [2].

RAN2#128 and RAN2#129 made the following agreements (from [3] and [4]):

Agreements (RAN2#128): 1. As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs) 2. RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Agreements (RAN2#129): - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer. - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - - The number of SMTC/gaps a UE needs to consider at any time will not be increased further - From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. - RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). - RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

This document discusses a way forward for the above, i.e. how to extend the current SMTC configurations, while not increasing the number of SMTC/gaps that a UE needs to consider at any time.

# 2	Discussion

# 2.1	Additional configured SMTCs

## The current specifications allow four SMTC offsets with one periodicity, per frequency.

## The first question to answer is how-many periodicities are required to be configured in the UE (not necessarily activated) and/or if more than four SMTC offsets are required to be configured in the UE (not necessarily activated). One of the RAN2#129 agreements read “ (…) if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough”.

- The distinction between “configured” and “activated” has been made above in order to fulfill the RAN2#129 agreement “The number of SMTC/gaps a UE needs to consider at any time will not be increased further”. The activation of SMTCs is further considered in Clause 2.2 in this document.

## RAN2 should decide the number of SMTC offsets and the number of SMTC periodicity (per frequency) to be supported by an NTN DL Enhancement UE. RAN2 may be assisted by RAN1 for this.

## Observation 1: There is a need to decide the number of SMTC offsets and SMTC periodicity that can be configured (not necessarily activated) by a UE supporting NTN Downlink Enhancement. RAN2 may be assisted by RAN1 for this.

## Proposal 1: Decide between the two options below:
Option 1: RAN2 send an LS to RAN1 asking the number of SMTC offsets and SMTC periodicities that can be seen by a UE in an NTN DL Enhancement configuration.
Option 2: RAN2 decide on the number of  number of SMTC offsets and SMTC periodicities that can be seen by a UE in an NTN DL Enhancement configuration.

## Note that the above does not mean that the extended new number of SMTC offsets/periodicities will have to be monitored simultaneously by the UE. See clause 2.2 below for this.

# 2.2	Activation of SMTCs

## To avoid impacting the UE power consumption, RAN2#129 agreed that “The number of SMTC/gaps a UE needs to consider at any time will not be increased further”.

## One way forward to achieve the above is that the UE uses the UE location in order to decide which SMTCs can be considered as activated among the set of configured SMTCs. This is because the UE only needs to perform the measurements on the cells serving its area.

## It should be noted that since RAN2#129 already agreed that in some scenarios “the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough”, the decision related with Proposal 2 in Clause 2.2 below can be taken before that the exact number of the SMTC extension is taken in Proposal 1 in Clause 2.1.

## Proposal 2: When additional SMTCs are configured (compared with 4 SMTC offsets per frequency), the UE uses the UE location in order to select the SMTCs to be activated among the configured SMTCs. The maximum number of activated SMTCs at any time remains as it was in the legacy for the maximum number of supported SMTCs by the UE.

# 3.	Conclusion

In this document, we suggested a way forward for the SMTCs given the RAN2#129 agreements.

## Observation 1: There is a need to decide the number of SMTC offsets and SMTC periodicity that can be configured (not necessarily activated) by a UE supporting NTN Downlink Enhancement. RAN2 may be assisted by RAN1 for this.

## Proposal 1: Decide between the two options below:
Option 1: RAN2 send an LS to RAN1 asking the number of SMTC offsets and SMTC periodicities that can be seen by a UE in an NTN DL Enhancement configuration.
Option 2: RAN2 decide on the number of  number of SMTC offsets and SMTC periodicities that can be seen by a UE in an NTN DL Enhancement configuration.

## Proposal 2: When additional SMTCs are configured (compared with 4 SMTC offsets per frequency), the UE uses the UE location in order to select the SMTCs to be activated among the configured SMTCs. The maximum number of activated SMTCs at any time remains as it was in the legacy for the maximum number of supported SMTCs by the UE.

# 4.	References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #106 Madrid, Spain, 09-12 December 2024.

- R1-2409341 Report of RAN1#118bis meeting.

- R2-2410913, RAN2-128 Report from Break-out session on NR-NTN and IoT-NTN.  Session Chair (ZTE Corporation).

- R2-2501333, RAN2-129 Report from Break-out session on NR-NTN and IoT-NTN.  Session Chair (ZTE Corporation).