3GPP TSG-RAN WG2 Meeting #129bis                                    	R2-2502651

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.8.4

Source:	Qualcomm Incorporated

Title:	MBS broadcast service area information

Document for:	Discussion and Decision

# Introduction

In RAN#129, following agreements were made on MBS broadcast service.

Agreements:

- In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

- It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In this document, we provide signaling details for MBS broadcast service area information.

# Discussion

Signaling of MBS broadcast service area

It was agreed in RAN2#128 that a new SIB is introduced to broadcast the list of coverage area information. In our view, the UEs not supporting intended service area do not have to acquire the unnecessary part, i.e., coordinates, from MCCH channel or SIB20. Each area information in the list can be identified by An Area ID. It is possible to use the Area ID in MCCH for serving cell MBS broadcast information.

Followings are agreed in RAN2#129:

- In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

- It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH)

We provide example below how the service area IDs can be signalled.

One potential issue with this approach is that when the UE is not in the intended area, it might have to check MCCH to know if any change in geographical area information has occured. As RAN2 has made following agreements, we could also leave this to UE.

- RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

If UE is not in the intended service area, it is left to UE implementation whether or how often to monitor the MCCH notification.

Service continuity

In our view, UE can already map the MBS broadcast frequency to the associated MBS service through USD (User Service Description) or SIB21. It is to note that the UE needs to be aware of the frequency of the service it is interested in before it can receive the MBS broadcast service. The UE knows the interested frequency either from SIB21 or from USD. Therefore, it may be simple just to map the provided geographical area to one or more MBS broadcast service frequency.

The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

... ,

[[

mbs-CoverageAreaInfoList-r19             MBS-CoverageAreaInfoList-r19       OPTIONAL,  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

MBS-CoverageAreaInfoList-r19 ::=   SEQUENCE (SIZE (1..maxMBS-AreaInfo-r19)) OF MBS-CoverageAreaInfo-r19

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-SessionAreaId-r19		MBS-SessionAreaId-r19,

mbs-FSAI-List-r19           MBS-FSAI-List-r17, OPTIONAL,  -- Need R

}

-- TAG-SIB21-STOP

-- ASN1STOP

MBS FSAI (Frequency Selection Area Identity) is another way how the intended service area can be associated with MBS session. SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. The UE can obtain the FSAI of the MBS service from USD. A frequency can cover multiple MBS service Areas and the MBS FSAI could be different for the different service area. In this case, the service area can be associated with the MBS session simply by FSAI.

For service continuity, extend SIB21 to include the list of applicable geographical service area IDs and corresponding FSAI list.

Cell Reselection evaluation process

Currently a UE is allowed to prioritize a frequency if the UE can receive the intended MBS service in that frequency. From the SIB21, if the UE is also aware that it is currently not in the intended service area, then such prioritization rule should be relaxed. Therefore, we think it is better to clarify the cell reselection evaluation process may also consider the intended service area information. Text proposal for TS 38.304 is provided below.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled: 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20; 2) Either: - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service. NOTE 0g: It is up to UE implementation which frequency to select, when the USD provides multiple frequencies for the service the UE is interested in. If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell. NOTE 0h: Example scenarios in which such down-prioritisation may be needed include the cases where camping is not possible for the UE on the MBS broadcast frequency (e.g. the MBS broadcast frequency belongs to a PLMN different from UE's registered PLMN) while the UE can receive the MBS broadcast service when camped on another frequency than the MBS broadcast frequency or current frequency, or SIBxx provides list of MBS broadcast service area information and the UE determines it is not in the service area of the interested MBS broadcast service provided on the candidate frequencies. NOTE 0i: The frequency prioritization for MBS broadcast, NR sidelink communication, or V2X sidelink communication may override the re-selection priorities for slice-based cell reselection.

Clarify that Cell Reselection evaluation process may consider the intended service area information for the priority of the frequencies at which UE can receive the MBS broadcast service.

# Conclusion

Following proposals are made.

Proposal 1	If UE is not in the intended service area, it is left to UE implementation whether or how often to monitor the MCCH notification.

Observation 1.	The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

Proposal 2	For service continuity, extend SIB21 to include the list of applicable geographical service area IDs and corresponding FSAI list.

Proposal 3	Clarify that Cell Reselection evaluation process may consider the intended service area information for the priority of the frequencies at which UE can receive the MBS broadcast service.