3GPP TSG-RAN WG2 Meeting #129bis	R2-2502659

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.4.2

Source:	InterDigital, Inc.

Title:	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes

Document for:	Discussion

# 1. Introduction

In RAN2#127bis [1], following agreements were made:

Agreement Entry/exit condition If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). The LPWUS monitoring exit condition does not include MR measurements.

In this contribution, we discuss LP-WUS operation in RRC_IDLE/INACTIVE mode.

# 2. Discussion

Dedicated configuration for LP-WUS

For LP-WUS monitoring, an LP-WUR UE needs to receive LP-WUS configuration (e.g., LP-SS configuration, entry/exit condtions). RAN2 agreed to receive LP-WUS configuration via system information, however, a necessity of dedicated configuration is still FFS. From our view, dedicated LP-WUS configuration from a gNB is not valid since an idle/inactive LP-WUR UE is allowed to camp on another cell via cell (re-)selection procedure [2].

Proposal 1: Do not support dedicated LP-WUS configuration for idle/iacitve LP-WUR UEs.

Dedicated (de-)activation signaling for monitoring LP-WUS

When the UE is monitoring LP-WUS, the UE needs an additional latency to wake up MR and monitor PO from LP-WUS monitoring. The additional latency can be problematic for some use cases which are delay sensitive, e.g., the additional latency can be longer than the required delay. To avoid the additional latency, UE dedicated (de-)activation signaling needs to be supported. For example, when a UE is expected to receive DL data with delay sensitive traffic (e.g., XR) frequently, then the network can deactivate the LP-WUS monitoring (i.e., not to start LP-WUS monitoring) via the dedicated deactivation signaling so that the UE can receive the delay sensitive data  without additional wake-up delay from LP-WUS monitoring. This (de-)activation signlaing can be configured with the RRC release message, similar to one of the legacy configuration using for idle/inactive state (e.g., suspendConfig, cellReselectionPriorities) via the RRC release message.

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

Entry condition with relaxed LR measurements

Since both measurements (e.g., serving cell) via MR and LR can be considered to determine satisfying the entry condtion [1], RAN2 considers further whether/how to use both metrics via MR and LR (e.g., RSRP and (LP-)RSRP) for the determination when both MR-based and LR-based thresholds are configured. If RAN2 decides to consider always two metrics, additional power consumption with LR measurement and complexity of UE behavior (i.e., when to measure LP-RSRP via LR) cannot be avoided.

As a solution, RAN2 considers some conditions when to use metric(s) via MR measurement only with relaxed (or skipping) LR measurement for determining whether satysifying the entry condtion. For example, RAN2 can consider to use metric via MR only when the measurement configuration/parameters for MR and LR is similar (e.g., similar bandwidth, carrier frequency, band) that assumes results of MR measurement is similar to the results of LR measurement. For LR measurement relaxation, another MR-based threshold (e.g., entry condition with an offset) can be considered. For example, if measured value of MR measurement is good enough (e.g., cell-center) with satisfying the threshold, then the UE can start LP-WUS monitoring without utilizing LR-based measurement (e.g.,  given that LR-based threshold assumed to be satisfied).

Proposal 3: For satisfying LP-WUS entry condition, support to use MR measurement only with LR measurement relaxation (i.e., skipping LR measurement). Detail condition is FFS.

Exit condition with LR measurement

RAN2 agreed that entry/exit condition for LP-WUS monitoring is configured by SIB. To support entry/exit conditions for LP-WUS monitoring, the UE is configured with conditions. For example, a threshold of LP-RSRP/LP-RSRQ value of received LP-SS can be configured. The satisfied condition (e.g., above the threshold) of received LP-SS can be regarded as successful reception and the UE can keep monitoring LP-WUS. In contrast, if the condition is not satisfied by detecting number of reception failures (e.g., below the threshold of LP-RSRP) or measured value is below the threshold during a period of time (e.g., avoiding ping-pong effect), the UE stops LP-WUS monitoring and starts to monitor the legacy PO.

Proposal 4: For satisfying LP-WUS exit condition, support a number of LP-WUS reception failures.

Proposal 5: To avoid ping-pong effect, support to measure a quality of LP-WUS reception during a period time.

Measurement both NR-SS and LP-SS

While measuring of entry/exit conditions, a UE being capable of OFDM-based LP-WUR can measure both NR-SS (existing SSB) and LP-SS (i.e., if supported) simultaneously. The measured values can be used for the decision of entry/exit condition. Since the qualities of NR-SS and LP-SS are different metrics (e.g., RSRP and LP-RSRP, respectively), RAN2 should discuss how to use the different metrics for the decisions. For example, using the higher quality or combined quality.

Proposal 6: For OFDM-based WUS, support measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

Information for LP-WUS configuration

According to the evaluation results from the TR [3], it is obvious that offloading RRM measurement from MR to LP-WUS is essential to avoid frequent wake up of MR from ultra deep sleep and achieve power saving gain from LP-WUS. Considering limited capacity of LP-WUR and reduced power consumption, providing information on neighboring cells (e.g., whether to provide LP-WUS related configuration) should be considered for LP-WUR based RRM measurement. Also, this information may be beneficial that the UE can (re-)select cell with supporting LP-WUS configuration while performing the cell (re-)selection procedure.

Proposal 7: Information on neighboring cells (e.g., LP-WUS capable) should be supported.

# 3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: Do not support dedicated LP-WUS configuration for idle/iacitve LP-WUR UEs.

Proposal 2: Support dedicated LP-WUS monitoring (de-)activation per UE via RRC release message.

Proposal 3: For satisfying LP-WUS entry condition, support to use MR measurement only with LR measurement relaxation (i.e., skipping LR measurement). Detail condition is FFS.

Proposal 4: For satisfying LP-WUS exit condition, support a number of LP-WUS reception failures.

Proposal 5: To avoid ping-pong effect, support to measure a quality of LP-WUS reception during a period time.

Proposal 6: For OFDM-based WUS, support measuring both NR-SS and LP-SS quality measurement for entry/exit conditions of LP-WUS monitoring. How to use two different metrics (e.g., NR-SS and LP-SS) for decision is further discussed.

Proposal 7: Information on neighboring cells (e.g., LP-WUS capable) should be supported.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #127bis.

[2] 3GPP 38.304, NR User Equipment (UE) procedures in idle mode and RRC inactive state.

[3] TR 38.869 V2.0.0 (2023-12), “Study on low-power wake up signal and receiver for NR (Release 18)”.