3GPP TSG-RAN WG2 Meeting #129bis	R2-2502671

# Wuhan, China, Apr. 07th – 11th, 2025

Agenda item:		8.2.4 (Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1.	Introduction

This document discusses ambient IoT (A-IoT) data transmission. In this contribution, we discuss mainly AS ID, segmentation and data (re)transmission for failure handling. The related agreements in R2-129 meetings are as follows.

Agreements on AS ID - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2 Agreements on segmentation - To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. - Segment retransmission is supported. - For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 - R2D segmentation is not supported for R19 A-IoT. Agreements on procedures to consider From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”. Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

2.	Discussion

AS ID

For CBRA, it is agreed that it is up to the reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID in R2-129 meeting. The related discussion about how this is signalled, which message is used, and size of AS ID has not been concluded yet. During the post email discussion ([POST129][036][AIoT] AS ID), there seems to be a majority of companies that support that the AS ID size is same as size of the random ID (i.e., 16 bits). If a single AS ID size is considered, it should be 16 bits considering that the size of random ID is 16 bits. As another option, the size of AS ID can be adjusted based on the expected number of devices in the service area. To support it, the length of AS ID needs to be indicated. It will increase the complexity of an A-IoT device. Additional information (i.e., length of AS ID) will reduce the benefits of using a short AS ID. So, it is preferred that it has a single AS ID size and the size is 16 bits.

Proposal 1. The size of AS ID is 16 bits.

Based on the above-mentioned agreement, random ID can be used for AS ID. It can be the baseline to use random ID as AS ID. When random ID is used as AS ID, Msg2 can be interpreted as a signal that confirms or assigns random ID as AS ID. When the reader decides to assign a new AS ID after receiving Msg1, the new AS ID can be contained in Msg2. For other options, new message for AS ID assignment and using Msg4 for AS ID assignment are proposed. However, introducing a new message for AS ID assignment requires additional message exchanges and makes the procedures complicated. Using Msg4 may require including device ID in Msg4 along with AS ID to indicate the target device. Since device ID is usually longer than random ID, using Msg4 requires more resources than using Msg2. So, it is preferred that Msg2 is used for AS ID assignment for CBRA.

Proposal 2. For CBRA, AS ID is assigned via Msg2.

For CFRA, there are 3 options for AS ID assignment after R2-129 meeting and the AS ID assignment options for CFRA further discussed during the post email discussion ([POST129][036][AIoT] AS ID).

- Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID.

- Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2

- Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

Since there was little support for option 3 among 3 options, the remaining two options (i.e., option 2 and option 4) are discussed further.

AS ID is used for the purpose of D2R scheduling and R2D reception of at least command procedure part of ‘inventory + command’ procedure. It needs to be considered to allocate a valid AS ID between a reader and a device before R2D command message transmission. For CFRA, a command procedure seems to begin with Msg2 because Msg2 includes R2D command message. Therefore, the only way to allocate a valid AS ID before Msg2 is to use Msg1. It is option 2: the device includes a random ID in Msg1 along with device ID. The reader can use the AS ID from transmitting Msg2, which includes R2D command message.

In addition, if an AS ID collision happens exceptionally even for CFRA and it is really required to handle an AS ID collision scenario even in case of CFRA, option 4 can be considered to handle the AS ID collision scenario.

Proposal 3. For CFRA, option 2 (i.e., the device includes a random ID in Msg1) is used for AS ID assignment. The AS ID can be used from Msg2 (including R2D command message) transmission for R2D reception.

Considering D2R FDMA, Msg1 in random access can be transmitted in different frequencies from different A-IoT devices at the same time. If multiple Msg1 transmitted in the same frequency have the same random ID, it is a collision. However, even if Msg1 transmitted in different frequencies have the same random ID, it is not a collision. The random ID can be used as AS ID for D2R scheduling. On the other hand, TDMA is used for R2D. If random ID is used as AS ID for R2D reception without consideration about frequency, there may be confusion between A-IoT devices which choose different frequency, but select the same random ID. For example, an R2D transmission to a specific A-IoT device can be received by another A-IoT devices. To resolve this issue, another restriction may be considered. One possible restriction is that random IDs are required to be different across frequencies for contention resolution. However, it will prevent multiple accesses on different frequencies perform independently. The related operation on the reader side will be more complicated and the performance of random access using D2R FDMA may be degraded. The approach is not preferred.

In the case that D2R FDMA is used, another way to resolve the above-mentioned issue is to use frequency information along with AS ID (i.e., random ID) for R2D reception. Although additional bits to indicate the frequency where the random access is successfully completed are required, random ID can be used as AS ID for R2D reception with no impact on Msg1 contention resolution on each frequency. This can be considered a way to resolve AS ID collision for R2D reception when D2R FDMA is used.

Proposal 4. Discuss using D2R frequency information along with AS ID for R2D reception if D2R FDMA is used.

During the post email discussion ([POST129][036][AIoT] AS ID), there may be a consensus that the device only keeps one AS ID at a time, and the device releases the AS ID upon power off. From the latter one, it is understood that an AS ID is stored in volatile memory. For the release of AS ID, it is suggested to focus on the following 5 options for further discussion. Regarding option 4b (the device releases the AS ID after completion of the command procedure), it seems valid. But it was considered as an option described at a somewhat high level. So, how to determine completion of the command procedure needs to be discussed for option 4b. Other options such as option 1, option 3 and option 6 are considered to include ways to determine completion of the command procedure. For option 1 and option 3, if AS ID is assigned once after the first successful random access, there may be no difference between option 1 and option 3.

- Option 1: The device releases the AS ID upon receiving Paging with same/new transaction id, i.e. same/different session/service

- Option 2: The device releases the AS ID upon timer expiry; The Timer could be configured by the reader, or predefined in the specification;

- Option 3: The device releases the AS ID upon new random ID is generated or receiving new assigned AS ID from the reader; Note: It will not lead new AS ID assignment option. FFS on whether the AS ID can be assigned at any time.

- Option 4b (Option 4b-1, Option 4b-2, Option 4b-3): The device releases the AS ID after completion of the command procedure

  - Option 4b-1: the device releases the AS ID upon receiving the ‘end’ indication from reader, or

  - Option 4b-2: upon completed the last D2R message transmission (a single command message)? Or

  - Option 4b-3: maximum number of command messages;

- Option 6: The device releases the AS ID upon receiving explicit release indication from the Reader

AS ID needs to be valid during at least a command procedure. In addition, based on the agreements about A-IoT paging discussion in R2-129 meeting, parallel service requests by the same reader are not supported, and the device is expected to only perform one procedure at a time. Therefore, AS ID validity or release of AS ID is much related to paging procedure and when the related data transmission for command procedure (or the service which triggers paging) is required to be completed. Regarding completion of data transmission for a command procedure, there may be 3 views: 1) within an access occasion or a set of access occasions, 2) within one paging round (e.g., any paging) within a service request (e.g., paging with same transaction ID).

For view 1) where the data transmission for command procedure is required to be completed within an access occasion, if random access is successfully completed, the random ID used for the random access is unique within the access occasion. Thus, the message transmissions related to the command procedure can be scheduled using the AS ID (i.e., random ID) within the access occasion without risk of AS ID collision. By the way, many companies are interested in handling AS ID collision. Based on that, it is understood that view 2) and 3) are considered by many companies.

Among the above 5 options, option 1 (i.e., the device releases the AS ID upon receiving Paging with same/new transaction id.) is understood that the AS ID is released after completion of the command procedure based on paging. Also, it includes the above-mentioned view 2) and view 3). The release of AS ID can be discussed further based on option 1 by adding view 1) to option1.

Proposal 5. Discuss the release of AS ID based on option 1 (i.e., the device releases the AS ID upon receiving paging with same/new transaction id.) and the device releases the AS ID upon receiving indication of Msg1 resource.

Segmentation and data (re)transmission for failure handling

During R2-129bis meeting, it is agreed that R2D segmentation is not supported for R19 A-IoT. Therefore, segmentation for A-IoT data transmission is supported for D2R transmission only. Also, it is agreed that segment retransmission is supported, and the reader explicitly indicates an offset in the MAC layer for segment retransmission.

In addition, there has been discussion about whether unsegmented packet can also be retransmitted or not. However, the discussion has not been concluded yet. We would like to discuss it further. More specifically, we focus on whether retransmission of unsegmented D2R response message is supported or not.

Retransmission works only if the unsegmented D2R response message to be transmitted is prepared by the device. However, the reader doesn’t know whether the device prepared the message when the reader detected transmission failure of unsegmented D2R response message. The reader can’t decide whether retransmission can be used for the transmission failure handling or not. The transmission failure is detected by a reader if the reader doesn’t receive the corresponding D2R response message after sending an R2D command message. There may be two reasons for the transmission failure. One is D2R transmission failure, and the other is R2D transmission failure. In A-IoT considering DT and DO-DTT traffic types, the device does not autonomously send any data including ACK/NACK feedback indication for the R2D transmission without triggering by the reader. Therefore, the reader can’t be aware of the R2D transmission failure. In other words, the reader can’t distinguish the reasons for transmission failure of unsegmented D2R response message between D2R transmission failure and R2D transmission failure. Even worse, in case of R2D transmission failure, the device has not received the R2D command message. So, it can’t prepare the unsegmented D2R response message. If the device receives the retransmission request, the device can’t perform retransmission properly because it has no message to be transmitted.

Observation 1. Retransmission can’t be used if transmission failure of unsegmented D2R response message is caused by the corresponding R2D command message transmission failure. The reader can’t distinguish the reasons for the transmission failure between R2D command message transmission failure and D2R response message transmission failure.

Based on the above analysis, in case of transmission failure of unsegmented D2R response message, it may happen that the device receives retransmission request even when it doesn’t have the unsegmented D2R response message if retransmission is used to handle the failure. To avoid the situation, retransmission should not be used for unsegmented D2R packet. Instead, the reader can repeat the R2D command message transmission to trigger the device to send the D2R response message.

Proposal 6. Retransmission is not used for unsegmented D2R packet.

Proposal 7. Confirm that, if the reader does not receive the expected D2R response message, the reader is allowed to re-send the R2D command message to trigger the device to send the expected D2R response message.

The transmission failure handling of the first segment has similar issues with transmission failure handling of unsegmented D2R packet. When the transmission failure of the first segment is detected by the reader, the reader doesn’t know whether the device received the corresponding R2D command message or not. According to the agreement of supporting segment retransmission, when the reader triggers segment retransmission for the first segment, the device has no D2R message to be transmitted if it didn’t receive the R2D command message. Then, the retransmission of the first segment can’t be performed properly. In this case, if some enhancements can be considered, the device may indicate that the R2D command is not received using the D2R scheduling indicated by the R2D transmission, and the reader sends the R2D command again if it receives the indication.

By the way, it is not clear that unsegmented transmission and segmented transmission can be identified at the beginning of the command procedure. If the reader can’t distinguish between unsegmented transmission and segmented transmission at the beginning of the command procedure, the transmission failure handling of the first segment and transmission failure handling of unsegmented D2R packet need to be same.

Based on the above-mentioned analysis, we would like to suggest a discussion about whether the first segment can be retransmitted or not. Of course, for the second segment and the following segments (if any), it seems clear that the device already received the R2D command message, and it has the D2R message to be transmitted. Therefore, the discussion is needed only for the first segment transmission.

Proposal 8. Discuss whether the first segment can be retransmitted or not.

3.	Conclusion

This document discusses AS ID, segmentation and data (re)transmission for failure handling.

Proposal 1. The size of AS ID is 16 bits.

Proposal 2. For CBRA, AS ID is assigned via Msg2.

Proposal 3. For CFRA, option 2 (i.e., the device includes a random ID in Msg1) is used for AS ID assignment. The AS ID can be used from Msg2 (including R2D command message) transmission for R2D reception.

Proposal 4. Discuss using D2R frequency information along with AS ID for R2D reception if D2R FDMA is used.

Proposal 5. Discuss the release of AS ID based on option 1 (i.e., the device releases the AS ID upon receiving paging with same/new transaction id.) and the device releases the AS ID upon receiving indication of Msg1 resource.

Observation 1. Retransmission can’t be used if transmission failure of unsegmented D2R response message is caused by the corresponding R2D command message transmission failure. The reader can’t distinguish the reasons for the transmission failure between R2D command message transmission failure and D2R response message transmission failure.

Proposal 6. Retransmission is not used for unsegmented D2R packet.

Proposal 7. Confirm that, if the reader does not receive the expected D2R response message, the reader is allowed to re-send the R2D command message to trigger the device to send the expected D2R response message.

Proposal 8. Discuss whether the first segment can be retransmitted or not.