3GPP TSG-RAN WG2 Meeting #129bis	R2-2502674

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.7.6

Source:	Ericsson

Title:	Views on XR Rate Control

Document for:	Discussion, Decision

# 1	Introduction

On the issue of per QoS flow vs per DRB level granularity for the MAC CE to indicate the recommended bit rate i.e., QoS flow vs RRC-based solution, the outcome hinges on further RAN3 evaluation on “whether gNB-DU can support per QoS flow data bit rate control when TU is available” as captured in [1]. So, our suggestion is to wait for RAN3’s LS response before down-selecting between the two solutions. Having said that, we believe the RRC-based solution is the only feasible solution.

In the last meeting, it was agreed that a new table will be specified to support the recommended bit rate indications for XR traffic based on range and granularity information provided in the LS from SA4 [2]. But companies had different views on the design of this new table in terms of the distribution of the bit rates, granularity and number of indices to cover the required range. Hence, these details were left for FFS.

In this contribution, we discuss the design of the new table considering the requirements of a gNB to provide a bit rate recommendation for the UE to perform UL rate control and the corresponding UE capabilities required to support this feature.

# 2	Discussion

## 2.1 	Per QoS Flow vs per DRB MAC CE

The down-selection between the two solutions for rate indication on a per QoS flow level granularity i.e., between the QFI in MAC CE and RRC-based with per DRB in MAC CE depends on the outcome of the RAN3 discussion on this topic. As mentioned in the LS reply “RAN3 will further discuss whether gNB-DU can support per QoS flow data bit rate control when TU is available”.

Given that TUs were allocated to RAN3 in the last plenary, it would be wise to wait for RAN3’s evaluation before making the decision in RAN2.

- Due to cross WG impacts, RAN2 needs to wait for the RAN3’s LS response before down selecting between the two solutions.

Nevertheless, between the two options, we have emphasized many times that the gNB-DU cannot support per QoS flow data bit rate control. As the gNB-DU is unable to distinguish between the traffic from different QoS flows mapped to the same DRB. It is only possible to perform a per QoS flow handling if there is a one-to-one mapping of the QoS flow to DRB.

- gNB-DU is unable to distinguish between different QoS flow traffic when multiple QoS flows are mapped to the same DRB. To effect per QoS flow handling, there should be a one-to-one mapping of the QoS flow to DRB.

Considering O2, the solution of including the QoS flow in the MAC CE is not feasible. For the RRC-based solution with a per-DRB MAC CE, there is minimal impact over the F1 interface and is in line with the ability of a gNB-DU to rate control at a per-DRB level. Thus, we propose that the RRC-based solution is used to perform rate indication from gNB to the UE.

RRC-based, while MAC CE is per DRB is used to perform XR rate control indication from gNB to the UE on a per QoS flow level.

The following describes the details of the RRC-based solution:

How does the RRC-based solution work?

Given that the network is aware of which QoS flow (or QFI) needs to be adapted, the network can (pre)configure at the UE (via DL RRC signaling), the set of QoS flows that are candidates for being rate adapted. Then, upon reception of the per DRB level MAC CE, the UE perform rate adaption on the candidate those QoS flows previously configured via RRC.

For example, the gNB configures QoS flow A to be a candidate for rate adaptation, but QoS flow B is not. If/when the UE later gets the MAC CE, the UE rate adapts QoS flow A.

Is there a restriction that only one QoS flow of the DRB (with multiple QoS flows mapped) can be rate adapted?

No, there is no such restriction. It is up to network implementation how many QoS flows are indicated in the DL RRC message.

How does the UE perform rate adaption across the configured QoS flows using the RBR MAC CE?

One option is that we reuse the legacy procedure and leave this to UE implementation. The other option is that in addition to the QoS flow IDs (QFIs) as indicated in the DL RRC message, the network can also include the ratio of the bit rates that need to be rate adapted across the configured QoS flows.

Thus, we propose the following for an RRC-based solution:

gNB can configure one or more QoS flows (mapped to a DRB) to be rate adapted.

Reuse the recommended bit rate MAC CE to indicate the desired bit rate.

For rate adaptation of more than one QoS flow within a DRB, consider two options:

  - Up to UE implementation how to perform rate adaption between the configured QoS flows.

  - gNB indicates the ratio of rate adaption between the QoS flows as part of the configuration in P2.

## 2.2	Granularity and Range for XR Rate Control

It was agreed that a new table is specified for XR rate control covering the range of bit rate values as captured in the LS reply from SA4 [2], the range varying between 100 kbps and 40 Mbps. In terms of the design of this new table, there are three aspects to consider:

- Distribution of the bit rate values for e.g., exponential or linear

- Granularity of the bit rates

- Number of indices to cover the range of bit rates

For (1), some companies propose to have an exponential distribution of the bit rate values whilst others prefer to have a linear distribution.

In our understanding, the idea of the gNB providing a bit rate recommendation is that the UE can perform UL rate control for the application when congestion occurs at the gNB. This enables the application to be throttled, and a lower recommended bit rate value can be provided relative to the current operational bit rate. In this regard, the exponential distribution is effective in achieving a steep bit rate reduction. However, the understanding among companies was also that this new table needs to work in both directions symmetrically i.e., increase and decrease. In which case, using an exponential distribution to increase the bit rate may lead to a sharp rise, potentially causing renewed congestion or worsening the problem.

- As the new table needs to work in both directions symmetrically (i.e., increase and decrease), the exponential distribution is not ideal as the sharp rise in the bit rate can potentially cause renewed congestion or exacerbates the situation.

Thus, a design principle for the new table should be such that it would allow for a sharp decrease to relieve congestion at the gNB and a gradual increase for better buffer management, and only a linear distribution can accommodate this behaviour.

- A design principle for the new table should allow for a sharp decrease to relieve congestion and gradual increase for flexible buffer management, and only a linear distribution can achieve this behaviour.

Next, we examine (2) and (3) collectively, as one design decision impacts the other. In general, the overhead of a MAC CE (in terms of #indices) should be minimized while still encompassing the necessary range.

- The format of such a rate control MAC CE should be such that it keeps the overhead (in terms of #indices) low whilst still covering the required range.

Considering O2, the LS reply from SA4 mentions that the granularity can be as small as 1% for some algorithms yet also coarser for other algorithms i.e., 10%. From SA4’s perspective, this is a general guidance on the bit rates and granularity of XR services.

However, considering UL rate control, it becomes apparent that a 1% granularity does not meet the requirements for a sharp decrease. Moreover, the number of indices needed to cover the range between 100 kbps and 40 Mbps would be excessively large, resulting in a high signaling overhead. This is unnecessary, given that the reply LS mentions that a coarser granularity e.g., 10% is also sufficient.

- The 1% granularity does not meet the requirements of a sharp decrease, and the number of indices required to cover the required range of bit rates is prohibitively large and unnecessary given that the SA4 LS mentions a coarser granularity is sufficient.

Therefore, a minimum granularity of 10% is necessary to perceive meaningful effects of throttling due to rate adaptation. This degree of granularity provides adequate control over bit rate adjustments in both directions i.e., facilitating a sharp decrease and a gradual increase. In terms of signaling overhead, 64 indices are sufficient to cover the required range, enabling the reuse of the current 6-bit bit rate index value.

- 10% granularity provides adequate control over bit rate adjustments in both directions i.e., facilitating a sharp decrease and a gradual increase. Moreover, the range of 100 kbps to 40 Mbps can be covered with 64 indices keeping the overhead to a minimum.

Based on the preceding discussion, we propose that the new table be designed with a linear distribution and 10% granularity, covering the range of 100 kbps to 40 Mbps. This approach allows for the reuse of the 6-bit bit rate index design for the MAC CE.

New table for XR rate control is to be designed to cover the range between 100 kbps and 40 Mbps with a:

  - Linear distribution

  - 10% granularity

  - 6-bit rate index design for MAC CE.

## 2.3 	UE Capability

As explained above, the specific solution on the QoS-flow level indication depends on the outcome of the RAN3 discussion. But it is straightforward that the ability to receive this information and perform rate adaption in the UL should be an optional UE capability with signalling. The following were the list of UE capabilities for the recommended bit rate MAC CE feature:

| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 [8]. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| recommendedBitRateMultiplier-r16 Indicates whether the UE supports the bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [8], clause 6.1.3.20. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321 [8]. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |

As we can see the different aspects (query and multiplier) of the feature are covered by different UE capabilities conditional on the recommendedBitRate capability. Thus, the same should be applicable to the XR rate control feature that if the rate query MAC CE is confirmed, it should be tied to a new UE capability conditional on the XR rate control UE capability.

Define a new (optional) per-UE capability with signaling for the XR rate control MAC CE.

If the working assumption is confirmed, define a new (optional) per-UE capability with signaling for the XR rate query MAC CE conditional on the UE capability for the XR rate control MAC CE.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	Due to cross WG impacts, RAN2 needs to wait for the RAN3’s LS response before down selecting between the two solutions.

Observation 2	gNB-DU is unable to distinguish between different QoS flow traffic when multiple QoS flows are mapped to the same DRB. To effect per QoS flow handling, there should be a one-to-one mapping of the QoS flow to DRB.

Observation 3	As the new table needs to work in both directions symmetrically (i.e., increase and decrease), the exponential distribution is not ideal as the sharp rise in the bit rate can potentially cause renewed congestion or exacerbates the situation.

Observation 4	A design principle for the new table should allow for a sharp decrease to relieve congestion and gradual increase for flexible buffer management, and only a linear distribution can achieve this behaviour.

Observation 5	The format of such a rate control MAC CE should be such that it keeps the overhead (in terms of #indices) low whilst still covering the required range.

Observation 6	The 1% granularity does not meet the requirements of a sharp decrease, and the number of indices required to cover the required range of bit rates is prohibitively large and unnecessary given that the SA4 LS mentions a coarser granularity is sufficient.

Observation 7	10% granularity provides adequate control over bit rate adjustments in both directions i.e., facilitating a sharp decrease and a gradual increase. Moreover, the range of 100 kbps to 40 Mbps can be covered with 64 indices keeping the overhead to a minimum.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RRC-based, while MAC CE is per DRB is used to perform XR rate control indication from gNB to the UE on a per QoS flow level.

Proposal 2	gNB can configure one or more QoS flows (mapped to a DRB) to be rate adapted.

Proposal 3	Reuse the recommended bit rate MAC CE to indicate the desired bit rate.

Proposal 4	For rate adaptation of more than one QoS flow within a DRB, consider two options:

a.	 Up to UE implementation how to perform rate adaption between the configured QoS flows.

b.	 gNB indicates the ratio of rate adaption between the QoS flows as part of the configuration in P2.

Proposal 5	New table for XR rate control is to be designed to cover the range between 100 kbps and 40 Mbps with a:

a. Linear distribution

b. 10% granularity

c. 6-bit rate index design for MAC CE.

Proposal 6	Define a new (optional) per-UE capability with signaling for the XR rate control MAC CE.

Proposal 7	If the working assumption is confirmed, define a new (optional) per-UE capability with signaling for the XR rate query MAC CE conditional on the UE capability for the XR rate control MAC CE.

# 4 	References

- R3-250805, Reply LS on XR UL Rate Control, 3GPP RAN WG3 #127, Athens, Greece.

- S4-242182, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, 3GPP SA WG4#120, November 18-22, 2024, Orlando, USA.

# 5	Annex

The following are the agreements from RAN2#128 meeting on the XR rate control objective:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

The following agreement were made in RAN2#129 on the XR rate control objective:

Agreements on XR rate control - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). Companies to check with their SA4 colleagues whether there are any issues with this