3GPP TSG-RAN WG2 #129bis	R2-2502682

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.0.2.20

Source:	Ericsson

Title:	In-band operation for NB-IoT

Document for:	Discussion, Decision

# 1	Introduction

RAN2 has received an LS [1] from RAN4 about the introduction of inband operation for NB-IoT NTN and the potential RAN2 impact.

1. Overall Description:

RAN4 introduced the inband operation for NTN IoT in NR NTN in TEI18. RAN4 has assumed there is no RAN2 impact. RAN4 has made below agreements

- In Rel-18, UE is only required to support the same operation mode for anchor and non-anchor carriers

- For in-band operation, RRC signalling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests.

- in Rel-18 for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB.

RAN4 would like to respectfully ask RAN2 to confirm above assumptions aligning with RAN2 specification.

2. Actions:

To RAN WG1/WG2 group.

ACTION: RAN4 would respectfully ask RAN2 to provide answer on the above.

# 2	Discussion

## 2.1	Background

Multi-carrier operation allows a NB-IoT to be configured with a specific anchor carrier where the physical channels and signals for synchronization, initial cell selection, and broadcast information are transmitted. In addition, it can be configured with one or more than one non-anchor carriers for SC-PTM reception, paging reception, and random access in RRC_IDLE and unicast transmission in RRC_CONNECTED. The UE switches and operates in one NB-IoT carrier at a time.

## 2.1	First assumption

RAN4 has introduced support for the inband operation mode in Release 18, alongside the standalone mode, which remains the only other supported mode in NTN. More specifically, this means that in Rel-17 there were no RAN4 requirements for inband operation and this is now added in Rel-18. In their LS, RAN4's initial assumption specifies that “the UE is only required to support the same operation mode for anchor and non-anchor carriers.” We interpret this assumption as indicating that the mixed operation mode feature, introduced in Release 15, does not apply to NTN. Mixed operation mode allows NB-IoT UEs to operate with an anchor carrier in standalone mode while non-anchor carriers are in inband or guard-band mode, and vice versa, depending on network configurations. Although mixed operation mode is an optional capability, its exclusion from NTN does not impact RAN2. Nonetheless, we recommend updating Table 5.5a-1 in Stage 2 to clearly indicate which deployment combinations are supported in NTN.

- RAN4's assumption is that UEs only need to support the same operation mode for anchor and non-anchor carriers which in principle indicates that mixed operation mode is not applicable to NTN in Release 18. This does not have impact in RAN2 specifications.

Update stage 2 and add a note to state which combinations are allowed in NB-IoT NTN. Take the TP as a baseline.

## 2.2	Second assumption

RAN4's second assumption specifies that “for in-band operation, RRC signaling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests.” Conformance tests are designed to ensure that devices meet the technical requirements specified by the standard and are defined in RAN5 specifications (e.g., TS 36.521-1), along with the (RRC) parameters necessary for each case. Therefore, we do not see any potential RAN2 impact. RAN4 has a section about conformance tests.

- RAN4's assumption that RRC should indicate guardband-r13 in operationModeInfo IE in MIB-NB does not have an impact in RAN2 specifications.

## 2.3	Third assumption

RAN4's third assumption specifies that “in Rel-18 for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB.” As a general principle, we share the same understanding that when a NB-IoT UE does not support a certain operation mode, it shall not be (physically) able to access to a cell deployed with that specific operation mode. This is because the operation mode is tightly related to timing and synchronization configurations described in RAN1 specifications (TS 36.211 and TS 36.213).

- A NB-IoT UE should not be able to access a cell with an operation mode it does not support.

Provided the type of deployments already supported in NTN, the network is expected to signal operationModeInfo in MIB-NB with either the value standalone (already supported) or guardband (recently introduced by RAN4). Therefore, we do not expect that the network broadcasts inband-SamePCI or inband-DifferentPCI for NTN in Release 18. Nonetheless, for the sake of clarity, we would welcome if a small clarification is included in the field description of operationModeInfo to facilitate implementation. A baseline text proposal is provided in the next section.

- The network is not expected to broadcast inband-SamePCI or inband-DifferentPCI for NTN in Release 18.

Finally, we understand that RAN4’s last assumption concerns future compatibility. Considering the case that inband-SamePCI or inband-DifferentPCI need to be adopted in the future for NTN, RAN2 should further discuss what would be the expected network and UE behaviour.

RAN2 to discuss whether signalling enhancements are needed to prevent UEs from access a NB-IoT NTN cell that sets operationModeInfo to inband-SamePCI or inband-DifferentPCI.

# 3	Text Proposals

## 3.1	TS 36.300

Table 5.5a-1: Anchor and non-anchor carrier deployment combinations in FDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Guard-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Standalone | Valid (Note 2, Note 3) | Valid (Note 2, Note 3) | Valid (Note 2) |

NOTE 1:	Both carriers associated with the same LTE cell.

NOTE 2:	Total frequency span to not exceed 20MHz and both anchor and non-anchor carriers synchronised.

NOTE 3:	Not applicable to SC-PTM reception.

NOTE 4:	Mixed operation mode is not supported in NTN in this release of the specification.

## 3.2	TS 36.331

#### –	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH in FDD.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-NB

-- ASN1START

MasterInformationBlock-NB ::=	SEQUENCE {

systemFrameNumber-MSB-r13		BIT STRING (SIZE (4)),

hyperSFN-LSB-r13				BIT STRING (SIZE (2)),

schedulingInfoSIB1-r13			INTEGER (0..15),

systemInfoValueTag-r13			INTEGER (0..31),

ab-Enabled-r13					BOOLEAN,

operationModeInfo-r13			CHOICE {

inband-SamePCI-r13				Inband-SamePCI-NB-r13,

inband-DifferentPCI-r13			Inband-DifferentPCI-NB-r13,

guardband-r13					Guardband-NB-r13,

standalone-r13					Standalone-NB-r13

},

additionalTransmissionSIB1-r15	BOOLEAN,

ab-Enabled-5GC-r16				BOOLEAN,

partEARFCN-r17					CHOICE {

spare							BIT STRING (SIZE (2)),

earfcn-LSB						BIT STRING (SIZE (2))

},

spare							BIT STRING (SIZE (6))

}

Guardband-NB-r13 ::=			SEQUENCE {

rasterOffset-r13				ChannelRasterOffset-NB-r13,

spare							BIT STRING (SIZE (3))

}

Inband-SamePCI-NB-r13 ::=		SEQUENCE {

eutra-CRS-SequenceInfo-r13		INTEGER (0..31)

}

Inband-DifferentPCI-NB-r13 ::=	SEQUENCE {

eutra-NumCRS-Ports-r13			ENUMERATED {same, four},

rasterOffset-r13				ChannelRasterOffset-NB-r13,

spare							BIT STRING (SIZE (2))

}

Standalone-NB-r13 ::=			SEQUENCE {

spare							BIT STRING (SIZE (5))

}

-- ASN1STOP

MasterInformationBlock-NB field descriptions

ab-Enabled Value TRUE indicates that access barring is enabled for UEs connected to EPC.

ab-Enabled-5GC Value TRUE indicates that access barring is enabled for UEs connected to 5GC.

additionalTransmissionSIB1 Value TRUE indicates that additional SIB1-NB transmissions are present. See TS 36.211 [21] and TS 36.213 [23]. E-UTRAN only configures additionalTransmissionSIB1 to TRUE if schedulingInfoSIB1 indicates that the number of NPDSCH repetitions is 16, see TS 36.213 [23], Table 16.4.1.3-3.

earfcn-LSB Indicates the 2 least significant bits of the EARFCN for NTN bands where 100 kHz raster is used, see TS 36.102 [113].

eutra-CRS-SequenceInfo Information of the carrier containing NPSS/NSSS/NPBCH. Each value is associated with an E-UTRA PRB index as an offset from the middle of the LTE system sorted out by channel raster offset. See TS 36.211[21] and TS 36.213 [23].

eutra-NumCRS-Ports Number of E-UTRA CRS antenna ports, either the same number of ports as NRS or 4 antenna ports. See TS 36.211 [21], TS 36.212 [22], and TS 36.213 [23].

hyperSFN-LSB Indicates the 2 least significant bits of hyper SFN. The remaining bits are present in SystemInformationBlockType1-NB.

operationModeInfo Deployment scenario (in-band/guard-band/standalone) and related information. See TS 36.211 [21] and TS 36.213 [23]. Inband-SamePCI indicates an in-band deployment and that the NB-IoT and LTE cell share the same physical cell id and have the same number of NRS and CRS ports. Inband-DifferentPCI indicates an in-band deployment and that the NB-IoT and LTE cell have different physical cell id. guardband indicates a guard-band deployment. standalone indicates a standalone deployment. In NTN, EUTRAN does not set this field to Inband-SamePCI or Inband-DifferentPCI in this version of the specification.

schedulingInfoSIB1 This field contains an index to a table specified in TS 36.213 [23], Table 16.4.1.3-3, that defines SystemInformationBlockType1-NB scheduling information.

systemFrameNumber-MSB Defines the 4 most significant bits of the SFN. As indicated in TS 36.211 [21], the 6 least significant bits of the SFN are acquired implicitly by decoding the NPBCH.

systemInfoValueTag Common for all SIBs other than MIB-NB, SIB14-NB, SIB16-NB, SIB31-NB and SIB33-NB.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN4's assumption is that UEs only need to support the same operation mode for anchor and non-anchor carriers which in principle indicates that mixed operation mode is not applicable to NTN in Release 18. This does not have impact in RAN2 specifications.

Observation 2	RAN4's assumption that RRC should indicate guardband-r13 in operationModeInfo IE in MIB-NB does not have an impact in RAN2 specifications.

Observation 3	A NB-IoT UE should not be able to access a cell with an operation mode it does not support.

Observation 4	The network is not expected to broadcast inband-SamePCI or inband-DifferentPCI for NTN in Release 18.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Update stage 2 and add a note to state which combinations are allowed in NB-IoT NTN. Take the TP as a baseline.

Proposal 2	RAN2 to discuss whether signalling enhancements are needed to prevent UEs from access a NB-IoT NTN cell that sets operationModeInfo to inband-SamePCI or inband-DifferentPCI.

# 4	References

[1]	R4-2503035, LS on NR NB-IoT in-band operation, Ericsson, RAN4#114, February 2025.