3GPP TSG-RAN WG2 Meeting #129bis	R2-2502692

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item	: 8.13.3

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-242349 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we mainly investigate the control procedure for approach 1 and approach 2 based on the long email discussion (i.e., [POST129][402][Relay]Control plane (Approach 2)). Also, we propose SIB and paging delivery in multi-hop relay circumstances.

2.	Discussion

2.1 Considering on approach 2

Based on the long email discussion (i.e., [POST129][402][Relay]Control plane (Approach 2)), we studied the specified approach 2 control procedure and spec impacts of it. RAN2 already agreed approach 1 is the baseline for the discussion. So, even if the approach 2 is allowed, we think it needs to be integrated into the approach 1. It should be prohibited that two different control schemes are implemented independently for the same purpose, i.e., multi-hop relay operation. Meanwhile, if we have to select one control scheme between them, the selected scheme has to have clear advantages compared to approach 1. Because approach 1 is easily extendible based on the legacy Rel-17 U2N, we expect the spec impact is not much. We should consider the remaining time of the Rel-19 multi-hop U2N relay completion.

The following are our concerns for approach 2 control itself and describe how it can be integrated with approach 1.

- Initial RRC setup procedure and SRAP header structure:

In approach 2, new SRAP header structures are proposed to deliver the initial UL/DL RRC messages. This enables making the linkage between the local ID and the PC5 link on the IDLE/INACTIVE intermediate relay UE itself. Considering the integration into approach 1, we think it can be a little bit complicated issue.

For example, in approach 1, Remote UE sends the first UL RRC message via specified SL_RLC0. If the parent intermediate Relay UE is in RRC_IDLE/INACTIVE, it becomes in RRC_CONNECTED. The intermediate Relay UE sends a message to request the local ID of the Remote UE to the gNB. After receiving the local ID of the Remote UE, the intermediate Relay UE sends the initial UL RRC message of the Remote UE to the gNB with the received local ID.

However, in approach 2, Remote UE will send its first UL RRC message with the proposed SRAP header regardless of the RRC state of the parent relay UE. The Remote UE doesn’t know the RRC state of the parent Relay UEs. In this case, what’s the behavior of the RRC_CONNECTED intermediate Relay UE?

Maybe the RRC_CONNECTED intermediate Relay UE may request the local ID of the Remote UE to the gNB by using the L2 ID of the received SRAP header from the Remote UE. After receiving the local ID of the Remote UE, the intermediate Relay UE sends the initial RRC message of the Remote UE toward gNB with the local ID. It is similar behavior of the intermediate Relay UE in approach 1. However, how does it work if the parent relay UE of the CONNECTED intermediate Relay UE is RRC_IDLE/INACTIVE?  It is delivered via SL-SRB0?  Which SRAP header structure should be used? Maybe a solution can be proposed to solve these kinds of issues.

However, we are not sure what are the benefits when we do that. We think the benefit should be clearer to find issues further and put our effort into approach 2.

- RRC state transition of the intermediate Relay UE:

In approach 2, when RRC_IDLE/INACTIVE intermediate Relay UE becomes in RRC_CONNECTED, gNB may not know which intermediate Relay UE belongs to which Remote UE. The gNB has to know which intermediate Relay UE belongs to which path toward which Remote UE because the gNB has to provide a proper configuration to the intermediate Relay UE to support multi-hop relay operation. One of the solutions can be this: if the intermediate Relay UE reports the L2 ID and local ID pairs storing itself that was used during the RRC_IDLE/INACTIVE state. Anyway, we need to discuss further about these relevant issues.

- How to release about the Remote UE related information when the intermediate Relay UE is in RRC_IDLE/INACTIVE:

If RLF or release happens among the DL link side of the intermediate Relay UE in RRC_IDLE/INACTIVE, the notification message may be delivered to the intermediate Relay UE. However, it’s not clear how the intermediate Relay UE in RRC_IDLE/INACTIVE releases the related information (e.g., the linkage of L2 ID and local ID, QoS profiles about the Remote UE, etc.). In the current notification message, the information of which path happens RLF isn't included. So, we may need to discuss how to handle this issue.

- QoS split:

According to the QoS split scheme in approach 2, the QoS split of some parts of the links is performed without measurement reports. In extreme cases, i.e., if all the intermediate Relay UE is in RRC_IDLE/INACTIVE, the QoS split has to be performed evenly. Even though gNB knows the link quality of only some parts, the gNB may find it hard to perform QoS split properly.

We think one of the reasons to apply L2 Relay operation, not L3 Relay operation, is that delicate control is possible based on the AS layer information. The part for delicate control can be the QoS management part. For the QoS management properly, the QoS split should be performed based on the measurement report. We think the proper QoS split is quite an important issue in RAN2. The current proposed approach 2 cannot do perform the proper QoS split because the IDLE/INACTIVE intermediate Relay UE cannot perform measurement reports.

- Service continuity:

Based on the description of approach 2, when Remote UE receives the HO command targeted to a single-hop path or multi-hop path, the Remote UE uses the approach 1 scheme to enforce the intermediate Relay UE to be in RRC_CONNECTED. This way will increase latency. We believe that one of the important advantages of approach 2 is reducing RRC setup latency. We think reducing latency during HO might be more important than reducing RRC setup latency. Another proposed way to support service continuity is that the gNB will select a path that all the intermediate Relay UEs along the path are in RRC_CONNECTED. However, if there is no target path such like that condition, the latency issue cannot be solved. We think approach 2 looks difficult to support service continuity.

We think RAN2 may handle/solve all the raised issues from other companies in the long run. But we are not still sure what’s the benefit of approach 2. As long as the benefits are not clear, we don’t find any motivation to provide our effort for the approach 2 completion. Based on the current agreed tree-like topology, the intermediate Relay UE cannot serve any other parent Relay UE except for the only parent Relay UE. Also, it cannot be in a CONNECTED state to the other cell, which is different from the last Relay UE’s serving cell. According to the RNA2 agreement, the serving cell of the intermediate Relay UE should always be the last Relay UE’s serving cell. In the current situation, we think the benefits of approach 2 are that it can reduce latency for the RRC setup procedure and the signaling overhead during RRC setup. However, we are not sure they are really important issues for multi-hop relay. While the Remote UE attaches a multi-hop relay, the Remote UE has to endure the service latency. And if all the intermediate Rely UE is already in RRC_CONNECTED, the effect of RRC setup latency reduction doesn’t exist.

Proposal 1: To support multi-hop relay operation, only one approach (e.g., approach 1 or approach 2) should be selected for description on the spec. Otherwise, we have to consider the integration of both approaches.

Proposal 2: Approach 2 may have additional issues that were not yet addressed during the long email discussion.

Proposal 3: To investigate approach 2 further, the benefit of approach 2 should be clarified.

Proposal 4a: Based on tree-like topology, the advantages of approach 2 that we can estimate are:

-	It can reduce the latency when RRC setup

-	It can reduce the signaling overhead during RRC setup

Proposal 4b: It needs to be clarified whether reducing RRC setup latency and signalling overhead during RRC setup is really important enough to outweigh the other shortcomings.

Proposal 5: If RAN2 selects one approach between approach 1 and approach 2, approach 1 has a clear benefit in the aspect of the spec impact.

Proposal 6: Considering the time budget to complete the Rel-19 multi-hop relay, i.e., we have only two more meetings for completing the multi-hop relay operation, we think it’s better to focus on the remaining issues of approach 1.

2.2 SIB/paging delivery

When the intermediate Relay UE receives SI request message from the Remote UE, the intermediate Relay UE stores which PC5 link has requested which SI. So, the L2 ID of the Remote UE doesn’t need to be included in the SI request message. An intermediate Relay UE which does not have a direct PC5 link with the Remote UE does not need to know the L2 ID of the Remote UE. It's enough the intermediate relay UE knows about which local ID is associated with which PC5 link.

Proposal 7: There is no need to include “remote UE ID” in the SI request message. It's enough that each intermediate Relay UE stores the information of which PC5 link has requested which SI message.

When an intermediate Relay UE receives the SI request message from the Remote UE, the intermediate Relay UE may already have the valid SI.  In this case, the intermediate Relay UE delivers the stored SI to the child intermediate Relay UE/Remote UE without forwarding the SI request message to its parent Relay UE. The intermediate Relay UE sends it via the PC5 link that had requested the SI.

Proposal 8: Intermediate Relay UE sends the requested SI to the child intermediate Relay UE/Remote UE via the PC5 link that had requested the SI.

Proposal 9: If the intermediate Relay UE already has valid SI information, the intermediate Relay UE can deliver it to the Remote UE without forwarding the message for requesting SI to its parent Relay UE.

In some cases, the intermediate Relay UE in RRC_IDLE/INACTIVE can be located within the same serving cell coverage of the last Relay UE. In this case, the intermediate Relay UE in RRC_IDLE/INACTIVE can receive the SIBs directly from the air. The SIBs can be for intermediate Relay UE itself or requested SIB from the child intermediate Relay UE/Remote UE. However, we think it is the implementation of intermediate Relay UE whether to receive the SIBs from the last Relay UE or Uu directly. There is no need to specify depending on the movement of the intermediate Relay UE from in-coverage to out-of-coverage or vice versa. An intermediate Relay UE in RRC_IDLE/INACTIVE delivers the SI request message to the last Relay UE. However, it's an implementation of the intermediate Relay UE whether to receive the SIB from the last Relay UE or the Uu directly.

In our understanding, dedicated SIB is prioritized over the broadcasted SIB. The intermediate Relay UE doesn’t know whether the last Relay UE is in RRC_IDLE/INACTIVE or RR_CONNECTED. However, considering the last Relay UE may be in RRC_CONNECTED, we think the intermediate Relay UE should receive the SIBs from the last Relay UE. We think the last Relay UE always is in charge of SIB delivery. However, if the intermediate Relay UE doesn’t mind receiving broadcasted SIBs from the Uu directly, it will be an intermediate Relay UE’s implementation.

Proposal 10: An intermediate Relay UE in RRC_IDLE/INACTIVE sends the SI request message to the last Relay UE. However, it's an implementation of the intermediate Relay UE whether to receive the SIB from the last Relay UE or the Uu directly, if the intermediate Relay UE exists in the same cell coverage of the last Relay UE (it does not specify the movement from in-coverage to out-of-coverage or vice versa).

When intermediate Relay UE becomes RRC_CONNECTED, the intermediate Relay UE releases its paging-related information in the parent UE at least for itself. However, the intermediate Relay UE does not need to release the paging-related information of its child intermediate Relay UEs or Remote UEs. The last Relay UE is the most stable UE to monitor paging in terms of Uu RSRP strength. So, we think the last Relay UE has to be in charge of paging monitoring for the other intermediate Relay UE and Remote UE, if they are in RRC_IDLE/INACTIVE.

Proposal 11: Only the last Relay UE is in charge of monitoring paging for the intermediate Relay UE(s) and Remote UE(s).

Proposal 12: When intermediate Relay UE becomes RRC_CONNECTED, the intermediate Relay UE releases paging-related information of itself in the parent Relay UE.

Proposal 13: Even when the intermediate Relay UE is in RRC_CONNECTED, the intermediate Relay UE forwards the paging-related information received from its child intermediate Relay UE(s) or Remote UE(s) to its parent intermediate Relay UE.

If the intermediate Relay UE in RRC_IDLE/INACTIVE is in the coverage, the intermediate Relay UE can receive paging of itself via Uu directly and also monitor the paging of the child intermediate Relay UEs and Remote UEs. To monitor paging, there is no need for the intermediate Relay UE to be in the same cell coverage of the last Relay UE. The intermediate Relay UE also can receive a paging in the other area if it is in the coverage. It is related to the tracking area. So, we think it can be an implementation of the intermediate Relay UE whether the intermediate Relay UE receives a paging from the last Relay UE or monitor via Uu directly. This does not need to be described on the spec.

Proposal 14: It is an implementation of the intermediate Relay UE whether the intermediate Relay UE/Remote UE in RRC_IDLE/INACTIVE monitors its paging via Uu directly or receives from the last Relay UE. It does not need to be specified in the spec.

2.3 SRAP header structure

In the last meeting, one of the FFS points was whether the SRAP header has to include the L2 ID of the Remote UE. We think it is not needed at least approach 1. Because the first intermediate Relay UE in RRC_CONNECTED reports the L2 ID of the Remote UE to request the local ID of the Remote UE. If gNB receives the local ID request message for the Remote UE, the gNB will assign a local ID for the Remote UE. The first DL RRC message from the gNB to the Remote UE includes the local ID of the Remote UE. After this procedure, gNB and Remote UE can communicate with each other by using the local ID. So, if we use approach 1, there is no need to include the L2 ID of Remote UE in the SRAP header.

Proposal 15: There is no need to include the L2 ID of the Remote UE in the SRAP header at least approach 1.

3.	Conclusion

Proposal 1: To support multi-hop relay operation, only one approach (e.g., approach 1 or approach 2) should be selected for description on the spec. Otherwise, we have to consider the integration of both approaches.

Proposal 2: Approach 2 may have additional issues that were not yet addressed during the long email discussion.

Proposal 3: To investigate approach 2 further, the benefit of approach 2 should be clarified.

Proposal 4a: Based on tree-like topology, the advantages of approach 2 that we can estimate are:

-	It can reduce the latency when RRC setup

-	It can reduce the signaling overhead during RRC setup

Proposal 4b: It needs to be clarified whether reducing RRC setup latency and signalling overhead during RRC setup is really important enough to outweigh the other shortcomings.

Proposal 5: If RAN2 selects one approach between approach 1 and approach 2, approach 1 has a clear benefit in the aspect of the spec impact.

Proposal 6: Considering the time budget to complete the Rel-19 multi-hop relay, i.e., we have only two more meetings for completing the multi-hop relay operation, we think it’s better to focus on the remaining issues of approach 1.

Proposal 7: There is no need to include “remote UE ID” in the SI request message. It's enough that each intermediate Relay UE stores the information of which PC5 link has requested which SI message.

Proposal 8: Intermediate Relay UE sends the requested SI to the child intermediate Relay UE/Remote UE via the PC5 link that had requested the SI.

Proposal 9: If the intermediate Relay UE already has valid SI information, the intermediate Relay UE can deliver it to the Remote UE without forwarding the message for requesting SI to its parent Relay UE.

Proposal 10: An intermediate Relay UE in RRC_IDLE/INACTIVE sends the SI request message to the last Relay UE. However, it's an implementation of the intermediate Relay UE whether to receive the SIB from the last Relay UE or the Uu directly, if the intermediate Relay UE exists in the same cell coverage of the last Relay UE (it does not specify the movement from in-coverage to out-of-coverage or vice versa).

Proposal 11: Only the last Relay UE is in charge of monitoring paging for the intermediate Relay UE(s) and Remote UE(s).

Proposal 12: When intermediate Relay UE becomes RRC_CONNECTED, the intermediate Relay UE releases paging-related information of itself in the parent Relay UE.

Proposal 13: Even when the intermediate Relay UE is in RRC_CONNECTED, the intermediate Relay UE forwards the paging-related information received from its child intermediate Relay UE(s) or Remote UE(s) to its parent intermediate Relay UE.

Proposal 14: It is an implementation of the intermediate Relay UE/Remote UE whether the intermediate Relay UE/Remote UE in RRC_IDLE/INACTIVE monitors its paging via Uu directly or receives from the last Relay UE. It does not need to be specified in the spec.

Proposal 15: There is no need to include the L2 ID of the Remote UE in the SRAP header at least approach 1.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.