# 3GPP TSG-WG2 Meeting #129bis              	   R2-2502698

# Wuhan, China, 7th April 2025 - 11th April 2025

# Source: 			CMCC

# Title:  Discussion on IoT NTN Store and Forward

# Agenda item:	8.9.2

# Document for: 	Discussion

Introduction

In the last RAN2#129 meeting, the following agreements [1] were approved as below:

Agreements:

- RAN2 currently assumes that a S&F mode changes does not necessarily imply a MME change

- The already agreed time information on transition from S&F operation mode to normal mode is provided in SIB31 (can work on signalling optimizations in case the new time information and T-service would carry the same information)

- Use absolute UTC time to broadcast the (S&F-> normal mode) time information. When this information is broadcast and indicates a time in the future the UE can assume the NW is currently operating in S&F mode (until the indicated S&F-> normal mode transition time).

- A change to the (S&F-> normal mode) time indication shall not result in system information change notification. Legacy rules for acquiring SIB31 apply

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Discussion

Based on the discussions and conclusions in the previous meetings, we continue discussing some RAN2 related aspects in S&F operation.

Time information

In the previous RAN2#128 meeting, it was agreed that the time information from S&F to normal mode was introduced to delay some NAS procedures. Since the time information is introduced in RAN2 to let UE wait until normal mode coming to delay some NAS procedure, it’s better to let UE high layer be aware of the time information. Whether UE decides to send initial access attempt or delay some NAS procedure till normal mode coming is out of RAN2 scope. What we need to do is to let UE high layer know the remaining time of S&F mode defined in AS layer once received or updated.

Proposal 1: UE AS will forward the time information (from S&F to normal mode) to the upper layers when the time information is received or updated.

As for the opposite directional time information from normal mode to S&F mode, it is still FFS whether to introduce or not. In our opinion, we think supporting time indication from normal mode to S&F mode is useful both for S&F capable UE and UE not supporting S&F function. Table 1 below summarize some benefits of bi-directional time information:

Table 1 The benefits for using time information for R19 S&F capable UE and not capable UE

| Benefits | R19 UE supporting S&F mode | R19 UE not supporting S&F mode |
| --- | --- | --- |
| Normal to S&F | Adjust configurations/services in advance to reduce performance degradation | - UE in connected mode can decide when to do measurement and whether to make cell reselection - UE in idle mode can decide whether to send random access request if there is MO data to send. |

- If the current mode is in normal NTN mode and soon switch to S&F operation:

- For S&F capable UEs, UE can adjust configurations/services in advance (e.g. release services with high QoS requirement or high latency requirements). So that UE can try to prevent performance degradation.

- For R19 UE not supporting S&F mode, UE can know when the service cell will be unavailable, thus UE in connected mode can also do the measurement and try to reselect to other cells or connect to other available satellite if any; Secondly, if there is limited time left for UE in idle mode (e.g. the t-service is going to expire soon), then even UE has MO data to send, UE will still decide not to enter RRC connected mode since the cell is going to switch to S&F mode soon.

Based on above analysis, we think the mode switching from normal mode to S&F mode can also be considered.

Proposal 2: It is proposed to introduce the opposite time information from normal mode to S&F mode.

As for using absolute value or relative value for the opposite directional time information, similar to the agreed time information from S&F to normal mode,we think it’s better to use the absolute value which is quite simple..

Proposal 3: Absolute UTC time is used to broadcast the time information from normal to S&F mode.

With regards to introduce a new time information or reuse legacy T-service, since the legacy UE and Rel-19 UE not supporting S&F function only recognize T-service, it is reasonable for the cell to indicate t-service expired when cell mode switching from normal to S&F mode. Thus, we think T-service can be reused as the time information from normal mode to S&F mode.

Proposal 4: T-service can be reused as the time information from normal mode to S&F mode.

RRC release

In the previous RAN2#128 meeting, we discussed whether to introduce a new RRC release cause when the feederlink is unavailable, from our point of view, whether the network releases UE to idle mode or not is totally based on network implementation. Besides, we discussed about the opposite time information from normal to S&F mode above, if that time information is supported, even without S&F RRC release cause, UE can still know why the network release it based on the opposite time information. Thus, we think it is not necessary to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable.

Proposal 5: Introducing a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not necessary.

For the possible changes of time information from S&F to normal mode, there was an agreement that a change to the (S&F-> normal mode) time indication shall not result in system information change notification. As for the changes of S&F operation indication in SIB1, we think similar rules can be applied at least for S&F capable UE in RRC connected mode. UE can know the mode switching based on time information or T-service and get the updated SIB from dedicated signalling. Otherwise, if S&F operation indication changes result in system information change notification which may let network release UE to idle state to monitor paging message, NB-IoT UE in RRC connected mode has to go back to idle mode to receive SIB1 and then enter RRC connected mode again which will greatly increase the power consumption. Besides, this kind of operation also have a potential risk that UE can not go back to RRC connected mode after receiving system information change notification due to the heavy load of network. We should avoid this kind of corner case. Thus, we propose that the changes of S&F operation indication will not result in UE in RRC connected mode go back to idle to monitor paging which may include system information change notification. IoT NTN UE in RRC connected mode can get the updated SIB message through dedicated signaling.

Proposal 6: When mode switching from normal to S&F, UE in RRC connected mode can update changed S&F operation indication through dedicated signaling, rather than receiving paging message in idle mode.

Neighbor cell information

In the previous RAN2#128 meeting, we talked about adding S&F neighbor cell information in SIB5 but didn’t reach an agreement. For adding S&F neighborhoods to SIB5 intraFreqAllowedCellList and interFreqAllowedCellList, the neighbor cells can be from the same satellite or from different satellites, The S&F neighbor cell information from different satellites can be obtained from the satellite ID list sent by the CN in the initial access rejection message. However, at present, we are still waiting for SA2's reply on whether all satellites in the satellite ID list support S&F mode. If all satellites support S&F, then we can use this list to obtain the neighborhood information of different satellites, but if some satellites in the list do not support S&F, the list then cannot be used to obtain the neighborhood information of different satellites, and other methods such as ISL or core network allocation may need to be considered to get the neighbour satellite cells information. Therefore, it is recommended to discuss this issue after SA2 gives feedback on the satellite ID list.

Proposal 7: It is proposed to defer introducing S&F specific neighbor cell list in SIB5 till SA2 gives feedback on the satellite ID list issue.

Paging enhancement

When S&F capable UE work in the cell operating in S&F mode or out of satellite coverage, UE does not need to monitor paging message all the time. That means UE can relax the paging monitoring for some scenarios shown as below:

- When UE is not within the satellite coverage area:

  - The UE can determine when the next satellite carrying the DL data it needs (paging itself) will arrive based on the waiting timer in NAS rejection message. Therefore, when the UE is not within the coverage area of the satellite that means there is no satellite carrying downlink data to wake up the UE. Thus, the UE does not need to monitor the paging message.

- When UE is within the satellite coverage area and camps on a cell in S&F operation:

- The network has already transmitted stored DL data: as for the normal procedure, the satellite sends the stored downlink messages to the UE. If there is no data communication and the DRX timer runs out, the UE goes back to idle state. Therefore, during the time information of the S&F operation, the network will not send new downlink data to the UE since feederlink does not recover. Consequently, unless the UE actively needs to transmit UL data, it will not enter the connected state, and thus the UE does not need to monitor paging messages.

- The satellite never recover its feederlink: That means the current satellite does not have data transmission with ground EPC. Thus, the satellite only has stored UE uplink data and no new arrived downlink data. Therefore, even UE is within the coverage area, without recovered feederlink, UE does not need to monitor paging and only switch back to RRC connected state when has uplink data to send.

Proposal 8: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data

- The satellite never recover its feederlink

Conclusions

- Time information

Proposal 1: UE AS will forward the time information (from S&F to normal mode) to the upper layers when the time information is received or updated.

Proposal 2: It is proposed to introduce the opposite time information from normal mode to S&F mode.

Proposal 3: Absolute UTC time is used to broadcast the time information from normal to S&F mode.

Proposal 4: T-service can be reused as the time information from normal mode to S&F mode.

- RRC release

Proposal 5: Introducing a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not necessary.

Proposal 6: When mode switching from normal to S&F, UE in RRC connected mode can update changed S&F operation indication through dedicated signaling, rather than receiving paging message in idle mode.

to monitor paging message which may include system information change notification.

- Neighbor cell information

Proposal 7: It is proposed to defer introducing S&F specific neighbor cell list in SIB5 till SA2 gives feedback on the satellite ID list issue.

- Paging enhancement

Proposal 8: S&F capable UE can relax the paging monitoring in the following scenarios:

- UE is not within the satellite coverage area

- UE is in a cell operating in S&F mode:

- The network has already transmitted stored DL data

- The satellite never recover its feederlink

References

- 3GPP Chair_Notes, R2-129 NR-NTN-IoT-NTN (Sergio) EOM