3GPP TSG RAN WG2 Meeting #129bis	R2-2502702

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.9.4

Source:	CMCC

Title:	Support of PWS messages for NB-IoT

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of PWS for NB-IoT and some agreements were achieved as follows [1]:

Agreements:

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells

- Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information.

- Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB

- We don’t introduce a new SIB for carrying geographical information for emergency areas

In this contribution, we would like to provide some further considerations on the remaining issues for PWS for NB-IoT.

Discussion

PWS reception in RRC_CONNECTED

In RAN #107, a revised WID [2] is approved, and a note about PWS notification in RRC_CONNECTED is as following:

Note: PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported.

Based on the latest WID scope, RAN2 could confirm not to support PWS notification in paging, direct indication, and system information in RRC_CONNECTED in R19. However, RRC dedicated signaling solution without RAN1 specification impact is still not excluded, and relying entirely on the network implementation to release UE to the RRC_IDLE for receiving the PWS message would lead to a longer reception latency of emergency message especially for disaster information. Therefore RRC dedicated signaling may be considered as a backup to support emergency message reception well in connected.

Proposal 1: RAN2 confirms not to support PWS notification in paging, direct indication, and system information in RRC_CONNECTED in R19.

Proposal 2: Kindly suggest RAN2 to further discuss RRC dedicated signaling solution for PWS notification in RRC_CONNECTED.

SIB1-NB acquisition

In RAN2 #128 meeting, we have agreed to extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. For whether support SIB1-NB acquisition is FFS.

Per the existing LTE mechanism, the UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, however, if etws-Indication or cmas-Indication is included in the Paging message, it would trigger the SystemInformationBlockType1 re-acquisition immediately, i.e., without waiting until the next system information modification period boundary. Then UE read the schedulingInfoList to acquire the System Information Block carrying specific PWS message(SystemInformationBlockType10 and SystemInformationBlockType11 for ETWS, SystemInformationBlockType12 for CMAS) if present.

- 5.2.1.4 Indication of ETWS notification <<omitted>> NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled. <<omitted>> - 5.2.1.5 Indication of CMAS notification <<omitted>> NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the cmas-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType12. The UE may or may not receive a Paging message including the cmas-Indication and/or systemInfoModification when SystemInformationBlockType12 is no longer scheduled. <<omitted>>

- 5.3.2.3 Reception of the Paging message by the UE <<omitted>> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12; <<omitted>>

Some companies provide enhancements solutions that network broadcasts early PWS scheduling message even though there is no PWS notification considering the latency. However, such enhancements may introduce some additional signaling overhead. Therefore, we could just extend the existing RRC procedure to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

Proposal 3: We could just extend the existing mechanism to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

UE capability

In existing TS 36.306, the PWS feature is optional to support without UE radio access capability.

- 6.2 PWS features - 6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. - 6.2.2 CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5]. - 6.2.3 KPAS It is optional for UE to support KPAS reception as specified in TS 36.331 [5]. The Korean Public Alert System (KPAS) uses the same AS mechanisms as defined for CMAS. Therefore a KPAS-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE. - 6.2.4 EU-Alert It is optional for UE to support EU-Alert reception as specified in TS 36.331 [5]. The Europearn Union Warning System EU-Alert uses the same AS mechanisms as defined for CMAS. Therefore a EU-Alert-capable UE shall support all behaviour that is included in TS 36.331 [5] and TS 36.304 [14] for a CMAS-capable UE.

Then section 4 provides optional features applicable in NB-IoT without UE radio access capability parameters. However there is no PWS feature, that is the PWS feature in section 6.2 depicted above is not applicable in NB-IoT, as description in section 4.

The following optional features without UE radio access capability parameters specified in clause 6 are applicable in NB-IoT: - RRC Connection Re-establishment for the Control Plane CIoT EPS Optimization (clause 6.7.5) - System Information Block Type 16 (clause 6.8.1) - Enhanced random access power control (clause 6.8.3) - MT-EDT for Control Plane CIoT EPS Optimisation (clause 6.8.10) - MT-EDT for User Plane CIoT EPS Optimisation (clause 6.8.11) - EDT for Control Plane CIoT EPS Optimization (clause 6.8.4) - Enhanced PHR (clause 6.8.6) - Carrier specific NRSRP thresholds for NPRACH resource selection (clause 6.8.15) - Radio Link Failure Report for NB-IoT (clause 6.10.2) - SC-PTM in Idle mode (clause 6.16.1) - Multiple TB scheduling for SC-PTM in Idle mode for NB-IoT (clause 6.16.2) - Relaxed monitoring (clause 6.17.1) - DL channel quality reporting in Msg3 for the anchor carrier (clause 6.17.2) - Serving cell idle mode measurements reporting (clause 6.17.3) - NSSS-Based RRM measurements (clause 6.17.4) - NPBCH-Based RRM measurements (clause 6.17.5) - RRM measurements on non-anchor paging carriers (clause 6.17.6) - NRS presence on non-anchor paging carriers (clause 6.17.7) - DL channel quality reporting in Msg3 for non-anchor carrier (clause 6.17.8) - Assistance information for inter-RAT cell selection to/from NB-IoT (clause 6.17.9) - RRC Connection Re-establishment for the Control Plane CIoT 5GS Optimisation (clause 6.18.3) - NB-IoT/5GC (clause 6.18.4) - MO-EDT for Control Plane CIoT 5GS Optimisation (clause 6.18.5) - AS RAI (clause 6.18.6) - Cell reselection measurements triggering based on service time (clause 6.19.1) - Discontinuous coverage (clause 6.19.2). - Early RLF triggering based on service time (clause 6.19.3). - Neighbour cell measurements based on service start time of the neighbour cell (clause 6.19.4). - UE autonomous release based on service time (clause 6.19.5). - Cell reselection measurements triggering based on location for (quasi-)fixed cell (clause 6.19.6). - Cell reselection measurements triggering based on location for earth moving cell (clause 6.19.7). - GNSS measurements during inactive time (clause 6.19.8). - SystemInformationBlockType33(-NB) reception in a TN cell (clause 6.19.9). The optional features without UE radio access capability parameters specified in clause 6 are not applicable in NB-IoT, unless they are listed above.

Therefore, to support PWS in NB-IoT, a simple solution is to add support PWS in section 4 of TS 36.306, then the PWS feature in 6.2 could be apply to NB-IoT directly.

Proposal 4: Introduce PWS reception in section 4 of TS 36.306, then the PWS feature in 6.2 could be apply to NB-IoT directly.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on broadcast of PWS messages for NB-IoT and have the following proposals:

Proposal 1: RAN2 confirms not to support PWS notification in paging, direct indication, and system information in RRC_CONNECTED in R19.

Proposal 2: Kindly suggest RAN2 to further discuss RRC dedicated signaling solution for PWS notification in RRC_CONNECTED.

Proposal 3: We could just extend the existing mechanism to NB-IoT NTN for SIB1-NB acquisition, and no further enhancement is needed.

Proposal 4: Introduce PWS reception in section 4 of TS 36.306, then the PWS feature in 6.2 could be apply to NB-IoT directly.

Reference

- R2-129 NR-NTN-IoT-NTN (Sergio)_ EOM

- RP-250472 WID Rev R19 IOT-NTN cl

- 3GPP TS 36.300 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18)

- 3GPP TS 36.331 V18.2.0 Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification (Release 18)