3GPP TSG RAN WG2 Meeting #129bis	R2-2502706

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.11.2

Source:	CMCC

Title:	Discussion on random access in SBFD

Document for:	Discussion

Introduction

In RAN2#129 meeting, the following agreements have been made for random access in SBFD, shown as [1]:

When a SBFD-aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD-aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

Then in this contribution, we would like to provide our further views on RAN2 impacts to support random access in SBFD.

Discussion

## 2.1 General

In RAN2#127 meeting, the following working assumptions are made for RA events [2]:

- 1. Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

While there is still one open issue, that is, some companies think that RACH resources for Position SI request and SI request may not need to be extended, since it is common for both SBFD-aware UEs and non-SBFD-aware UEs. From our point of view, supporting Position SI request and SI request in SBFD symbol is technically feasible and has the potential to support SBFD-aware UE in obtaining latest Position SI and SI faster. While we are okay if majority think that SBFD operation is not needed for Position SI request and SI request to reduce MAC spec impact.

Proposal 1: Supporting Position SI request and SI request in SBFD symbol is technically feasible. However, we are okay if majority think that SBFD operation is not needed for Position SI request and SI request.

In RAN1#120 meeting, the following conclusion has been made [3]:

Conclusion There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

Since there is no consensus whether to support 2-step RACH or not in RAN1, RAN2 is asked to confirm that 2-step RACH is not supported in R19 and study the RACH configurations for 4-step CBRA and 4-step CFRA. For a comprehensive view, the detailed analysis of the 4-step CBRA and 4-step CFRA PRACH configuration based on Option 1 with Alt 1-1 or Option 2 with Alt 2-3 is conducted in section 2.2.2 and 2.2.3 respectively.

Observation 1: There is no consensus whether to support 2-step RACH or not in RAN1.

Proposal 2: RAN2 is asked to confirm that 2-step RACH is not supported in R19.

## 2.2 	Configuration of RACH resource

### 2.2.1 Background

For legacy NR, there are different CBRA and CFRA events and corresponding PRACH resource indication/configuration signallings as summarized in Table 1.

Table 1. RA events and corresponding PRACH resource indication/configuration signallings

| RA event | CBRA PRACH resource indication/configuration signalling | CFRA PRACH resource indication/configuration signalling |
| --- | --- | --- |
| Initial access from RRC_IDLE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| RRC Connection Re-establishment procedure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronize" | - CBRA resource configured by RACH-ConfigCommon - PDCCH order trigger | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronization status is "non-synchronized" | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| SR failure | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Request by RRC upon synchronous reconfiguration (e.g. handover) | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by RACH-ConfigDedicated |
| RRC Connection Resume procedure from RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| To establish time alignment for a secondary TAG | - N/A | - CFRA resource configured by RACH-ConfigCommon - PDCCH order indicating the used PRACH resource and preamble |
| Request for Other SI | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource and preamble configured by SI-RequestConfig or SI-RequestConfigRepetition |
| Beam failure recovery | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource, preamble and root sequence configured by BeamFailureRecoveryConfig |
| SDT in RRC_INACTIVE | - CBRA resource configured by RACH-ConfigCommon | - N/A |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | - CBRA resource configured by RACH-ConfigCommon | - CFRA resource configured by RACH-ConfigCommon - PDCCH order trigger |
| Early UL synchronization with an LTM candidate cell | - N/A | - CFRA resource configured by EarlyUL-SyncConfig - PDCCH order trigger |
| RACH-based LTM cell switch | - CBRA resource configured by RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger | - CFRA resource configured by RACH-ConfigDedicated or RACH-ConfigCommon - LTM Cell Switch Command MAC CE trigger |

From Table 1, it can be observed that there are multiple events which can trigger random access procedures  and different RA procedures (CBRA or CFRA) may be triggered by different RA events. Besides, it can also be found that CBRA PRACH resource is configured by RACH-ConfigCommon and CFRA PRACH resource can be configured by different resource configuration IEs such as RACH-ConfigDedicated, BeamFailureRecoveryConfig etc., for different RA events.

Observation 2: CBRA PRACH resource is configured by RACH-ConfigCommon.

Observation 3: CFRA PRACH resource can be configured by different resource configuration signallings such as RACH-ConfigDedicated, BeamFailureRecoveryConfig etc., for different RA events.

### 2.2.2 Configuration for 4-step CBRA

- Option 1 with Alt 1-1

- Based on the higher layer parameters determined by RAN1 in R1-2501645, a sbfd-RACHSingleConfig IE should be introduced in SIB1 to indicate whether RACH configuration Option 1 is enabled or not. In terms of separate power control parameters, sbfd-RACHSingleConfig-preambleReceivedTargetPower has been agreed and some other parameters such as: powerRampingStep, preambleTransMax, etc., are still in RAN1 discussion. To this end, an optional field sbfd-RACHSingleConfig-preambleReceivedTargetPower which is only applicable when SBFD RA operation is enabled, can be introduced in RACH-ConfigGeneric. Note that if more power control parameters are agreed in RAN1, RAN2 can then capture them in RRC spec.

- Option 2 with Alt 2-3

- For CBRA configuration Option 2 with Alt 2-3, two separate RACH configurations are configured. Based on the higher layer parameters determined by RAN1 in R1-2501645, a sbfd-RACHDualConfig IE is introduced in BWP-UplinkCommonon IE and one sbfd-RACHDualConfig-ValidROAcrossSymbolTypes field is introduced in sbfd-RACHDualConfig IE, which are marked in green. In addition to BWP-UplinkCommonon, we think that the sbfd-RACHDualConfig IE, which is marked in yellow, can also be introduced in per AdditionalRACH-Config IE for a specific FeatureCombination.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }                                  OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                                    OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                                   OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17                  OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                               OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18   SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF  RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

additionalRACH-perPCI-ToReleaseList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF AdditionalPCIIndex-r17

OPTIONAL,  -- Need N

rsrp-ThresholdMsg1-RepetitionNum2-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum4-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

rsrp-ThresholdMsg1-RepetitionNum8-r18    RSRP-Range                                                      OPTIONAL,  -- Need R

preambleTransMax-Msg1-Repetition-r18     ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}      OPTIONAL   -- Cond Msg1Rep1

]]

[[

sbfd-RACHDualConfig-r19            SetupRelease { SBFD-RACHDualConfig }                                      OPTIONAL,   -- Need M

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

[[

sbfd-RACHDualConfig-r19            SetupRelease { SBFD-RACHDualConfig-r19 }                                      OPTIONAL,   -- Need R

]]

...

}

NumberOfMsg3-Repetitions-r17::=         ENUMERATED {n1, n2, n3, n4, n7, n8, n12, n16}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

-- ASN1START

-- TAG-SBFD-RACHDualConfig-START

SBFD-RACHDualConfig-r19 ::=                SEQUENCE {

rach-ConfigCommon                   RACH-ConfigCommon

sbfd-RACHDualConfig-ValidROAcrossSymbolTypes-r19                  ENUMERATED {true}                                     OPTIONAL,   -- Need R

-- TAG-SBFD-RACHDualConfig-STOP

-- ASN1STOP

Proposal 3: For 4-step CBRA configuration Option 1 with Alt 1-1, a sbfd-RACHSingleConfig IE is introduced in SIB1 and an optional field sbfd-RACHSingleConfig-preambleReceivedTargetPower which is only applicable when SBFD RA operation is enabled, can be introduced in RACH-ConfigGeneric. Wait for RAN1 if more higher layer parameters are needed

Proposal 4: For 4-step CBRA configuration Option 2 with Alt 2-3, a sbfd-RACHDualConfig IE is introduced in BWP-UplinkCommonon IE and one sbfd-RACHDualConfig-ValidROAcrossSymbolTypes field is introduced in sbfd-RACHDualConfig IE.

Proposal 5: For 4-step CBRA configuration Option 2 with Alt 2-3, RAN2 is asked to agree that the sbfd-RACHDualConfig IE can also be introduced in per AdditionalRACH-Config IE for a specific FeatureCombination.

### 2.2.3 Configuration for 4-step CFRA

- In terms of CFRA, it has been agreed in RAN1 as following:

Agreement For CFRA triggered by PDCCH order for SBFD-aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

- For other CFRA events which are not triggered by PDCCH order, the following agreement is made in RAN2#128 meeting [4]:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

- In this case, it seems that there is no need to distinguish between Option 1 with Alt 1-1 and Option 2 with Alt 2-3 for CFRA configuration, since just adding a RO type indication to each IE for CFRA configuration or to related signallings is enough. Note that SI request case needs to be discussed separately because it seems that it is unable to configure the appropriate type of RO per SBFD-aware UE. As given in Observation 2, we think that RAN2 can discuss how to indicate the RO type for each CFRA event when performing configuration (FFS for the SI request case). The detailed analysis are given below:

- Beam failure recovery

- One RO type indication can be introduced in BeamFailureRecoveryConfig IE to indicate whether legacy RO or additional RO should be used for SBFD-aware UE.

- Request by RRC upon synchronous reconfiguration (e.g. handover)

- One RO type indication can be introduced in RACH-ConfigDedicated IE to indicate whether legacy RO or additional RO should be used for SBFD-aware UE.

- Request for Other SI (FFS)

- If SBFD operation is also supported for SI request case, one RO type indication can be introduced in SI-RequestConfig and SI-RequestConfigRepetition IE. However this may imply that all SBFD-aware UE will use the same type of RO, e.g. legacy RO or additional RO.

Observation 4: It seems that there is no need to distinguish between Option 1 with Alt 1-1 and Option 2 with Alt 2-3 for CFRA, since just adding a RO type indication to each IE for CFRA configuration or to related signallings is enough.

Observation 5: One RO type indication can be introduced in SI-RequestConfig and SI-RequestConfigRepetition IE, but this may imply that all SBFD-aware UE will use the same type of RO for SI request.

Proposal 6: RAN2 is asked to agree that there is no need to distinguish between Option 1 with Alt 1-1 and Option 2 with Alt 2-3 for CFRA configuration.

Proposal 7: RAN2 can discuss how to indicate the RO type for each CFRA event when performing configuration (FFS for the SI request case).

Proposal 8: One RO type indication can be introduced in BeamFailureRecoveryConfig IE.

Proposal 9: One RO type indication can be introduced in RACH-ConfigDedicated IE.

Proposal 10: If SBFD operation is also supported for SI request case, one RO type indication can be introduced in SI-RequestConfig and SI-RequestConfigRepetition IE.

## 2.3 Determination of RO type

SBFD-aware UEs need to determine the available RACH resources, that is whether legacy ROs and/or additional ROs can be used. Similar to the case analyzed in section 2.2, the behavior of SBFD-aware UE is also divided into two situations: CBRA or CFRA.

### 2.3.1 Determination of RO type for CBRA

In RAN2#129 meeting, it has been agreed that [1]:

When a SBFD-aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD-aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration.

Therefore, it is clear that if the SBFD-aware UE supports the SBFD RACH configuration option configured by network, both additional RO and legacy RO are available for it. On the other hand, if the SBFD-aware UE do not support the SBFD RACH configuration option configured by network, only legacy RO is available for it. Therefore, we think the available RO types of SBFD-aware UE for CBRA under different configuration cases is very clear.

- Observation 6: The available RO types of SBFD-aware UE for CBRA under different configuration cases is very clear.

Based on capabilities of SBFD-aware UEs and RRACH configuration, if both legacy ROs and additional ROs are available, the initial PRACH transmission attempt and re-attempt should be further discussed.

#### 2.3.1.1 Initial PRACH transmission attempt

##### 2.3.1.1.1 RO type selection rule

In RAN2#129 meeting, the following agreements are made:

For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

In terms of the signalling for indicating the RO type to SBFD-aware UE for CBRA case, SIB1 seems to be natural.

For SBFD, how to define the initial PRACH resource selection rule based on SSB RSRP depends on the motivation of enabling PRACH transmission in SBFD symbols when explicit indication is absent. In RAN1#116 meeting, it has been agreed that [5]:

Conclusion If PRACH is allowed in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode, RAN1 observed the following: - The benefits include at least one or more of the following: - reduced random access latency - reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots - improved coverage of PRACH with sparse UL resources - increased cell range of PRACH with sparse UL resources - PRACH transmissions in UL subband in SBFD symbols may cause UE-to-UE CLI (similar to the case of RRC connected mode UEs) for some deployment scenarios. Initial studies based on two companies’ evaluation results, the DL performance degradation due to UE-to-UE CLI caused by PRACH transmission in SBFD symbols is not significant for indoor office scenario and Urban Macro scenario.

To this end, we think that different RO type selection rules should be supported for different motivations. For example, if the motivation is to improve PRACH capacity and reduce PRACH collision probability, it would be better that cell-center SBFD-aware UEs use the additional ROs, which implies UE should select additional ROs if SSB RSRP is higher than a threshold. If the motivation is to improve the PRACH coverage by using long PRACH format or using PRACH repetition, it is suitable that cell-edge SBFD-aware UEs use the additional ROs, which implies UE should select additional ROs if SSB RSRP is lower than a threshold. Therefore, RAN2 is asked to agree that network can further indicate whether SBFD-aware UE whose RSRP is below the configured RSRP threshold or SBFD-aware UE whose RSRP is above the configured RSRP threshold selects the additional RO. While note that within one cell, only one selection rule for initial RO type is configured/enabled to SBFD-aware UEs at a time for simplicity.

- Observation 7: There are different motivations for enabling RACH in SBFD symbols.

- Proposal 11: For CBRA, the indication of RO type for initial PRACH transmission is introduced in SIB1.

- Proposal 12: For CBRA, network can further indicate whether SBFD-aware UE whose RSRP is below the configured RSRP threshold or SBFD-aware UE whose RSRP is above the configured RSRP threshold selects the additional RO.

- Proposal 13: For CBRA, only one RO type selection rule is supported/enabled in one cell at a time.

Apart from the above analysis, RAN1 has also agreed that:

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

For SBFD, separate RSRP threshold and/or repetition number can be configured for additiona ROs and legacy ROs. For initial attempt, we think that we the same principle should be followed for PRACH transmission with repetition and PRACH transmission without repetition, i.e., select one type between legacy ROs and additional ROs based on explicit indication or configured SSB RSRP threshold.

- Proposal 14: RAN2 is asked to agree that, for initial RO type selection, the same principle should be followed for PRACH transmission with repetition and PRACH transmission without repetition.

Last but not least, some companies think that the behavior of SBFD-aware UEs when both explicit indication and RSRP threshold are absent should be discussed. For us, we prefer the scheme that the RSRP threshold is mandatorily configured, if the both legacy RO and additional RO are configured in one cell. To this end, if no RO type is explicitly indicated by network, a UE selects the RO type based on the DL RSRP threshold as RAN2 agreed. However, if the mentioned scheme can not be agreed, the behavior of SBFD-aware UE depends on its implementation in case that both explicit indication and RSRP threshold are absent.

- Proposal 15: RAN2 is asked to agree that the RSRP threshold for CBRA RO type selection between legacy RO and additional RO is mandatorily configured, if both legacy RO and additional RO are configured in one cell.

- Proposal 16: If Proposal 15 can not be agreed, RAN2 is asked to agree that the behavior of SBFD-aware UE depends on its implementation in case that both explicit indication and RSRP threshold for RO type selection are absent for CBRA.

##### 2.3.1.1.2 RO type selection procedure

In RAN2#129 meeting, it has been agreed that [1]:

FFS whether RO type selection is performed before or after the RA type selection.

From our point of view, RAN2 should firstly discuss whether to perform RO type selection before selection of the set of random access resourced or after that. In legacy NR, the fallback scheme, e.g. 2-step RA fallback to 4-step RA, is performed within the same set of random access resources. With the consideration that we also support at least additional RO fallback to legacy RO for 4-step CBRA, we think that should be also performed within the one set of random access resources. To this end, the RO type selection should be after the selection of set of random access resources which is specified in Section 5.1.1b in TS 38.321.

After that, we think that whether RO type selection is before RA type selection or after RA type selection should be further discussed. Take the rule where additional RO is selected when RSRP is higher than a certain threshold (SBFD-RSRP-Threshold) without considering Msg1 repetition as an example. The following two options are analyzed:

- Option 1: RO type selection is before RA type selection

- For this case, if the SBFD-RSRP-Threshold configured is not higher than msgA-RSRP-Threshold, then only 4-step RA resources including both legacy RO and additional RO can be used by UEs within this cell, as shown in Fig.1 (a). If SBFD-RSRP-Threshold configured is higher than magA-RSRP-Threshold, 4-step RA resources with additional RO, 2-step RA resources and 4-step RA resources with legacy RO can be used successively from cell center to cell-edge, as shown in Fig.1 (b).

- Fig.1 An example of Option 1.

- Option 2: RO type selection is after RA type selection

- For this case, if the SBFD-RSRP-Threshold configured is not lower than msgA-RSRP-Threshold, then only 2-step RA resources and 4-step RA resources with legacy RO can be used by UEs within this cell, as shown in Fig.2 (a). If SBFD-RSRP-Threshold configured is lower than msgA-RSRP-Threshold, 2-step RA resources, 4-step RA resources with additional RO and 4-step RA resources with legacy RO can be used successively from cell center to cell-edge, as shown in Fig.2 (b).

Fig.2 An example of Option 2.

From Fig.1 and Fig.2, it is clear that the relationship between RO type selection and RA type selection affects the RA resources that can be selected by UEs in cell center under the rule where additional RO is selected when RSRP is higher than a certain threshold. It is easy to understand that there will be similar situations when considering above two options, if the rule where additional RO is selected when RSRP is lower than a certain threshold is also supported.

Therefore, we think that RAN2 should carefully design the MAC procedure to capture the RO selection procedure. From our point of view, the RO type selection seems to have higher priority than RA type selection, because in additional to the configured RSRP threshold, network can also indicate the RO type explicitly during RO type selection. However, in general, we are open to both options.

- Observation 8: In legacy NR, the fallback scheme, e.g. 2-step RA fallback to 4-step RA, is performed within the one set of random access resources.

- Observation 9: The relationship between RO type selection and RA type selection affects the RA resources that can be selected by UEs in cell center under the rule where additional RO is selected when RSRP is higher than a certain threshold.

- Proposal 17: The RO type selection is after the selection of set of random access resources which is specified in Section 5.1.1b in TS 38.321.

- Proposal 18: RAN2 is asked to discuss and determine whether to capture the RO selection procedure before or after RA type selection in TS 38.321.

#### 2.3.1.2 PRACH transmission re-attempt

First, for the time being, let's not consider the situation of Msg1 repetition. For PRACH transmission re-attempt, it has been agreed in RAN2#129 meeting that [1]:

FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

- In RAN2, it has been discussed in several RAN2 meetings, but no agreement has been reached yet. With the consideration that the factors which cause the failure of random access attempt includes high interference, collision between multiple UEs, etc., allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently. Besides, since additional ROs may be configured in different format compared to legacy ROs for Option 2, allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may also bring benefits for improved coverage of PRACH case which has been agreed by RAN1 in RAN1#116 meeting [5]. For example, if additional ROs are configured in long format while legacy ROs are in short format, allowing SBFD-aware UEs which is far away from gNB to fallback to additional ROs can bring some benefits. Therefore, RAN2 is asked to support SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs. For the sake of simplicity, within one cell, only one direction, that is, either legacy ROs fallback to additional ROs or additional ROs fallback to additional ROs is supported. To this end, the fallback direction in one cell can be uniformly indicated through simple identification.

Observation 10: Allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently.

Observation 11: Since additional ROs may be configured in different format compared to legacy ROs for Option 2, allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may also bring benefits for some cases.

Proposal 19: RAN2 is asked to support SBFD-aware UEs which select legacy ROs as initial attempt switch to additional ROs after certain (configured) number of times of RACH attempt.

Proposal 20: Within one cell, only one direction, that is, either legacy ROs fallback to additional ROs or additional ROs fallback to additional ROs is supported.

Furthermore, for the case where Msg1 repetition is further taken into consideration, the change in the number of preamble repetition also needs to be further considered in addition to the switching of the RO type. To this end, the first issue is that whether both RO type switch and increase of preamble repetition number are allowed during one RACH procedure. From our point of view, RAN2 is asked to support this because it can bring better performance to 4-step CBRA. With that, we think that both the switching of RO type and increasing on the preamble repetition number should be based on the existing counter PREAMBLE_TRANSMISSION_COUNTER, the values of preambleTransMax-Msg1-Repetition and preambleTransMax-SBFD which are configured by the network determine whether to increase the number of repetitions of Msg1 first or to switch the RO type first. In fact, different alternatives can be considered for RACH configuration Option 1 and RACH configuration Option 2. Assume preambleTransMax-Msg1-Repetition is less than preambleTransMax-SBFD which is configured for RO type switch, then:

- For RACH configuration Option 1, only rsrp-ThresholdMsg1-RepetitionNum2/4/8 can be separately configured:

  - Option 1-1: first increase the number of PRACH repetitions when preamble transmission counter satisfies certain condition and use same RO type (i.e., legacy RO or additional RO) as the earlier PRACH repetitions, then when the number of PRACH repetitions equals the maximum number for RO type switch plus 1, use the other RO type with the same Msg1 repetition number as before the switch.

  - Option 1-2: increase the number of PRACH repetitions when preamble transmission counter satisfies certain condition, and then for each PRACH repetition number, first use the same type as initial attempt and then use the other type.

The following Table 1 gives an example about the above options, with the assumption that repetition number 2, 4 and 8 are configured for RACH configuration 1.

Table 1. Example of RO type and repetition number selection for RACH configuration Option 1

|  | Initial attempt | Re-attempt | Re-attempt | Re-attempt | Re-attempt |
| --- | --- | --- | --- | --- | --- |
| Option 1-1 | additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=4 | legacy RO Repetition N=8 |  |
| Option 1-2 | additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=4 | additional RO Repetition N=8 | legacy RO Repetition N=8 |

It is clear that for Option 1-2, the RO type may been switched multiple times and with different directions, which greatly increases the complexity of the MAC spec. Therefore, we think that Option 1-1 is better.

- For RACH configuration Option 2, both the repetition number and RSRP threshold can be separately configured:

  - Option 2-1: first increase the number of PRACH repetitions when preamble transmission counter satisfies certain condition and use RO groups of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH repetitions. When the number of PRACH repetitions equals the maximum number for RO type switch plus 1, UE can use the other type if the current Msg1 repetition number or higher Msg1 repetition number is also configured for the other type. UE can continue increasing the number of PRACH repetitions until reaches the maximum configured Msg1 repetition number configured for the other RO type.

  - Option 2-2: increase the number of PRACH repetitions when preamble transmission counter satisfies certain condition. For each PRACH repetition number, if the current Msg1 repetition number is both configured for two RO types, first use the same RO type as initial attempt and then use the other type, otherwise, use the RO type configured with current Msg1 repetition number.

The following Table 2 gives an example about the above options, with the assumption that repetition number 4 and 8 are configured for legacy RO and repetition number 2, 4 and 8 are configured for additional RO in RACH configuration 2.

Table 2. Example of RO type and repetition number selection for RACH configuration Option 2

|  | Initial attempt | Re-attempt | Re-attempt | Re-attempt | Re-attempt |
| --- | --- | --- | --- | --- | --- |
| Option 2-1 | additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=4 | legacy RO Repetition N=8 |  |
| Option 2-2 | additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=4 | additional RO Repetition N=8 | legacy RO Repetition N=8 |

The following Table 3 gives another example about the above options, with the assumption that repetition number 8 are configured for legacy RO and repetition number 2, 4 and 8 are configured for additional RO in RACH configuration 2.

Table 3. Another example of RO type and repetition number selection for RACH configuration Option 2

|  | Initial attempt | Re-attempt | Re-attempt |
| --- | --- | --- | --- |
| Option 2-1 | additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=8 |
| Option 2-2 | additional RO Repetition N=2 | additional RO Repetition N=4 | legacy RO Repetition N=8 |

From Table 2, It can also be found that Option 2-2 has same issue as that in Option 1-2. While for  Option 2-1, there is no such issue. Therefore, we think that Option 2-1 is better.

Proposal 21: RAN2 is asked to agree that both RO type switch and increase of preamble repetition number are allowed during one RACH procedure.

Proposal 22: Both the switching of RO type and increasing on the preamble repetition number should be based on the existing counter PREAMBLE_TRANSMISSION_COUNTER.

Proposal 23: For RACH configuration Option 1, the following scheme is supported for PRACH transmission re-attempt with preamble repetitions in one random access procedure:

  - when the number of PRACH transmissions equals the maximum number for RO type switch plus 1, use the other RO type with the same Msg1 repetition number as before the switch. Note that whether to increase the number of repetitions of Msg1 first or to switch the RO type first depends on network configuration.

Proposal 24: For RACH configuration Option 2, the following scheme is supported for PRACH transmission re-attempt with preamble repetitions in one random access procedure:

  - when the number of PRACH transmissions equals the maximum number for RO type switch plus 1, UE can use the other RO type if the current Msg1 repetition number or higher Msg1 repetition number is also configured for the other type. Note that whether to increase the number of repetitions of Msg1 first or to switch the RO type first depends on network configuration.

### 2.3.2 Determination of RO type for CFRA

In RAN1#119 meeting [6], the following agreements are made for CFRA:

Agreement For CFRA triggered by PDCCH order for SBFD-aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order

- And in RAN2#128 meeting [4], similar agreement has also been made as following:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed).

- While in terms of the indication of RO type, we think that there may be three cases with consideration of all the CFRA events. For case 1, the RO type is indicated via 1-bit field in the DCI of PDCCH order which has been agreed by RAN1. While for UL early sync, we think that RAN2 may need to check with RAN1 whether the RO type indication will be included in PDCCH order or in EarlyUL-SyncConfig IE. For case 2, the RO type is indicated in the IE for configuration as our introduction in section 2.2.3. Finally, for case 3, indication of the RO type is indicated in MAC CE. For example, for RACH-based LTM cell switch, the RO type indication can be provided in LTM Cell Switch Command MAC CE.

- As for the SBFD-aware UE’s behavior for determining the RO type for CFRA, we think the they just follow the explicit indication from network.

Observation 12: There may be three cases to indicate the RO type for CFRA, that is, with DCI, with MAC CE or with the IE for CFRA configuration.

Proposal 25: For UL early sync, RAN2 is asked to check with RAN1 whether the RO type indication will be included in PDCCH order or in EarlyUL-SyncConfig IE.

Proposal 26: For RACH-based LTM cell switch, the RO type indication can be provided in LTM Cell Switch Command MAC CE.

## 2.4 Msg3-based early indication

Msg1-based early indication in legacy ROs has been agreed not to be supported in RAN2. For Msg3-based early indication, it has been widely discussed in RAN2#128 but no consensus was reached. If Msg3-based early indication is supported, dedicated LCID(s) can be allocated to SBFD-aware UEs. With that, gNB can identify a SBFD-aware UE based on the logical channel ID in Msg3 and thus schedule subsequent uplink transmission, such as: NAS messages within NAS initial access procedure, etc., in SBFD symbol(s) for SBFD-aware UE. In terms of using reserved LCID(s) in present NR, there is almost no influence on the SBFD-aware UE itself and other UEs. However, if Msg3-based early indication is not supported, gNB can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

- From our point of view, RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs, because there is no impact on PRACH capacity of non-SBFD-aware UEs and can bring benefits in increasing the performance of SBFD operation.

Observation 13: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 14: If Msg3-based early indication is not supported, network can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

Proposal 27: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on random access in SBFD:

Observation 1: There is no consensus whether to support 2-step RACH or not in RAN1.

Observation 2: CBRA PRACH resource is configured by RACH-ConfigCommon.

Observation 3: CFRA PRACH resource can be configured by different resource configuration signallings such as RACH-ConfigDedicated, BeamFailureRecoveryConfig etc., for different RA events.

Observation 4: It seems that there is no need to distinguish between Option 1 with Alt 1-1 and Option 2 with Alt 2-3 for CFRA, since just adding a RO type indication to each IE for CFRA configuration or to related signallings is enough.

Observation 5: One RO type indication can be introduced in SI-RequestConfig and SI-RequestConfigRepetition IE, but this may imply that all SBFD-aware UE will use the same type of RO for SI request.

- Observation 6: The available RO types of SBFD-aware UE for CBRA under different configuration cases is very clear.

- Observation 7: There are different motivations for enabling RACH in SBFD symbols.

- Observation 8: In legacy NR, the fallback scheme, e.g. 2-step RA fallback to 4-step RA, is performed within the one set of random access resources.

- Observation 9: The relationship between RO type selection and RA type selection affects the RA resources that can be selected by UEs in cell center under the rule where additional RO is selected when RSRP is higher than a certain threshold.

Observation 10: Allowing SBFD-aware UEs which select leagcy ROs as initial attempt fallback to additional ROs may offload PRACH transmission and reduce probability of PRACH collision more efficiently.

Observation 11: Since additional ROs may be configured in different format compared to legacy ROs for Option 2, allowing SBFD-aware UEs which select legacy ROs as initial attempt fallback to additional ROs may also bring benefits for some cases.

Observation 12: There may be three cases to indicate the RO type for CFRA, that is, with DCI, with MAC CE or with the IE for CFRA configuration.

Observation 13: With Msg3-based early indication, there is almost no impact on PRACH capacity of non SBFD-aware UEs.

Observation 14: If Msg3-based early indication is not supported, network can only identify a SBFD-aware UE through UE capability transfer which is later than that through Msg3-based early indication.

General

Proposal 1: Supporting Position SI request and SI request in SBFD symbol is technically feasible. However, we are okay if majority think that SBFD operation is not needed for Position SI request and SI request.

Proposal 2: RAN2 is asked to confirm that 2-step RACH is not supported in R19.

Configuration for 4-step CBRA

Proposal 3: For 4-step CBRA configuration Option 1 with Alt 1-1, a sbfd-RACHSingleConfig IE is introduced in SIB1 and an optional field sbfd-RACHSingleConfig-preambleReceivedTargetPower which is only applicable when SBFD RA operation is enabled, can be introduced in RACH-ConfigGeneric. Wait for RAN1 if more higher layer parameters are needed

Proposal 4: For 4-step CBRA configuration Option 2 with Alt 2-3, a sbfd-RACHDualConfig IE is introduced in BWP-UplinkCommonon IE and one sbfd-RACHDualConfig-ValidROAcrossSymbolTypes field is introduced in sbfd-RACHDualConfig IE.

Proposal 5: For 4-step CBRA configuration Option 2 with Alt 2-3, RAN2 is asked to agree that the sbfd-RACHDualConfig IE can also be introduced in per AdditionalRACH-Config IE for a specific FeatureCombination.

Configuration for 4-step CFRA

Proposal 6: RAN2 is asked to agree that there is no need to distinguish between Option 1 with Alt 1-1 and Option 2 with Alt 2-3 for CFRA configuration.

Proposal 7: RAN2 can discuss how to indicate the RO type for each CFRA event when performing configuration (FFS for the SI request case).

Proposal 8: One RO type indication can be introduced in BeamFailureRecoveryConfig IE.

Proposal 9: One RO type indication can be introduced in RACH-ConfigDedicated IE.

Proposal 10: If SBFD operation is also supported for SI request case, one RO type indication can be introduced in SI-RequestConfig and SI-RequestConfigRepetition IE.

RO type selection rule for CBRA

- Proposal 11: For CBRA, the indication of RO type for initial PRACH transmission is introduced in SIB1.

- Proposal 12: For CBRA, network can further indicate whether SBFD-aware UE whose RSRP is below the configured RSRP threshold or SBFD-aware UE whose RSRP is above the configured RSRP threshold selects the additional RO.

- Proposal 13: For CBRA, only one RO type selection rule is supported/enabled in one cell at a time.

- Proposal 14: RAN2 is asked to agree that, for initial RO type selection, the same principle should be followed for PRACH transmission with repetition and PRACH transmission without repetition.

- Proposal 15: RAN2 is asked to agree that the RSRP threshold for CBRA RO type selection between legacy RO and additional RO is mandatorily configured, if both legacy RO and additional RO are configured in one cell.

- Proposal 16: If Proposal 15 can not be agreed, RAN2 is asked to agree that the behavior of SBFD-aware UE depends on its implementation in case that both explicit indication and RSRP threshold for RO type selection are absent for CBRA.

RO type selection procedure for CBRA

- Proposal 17: The RO type selection is after the selection of set of random access resources which is specified in Section 5.1.1b in TS 38.321.

- Proposal 18: RAN2 is asked to discuss and determine whether to capture the RO selection procedure before or after RA type selection in TS 38.321.

PRACH transmission re-attempt

Proposal 19: RAN2 is asked to support SBFD-aware UEs which select legacy ROs as initial attempt switch to additional ROs after certain (configured) number of times of RACH attempt.

Proposal 20: Within one cell, only one direction, that is, either legacy ROs fallback to additional ROs or additional ROs fallback to additional ROs is supported.

Proposal 21: RAN2 is asked to agree that both RO type switch and increase of preamble repetition number are allowed during one RACH procedure.

Proposal 22: Both the switching of RO type and increasing on the preamble repetition number should be based on the existing counter PREAMBLE_TRANSMISSION_COUNTER.

Proposal 23: For RACH configuration Option 1, the following scheme is supported for PRACH transmission re-attempt with preamble repetitions in one random access procedure:

  - when the number of PRACH transmissions equals the maximum number for RO type switch plus 1, use the other RO type with the same Msg1 repetition number as before the switch. Note that whether to increase the number of repetitions of Msg1 first or to switch the RO type first depends on network configuration.

Proposal 24: For RACH configuration Option 2, the following scheme is supported for PRACH transmission re-attempt with preamble repetitions in one random access procedure:

  - when the number of PRACH transmissions equals the maximum number for RO type switch plus 1, UE can use the other RO type if the current Msg1 repetition number or higher Msg1 repetition number is also configured for the other type. Note that whether to increase the number of repetitions of Msg1 first or to switch the RO type first depends on network configuration.

Determination of RO type for CFRA

Proposal 25: For UL early sync, RAN2 is asked to check with RAN1 whether the RO type indication will be included in PDCCH order or in EarlyUL-SyncConfig IE.

Proposal 26: For RACH-based LTM cell switch, the RO type indication can be provided in LTM Cell Switch Command MAC CE.

Msg3-based early indication

Proposal 27: RAN2 is proposed to support Msg3-based early indication for SBFD-aware UEs.

Reference

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others”, RAN2#129, Athens, Greece.

- “Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD”, RAN2#127, Maastricht, Netherlands.

- “RAN1 Chair’s Notes”, RAN1#120, Athens, Greece.

- “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, and SBFD”, RAN2#128, Orlando, USA.

- “RAN1 Chair’s Notes”, RAN1#116, Athens, Greece.

- “RAN1 Chair’s Notes”, RAN1#119, Orlando, USA.