3GPP TSG-RAN WG2 Meeting #129bis	R2-2502709

Wuhan, China, 7 – 11 Apr, 2025

Agenda Item:	8.10.5

Source:	CMCC

Title:	MHI/UHI Enhancement for SCG Deactivation/Activation

Document for:	Discussion

# 1 Introduction

In RAN2#127 meeting, potential MHI/UHI Enhancement for SCG Deactivation/Activation was discussed and following agreements achieved [1]:

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

Then, in the RAN3#126, RAN3 discussed the solution of MHI/UHI Enhancement for SCG Deactivation/Activation, and understands the UE-based solution is beneficial for monitoring SCG deactivation/activation across RRC state transitions, which included in the reply LS from RAN3 [2]:

RAN3 thanks RAN2 for the LS on MHI enhancement solution for SCG deactivation/activation. RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation. RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

In RAN2#129, RAN2 has the following agreements:

=> We will introduce a UE based solution for SCG act/deact MHI, as an optional UE feature. => UE reports the time the UE has spent in PSCell with SCG activated, TBD details, e.g. percentage of time or absolute time, etc.

Based on the previous discussion, we further provide our understanding for the details of UE-based solution.

# 2 Discussion

Based on the agreements of RAN2#129, RAN2 has agreed to introduce UE based solution for MHI SCG activation/ deactivation and UE reports the time UE has spent in PSCell with SCG activated, but the details are FFS.

Two ways to represent SCG activated time:

Opt 1: The accumulated time/absolute time of UE in SCG activated state

Opt 2: The ratio of time of SCG activation

Observation 1: Two options for the detail of time information  of UE spent in PSCell SCG activation state:

Opt 1: The accumulated time/absolute time of UE in SCG activated state

Opt 2: The ratio of time of SCG activation

RAN3 has already initially discussed the detailed time information but no agreement achieved. As the current VisitedCellInfoList included in MHI already including the timeSpent-r16, which maximum value is 4095s, the ratio based on the timeSpent-r16 seems the straightforward solution to represent the time UE spends in SCG activated state. However, this approach cannot provide a accurately indication what the exact time UE spent in SCG activation state.

The accumulated time, which is the sum of time of UE spent in separate SCG activation time window of the PSCell, is a more accurate value to indicate the time information. Based on the accumulated time, the network can accurately get how long the PSCell is activated or deactivated. But, one issue for this option should be considered. Since the maximum value of existing timeSpent-r16 is 4095s, and when the duration of stay exceeds 4095s, the UE shall set it to 4095s, which means that in some cases both the absolute time in SCG activation and time spent in the PSCell should be set to 4095s. The network cannot distinguish whether UE spent in the SCG activation state is 4095s or more than 4095s or whether the UE spent in the SCG activation state is less than the duration of UE stays in the PSCell, as for all cases, the value of both IE will only be set to 4095s. However, from our understanding, as the SCG activation time over 4095s is long enough to prove DC is needed, when the value of time spent in SCG activation is set to 4095s, no need to distinguish whether the SCG activation state is 4095s or more than 4095s. So, we slightly prefer Opt 1.

Observation 2: The accumulated time is a more accurate value to indicate the time information as the network can accurately get how long the PSCell is activated or deactivated.

Proposal 1: Include the accumulated time of UE spent in SCG activation state in MHI.

# 3 Conclusion

This contribution provides our considerations on the MHI/UHI enhancement for SCG activation and deactivation, enabling network to understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, following observations and proposal are made:

Observation 1: Two options for the detail of time information  of UE spent in PSCell SCG activation state:

Opt 1: The accumulated time/absolute time of UE in SCG activated state

Opt 2: The ratio of time of SCG activation

Observation 2: The accumulated time is a more accurate value to indicate the time information as the network can accurately get how long the PSCell is activated or deactivated.

Proposal 1: Include the accumulated time of UE spent in SCG activation state in MHI.

# Reference

[1]	Draft_R2_127_Meeting_Report

[2] R3-247906, Reply LS on MHI enhancement solution for SCG deactivation/activation, Huawei