3GPP TSG-WG2 Meeting #129bis			R2-2502718

Wuhan, China, Apr. 7th-11th, 2025

Agenda item:	8.7.4.1

Title:	Further consideration on LCP enhancement for XR

Source:	CMCC

Document for:	Discussion

Introduction

The last meeting [1] discusses some critical issues on LCP enhancement for XR and leaves the following agreements and FFS:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

This contribution discusses further details on LCP enhancement for XR, especially on intra-UE prioritization.

Discussion

2.1 Bj for LCH priority-adjusted data

A negative Bj typically indicates that the configured PBR is insufficient to accommodate the existing XR service data rate, leading to potential transmission delays or even starvation for certain logical channels.

This could be particularly detrimental for delay-sensitive services, as their PDU/PDU sets might be discarded before transmission. In scenarios where additional priority is configured to an LCH to facilitate latency-sensitive traffic, LCP based solely on Bj constraints may counteract the intended benefit of priority adjustment.

Therefore, allowing an LCH with priority-adjusted data to be transmitted even when Bj is negative could help mitigate excessive delay, provided that this behavior is explicitly controlled by the gNB to prevent unintended resource depletion or starvation of other traffic.

Furthermore, the network's configuration of the additional priority may depend on whether transmission is permitted with a negative Bj. If LCH priority-adjusted data is always allowed to be transmitted, the gNB may configure a moderate additional priority to balance resource allocation. Conversely, if transmission remains subject to positive Bj constraints, the gNB may opt for a higher additional priority to ensure timely delivery of delay-sensitive traffic. This highlights the interplay between priority configuration and Bj management in achieving an optimal balance between resource efficiency and latency requirements.

Observation 1: Allowing transmission of LCH priority-adjusted data when Bj is negative could improve the delay performance for latency-sensitive traffic while requiring network-controlled configuration to prevent potential resource contention.

Proposal 1: Allow an LCH with an upgraded priority to be transmitted even when Bj is negative, provided that this behaviour is explicitly configured by the network.

2.2 Specification impact of intra-UE prioritization

The determination of the priority of an uplink grant within a UE follows the framework defined in TS 38.321 clause 5.4.1. When lch-basedPrioritization is configured, the priority of an uplink grant is determined by the highest priority among the logical channels that are either already stored in the HARQ buffer or have available data for multiplexing, subject to the mapping restrictions outlined in clause 5.4.3.1.2.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2.

With the introduction of an additional priority for LCHs, there is a potential impact on intra-UE prioritization since the priority used for uplink grant selection may differ from the originally configured LCH priority.

If the additional priority is incorporated into the intra-UE prioritization process, it could influence which logical channels are granted uplink resources first, potentially affecting overall resource allocation scheme. However, since the existing LCP mechanism already considers the additional priority, the specification impact remains relatively moderate. The extent of specification updates required depends on whether the clause 5.4.3.1.2 needs to be explicitly revised to accommodate the additional LCH priority.

Observation 2: Integrating the additional LCH priority into intra-UE prioritization could influence uplink grant selection, but the impact on existing specifications is limited since current LCP procedure already includes additional priority.

Proposal 2: There will be few impacts on including the additional LCH priority in the uplink grant priority determination.

2.3 Additional priority impact on SR

The MAC entity may be configured with multiple SR configurations, where each SR configuration is associated with one or more logical channels. Each LCH can be associated with only one SR configuration. Hence, if additional priority is used for SR, LCH may use the SR configuration for other LCH or LCH may be configured with an additional SR configuration.

If an SR is triggered by LCH-priority adjusted data, associating the SR configuration with the LCH that has the same priority as the adjusted data would allow the gNB to infer the relative urgency of the transmission more accurately. This approach could enhance scheduling efficiency by ensuring that SR-based resource requests better align with the actual priority of the data requiring transmission. However, it introduces additional configuration complexity, as the network must ensure that the SR mapping correctly reflects the dynamically adjusted priority of the LCH.

Observation 3a: Aligning the SR configuration with the adjusted LCH priority improves the accuracy of priority signalling, at a cost of both UE and NW implementation complexity, e.g., using SR configuration for other LCH or configuring multiple SR resource for one LCH.

Observation 3b: SR is usually triggered by BSR/DSR, the priority of SR is not that critical as long as NW allocates a UL grant accommodating BSR or DSR MAC CE, which NW usually does.

Proposal 3: SR is not affected by additional priority.

Conclusion

This contribution offers the following observations and proposals:

Impact on Bj:

Observation 1: Allowing transmission of LCH priority-adjusted data when Bj is negative could improve the delay performance for latency-sensitive traffic while requiring network-controlled configuration to prevent potential resource contention.

Proposal 1: Allow an LCH with an upgraded priority to be transmitted even when Bj is negative, provided that this behaviour is explicitly configured by the network.

Intra-UE prioritization:

Observation 2: Integrating the additional LCH priority into intra-UE prioritization could influence uplink grant selection, but the impact on existing specifications is limited since current LCP procedure already includes additional priority.

Proposal 2: There will be few impacts on including the additional LCH priority in the uplink grant priority determination.

Observation 3a: Aligning the SR configuration with the adjusted LCH priority improves the accuracy of priority signalling, at a cost of both UE and NW implementation complexity, e.g., using SR configuration for other LCH or configuring multiple SR resource for one LCH.

Observation 3b: SR is usually triggered by BSR/DSR, the priority of SR is not that critical as long as NW allocates a UL grant accommodating BSR or DSR MAC CE, which NW usually does.

Proposal 3: SR is not affected by additional priority.

References

[1]	R2_129 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM