3GPP TSG-RAN WG2 Meeting #129bis	R2-2502721

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.3

Source:	CMCC

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

Introduction

At the RAN2#129 meeting we reached the following agreements [1]:

RAN2 #129: Agreements on L1 event triggered MR - Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". - Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). - Do not create a new resource configuration just for the event triggered measurements. - Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused". - Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report. - Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue. - TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval. - Cell specific offset is not supported for LTM L1 event evaluation. - The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity. - Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. - Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam. - R18 differential RSRP encoding is used as baseline. - LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant. - Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)) - Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP - Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur. - For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used. - S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

Discussion

Event triggered reporting configuration

The configuration process of R19 LTM still follows the framework of R18 LTM, where the source gNB generates a common RS resource pool and then sends the LTM-CSI-ResourceConfig IE to all candidate cells. The current TS 38.300 running CR is as follows:

Based on the conclusions of the RAN2 meeting, the LTM-CSI-ReportConfig IE and LTM-CSI-ResourceConfig IE in R18 will be reused. The current TS 38.331 running CR is as follows. Similar to the SSB in R18, the NZP-CSI-RS-ResourceId and LTM-CandidateId-r18 also have a one-to-one correspondence. This means that a single LTM-CSI-ResourceConfig can include multiple measurement resources for multiple candidate cells. Likewise, in LTM-CSI-ReportConfig, the LTM-CSI-ResourceConfigId-r18 remains uniquely associated with the LTM-CSI-ResourceConfig.

However, for R19 event-triggered measurement reporting, the mechanism where all candidate cells share the same common RS resource pool and a single reporting config corresponds to multiple candidate cell measurement resources may have some shortcomings [2]. The main issues are as follows:

- The resource configuration is generated by the source cell, while the reporting configuration is generated by each candidate cell. In the subsequent LTM scenario, when a candidate cell becomes the new source cell, it may not want to apply the same reporting configuration to all candidate cells due to existing differences.

- In legacy L3 measurements, the network may configure events for different purposes, such as Event A4, which is commonly used for load balancing. Similarly, in LTM, there may be similar scenarios where different events are configured for different candidate cells.

Therefore, a possible solution that does not affect the existing configuration framework is to introduce a candidate cell identifier, e.g., Candidate ID, in the LTM-CSI-ReportConfig. This would allow for greater flexibility in reporting configuration for different candidate cells.

Proposal 1: In the reporting configuration, further indicate which candidate cells' RS resources can trigger the evaluation of the configured events(e.g., via Candidate ID).

Event triggered reporting content

Issue 1: Serving cell beam reporting

If the serving cell is configured as one of the candidate cells, the UE can report serving cell beams in the measurement report. Moreover, these serving cell beams are included within the maximum reported beam number N.

Proposal 2: If the serving cell is configured as one of the candidate cells, the UE can report serving cell beams in the measurement report. Moreover, these serving cell beams are included within the maximum reported beam number N.

Another question is whether to allow reporting of current beam. For event-triggered measurement reporting, the current beam for event evaluation is SSB which is QCLed with the QCL RS in the indicated TCI state or QCL RS in the indicated TCI state. The measurement result of this beam reported to gNB is beneficial for gNB to know the quality of both the current beam of serving cell and candidate cells, which could provide more information for gNB to determine whether cell switch is triggered or not.

The measurement reporting of R18 LTM reuses the CSI reporting framework, and whether the measurement results of serving cell beams is included in the CSI report can be configured by RRC parameter. However, for the measurement reports of the current beam and event-triggered reporting, the situation is somewhat different. In our view, when a configured event is triggered, it indicates a potential handover. At this moment, it is meaningful for the gNB to be aware of the quality of the current beam. Therefore, in the MAC CE of the event-triggered measurement report, the measurement result of the current beam should always be included.

If the current beam for event evaluation is reported, the reported beam is not counted in the maximum reported beam number N. In other words, if the current beam for event evaluation is included in the measurement report, the maximum number of reported beams is (N +1) beams.

In addition, since gNB and UE are aligned on the SSBRI/CRI of current beam for event evaluation, so UE does not need to report the corresponding SSBRI/CRI of current beam in the measurement report.

Proposal 3: In the MR MAC CE of event-triggered measurement reporting, the measurement result of the current beam is always included, and the reported beam is not counted in the maximum reported beam number N.

Proposal 4: When reporting the measurement results of the current beam, UE does not need to report the corresponding SSBRI/CRI in the beam report.

Issue 2: Need of explicit indication to inform whether the reported beam meets the event condition?

First, let's clarify why an explicit indication is needed to inform whether the reported beam meets the event condition. As shown in the figure below, when Beam 2 satisfies the event condition and triggers the measurement report, there may be another Beam 4 that, although of sufficient quality, has not yet completed the TTT evaluation. Therefore, the gNB cannot determine whether the reported beam meets the event condition solely based on the measurement value.

Fig. 1

Observation 1：The reason for needing an explicit indication to inform whether the reported beam meets the event condition is that, when triggering the measurement report, there may be beams with sufficient quality that have not yet completed the TTT evaluation.

In the RAN2#129 meeting, although the explicit indication in the MR MAC CE was agreed, the detailed signaling has not yet been clarified.

RAN2#129 Agreement - Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signaling.

In the email discussion, we agreed to use differential RSRP as the baseline, meaning the absolute RSRP value of the strongest beam should be used as the reference. However, we believe it is necessary to clarify that this strongest reference beam should meet the event condition, meaning it must have completed the TTT evaluation, such as Beam 2 in Fig. 1.

Proposal 5: For differential RSRP, the first reported beam in MAC CE should be the beam with the highest RSRP value that has completed the TTT evaluation.

Specifically, as shown in Fig. 1, both Beam 3 and Beam 4 are beams that do not meet the event condition. the gNB can differentiate between the two based on the measurement values. Therefore, for the reported beams, it need to include an one bit flag that indicates whether the beam has fulfilled the event condition or not.

However, for example, Beam 4, its RSRP value may be higher than Beam 2, but it has not yet completed the TTT evaluation. In the differential RSRP framework, Beam 4 would require an additional bit to indicate that the absolute value corresponding to its differential value is higher than the reference value. Therefore, a total of two bits are needed to indicate beams that do not meet the event condition.

Proposal 6: For the reported beams with differential RSRP, two bits of indication are required: one bit to indicate whether the beam meets the event condition and one bit to indicate whether the absolute value corresponding to the differential value is higher than the reference value.

Issue 3: Need of explicit indication to inform whether the reported beam leaves the event condition?

The final question is whether an explicit indication is needed to inform whether the reported beam leaves the event condition. In practice, the gNB can make an implicit judgment based on the SSBRI/CRI and the corresponding measurement results in the received measurement reports.

Taking Beam 1 in Fig. 1 as an example:

- When it meets the event condition and triggers a measurement report, the report will contain the corresponding SSBRI/CRI along with a measurement result above the threshold.

- Later, when it meets the leaving condition and triggers another measurement report, the report will again carry the same SSBRI/CRI, but with a measurement result below the threshold.

Therefore, by matching the SSBRI/CRI and comparing the measurement results before and after, the gNB can determine that this beam has met the leaving condition.

Proposal 7: No explicit indication is needed to inform whether the reported beam leaves the event condition, as the gNB can make an implicit judgment based on the comparison of SSBRI/CRI and measurement results.

MR MAC CE format

Based on the above discussion, the MR MAC CE format is shown below:

Fig.2 MR MAC CE format with differential RSRP

- T: Indication of whether the beam meets the event condition.

- S: Indication of whether the absolute value corresponding to the differential value is higher than the reference value.

Proposal 8: RAN2 to agree the MAC CE format in Figure.2 as the baseline.

LS reply：Support of semi-persistent CSI-RS

In RAN1#118bis meeting, the working assumption regarding whether to support semi-persistent CSI-RS for candidate cell L1-RSRP measurement for gNB scheduled reporting is left for further discussion based on the Reply LS from RAN3.

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283.

Since the Reply LS from RAN3 has confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS[3]. For gNB scheduled L1-RSRP measurement and reporting, compared with periodic CSI-RS, semi-persistent CSI-RS can be activated and deactivated flexibly for UE to measure the CSI-RSs for a subset of candidate cells, which could reduce the measurement and report overhead for UE.

The signaling procedure can be summarized as follows:

Fig.3 Activation procedure

First, the serving cell sends an activation request for the semi-persistent CSI-RS resource to the candidate cell. Upon receiving the request, the candidate cell starts transmitting the corresponding semi-persistent CSI-RS according to the RRC configuration and sends a success confirmation back to the serving cell. Finally, after receiving the successful activation confirmation, the serving cell notifies the UE via a dedicated MAC CE, allowing it to begin the corresponding semi-persistent CSI-RS measurements.

Fig.4 Deactivation procedure

For deactivation, the serving cell does not need to wait for the candidate cell's feedback before notifying the UE to deactivate the semi-persistent CSI-RS resource. The process is considered complete once the serving cell receives the successful deactivation confirmation from the candidate cell.

In the current RRC running CR, the CSI-RS resource configuration is as follows:

That is, the CSI-RS resource type will be defined in the LTM-Candidate IE, and the LTM-CSI-RS-ResourceSet-r19 IE in LTM-CSI-ResourceConfig will only contain CSI-RS resources of the same type. Therefore, when activating semi-persistent CSI-RS resources, the LTM-CSI-ResourceConfig ID can be used for indexing. The MAC CE can be designed as follows:

Fig.5 SP CSI-RS Resource Activation/Deactivation MAC CE

Proposal 9: RAN2 to agree the MAC CE format in Figure.5 as the baseline.

Conclusions

In this contribution, we analyse for measurement event evaluation, following are the observations and proposals.

Observations:

Observation 1：The reason for needing an explicit indication to inform whether the reported beam meets the event condition is that, when triggering the measurement report, there may be beams with sufficient quality that have not yet completed the TTT evaluation.

Proposals:

Proposal 1: In the reporting configuration, further indicate which candidate cells' RS resources can trigger the evaluation of the configured events(e.g., via Candidate ID).

Proposal 2: If the serving cell is configured as one of the candidate cells, the UE can report serving cell beams in the measurement report. Moreover, these serving cell beams are included within the maximum reported beam number N.

Proposal 3: In the MR MAC CE of event-triggered measurement reporting, the measurement result of the current beam is always included, and the reported beam is not counted in the maximum reported beam number N.

Proposal 4: When reporting the measurement results of the current beam, UE does not need to report the corresponding SSBRI/CRI in the beam report.

Proposal 5: For differential RSRP, the first reported beam in MAC CE should be the beam with the highest RSRP value that has completed the TTT evaluation.

Proposal 6: For the reported beams with differential RSRP, two bits of indication are required: one bit to indicate whether the beam meets the event condition and one bit to indicate whether the absolute value corresponding to the differential value is higher than the reference value.

Proposal 7: No explicit indication is needed to inform whether the reported beam leaves the event condition, as the gNB can make an implicit judgment based on the comparison of SSBRI/CRI and measurement results.

Proposal 8: RAN2 to agree the MAC CE format in Figure.2 as the baseline.

Proposal 9: RAN2 to agree the MAC CE format in Figure.5 as the baseline.

References

- Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES.

- R2-2500443, ZTE Corporation, Discussion on L1 event triggered MR.

- R3-247911 Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells.