3GPP TSG-RAN WG2 Meeting #129bis	                        R2-2502722

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.4.3

Source: 	CMCC

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:	Discussion

- Introduction

At the RAN2#129 meeting, we had the following agreements[1]:

- The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements.

- If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

At the RAN2#128 meeting, we had the following agreements[2]:

- The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

- FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

- FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

At the RAN2#127bis meeting, we reached the following agreements[3]:

Working assumption

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

- FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

At the RAN2#127 meeting, we reached the following agreements[4]:

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

- The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

- The exit condition is based on the LR measurement results.

- Discussion

2.1 Clarification of Cases

In the LP-WUS discussion of RAN4, they listed several cases for measurement offloading/relaxation. In order to avoid duplication of work and conduct efficient discussions within a limited time, RAN2 should prioritize research based on the cases approved by RAN4.

Case #1 and #3 have been agreed by RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

RAN4 to further discuss Case #2:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

Based on the definitions of the cases mentioned above, we must clarify the scope of application for these three cases, as this is critical to ensure clarity in the following discussions.

For Case #1 and Case #2, according to RAN4 definitions, MR is turned off, meaning neighbor cell measurements are not required. Therefore, the thresholds for Case #1 and Case #2 should at least be greater than SIntraSearchP. Based on this, we first discuss whether to support Case #2. In urban environments, due to the irregular shapes of cells, the UE may suddenly detect a better cell at a corner, leading to a handover and thereby requiring a conservative configuration of SIntraSearchP. In rural environments, cells are typically regular circular shapes, and the network can configure a lower intra-frequency measurement trigger threshold to help the UE save power. As shown in figure 1:

Figure 1

So in our opinion, whether Case #2 is needed depends on the SIntraSearchP configured by the network. If SIntraSearchP is configured with a lower value, such as in rural environments, which means that the UE does not need to perform neighbor measurements in a larger area, then it is reasonable to support Case #2 as a supplement to Case #1 or to use Case #2 only. In short, Case #2 can be supported, but it depends on the network configuration.

Observation 1: Case #2 is a possible case depending on network configuration for various scenarios, e.g. urban environments and rural environments.

Proposal 1: For Case #1 or #2, the entry threshold is equal to or higher than SIntraSearchP.

Further, if both Case #1 and Case #2 are allowed to be configured, then the Case #2 region should be between the Case #1 threshold and SIntraSearchP.

For Case #3, RAN4 agreed that MR RRM relaxation scaling factor for serving cell/neighbour cell is 16, which is significantly larger than that of legacy Rel-16 UE power saving WI (i.e., 3).

RAN4#114 Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16.

Considering that LR cannot assist MR in performing neighbor cell measurement, in our opinion, R19 Neighboring cell measurement relaxation should have stricter evaluation conditions than R16. Therefore, the MR threshold in the entry condition of R19 Neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16.

Proposal 2: The threshold in the entry condition of R19 measurement relaxation should be higher than SSearchThresholdP in R16, and lower than SIntraSearchP.

Based on the above discussion, the relationship between Cases is shown in the following figure:

Figure 2 Clarification of Cases

2.2 Case #1

Considering that the latency of starting MR is as high as hundreds of milliseconds, we should be very cautious about offloading serving cell measurements to LR. If a UE is in a high mobility state, it is likely to reach the cell edge quickly, so the UE should use MR to perform neighbor measurements to prepare for possible handovers. If the UE performs measurement offloading and turns off MR at this time, on the one hand, due to the extremely high startup delay of the MR, the UE is likely to miss the best handover opportunity; on the other hand, it will also cause unnecessary conversion power consumption. Therefore, the mobility of UE should be considered for serving cell measurement offloading, that is, measurement offloading should not be performed when the UE is in a high-speed state.

Proposal 3: UEs with high-mobility do not perform serving cell measurement offloading.

2.3 Case #3

For Case #3, we believe there are three key issues to focus on:

- The transition between Case #3 and Case #1/2. The original entry/exit criteria for Case #3 are simply defined as being above or below a certain threshold, which is inaccurate.

- According to RAN4's conclusions, the MR relaxation factor for Case #3 has been set to 16, which is a highly aggressive value. Therefore, when defining the entry/exit conditions for Case #3, caution should be exercised, and LR measurement results should be fully utilized to enhance reliability.

- The relationship between measurement relaxation for the serving cell and neighboring cells in Case #3.

Case #3 entry/exit condition

We would like to revisit the entry condition for Case #3, as we believe the conclusion from the RAN2 #128 and #129 meeting was not rigorous enough.

RAN2#128 Agreements - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

RAN2#129 Agreements - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

RAN4 has agreed on an MR relaxation factor of 16 for Case #3, which will significantly reduce the reliability of MR measurements. At the same time, the MR ramp-up delay in Case #1 is quite large and results in higher power consumption. Considering these two factors, we believe that the entry/exit criteria should follow the principle of "strict entry, relaxed exit."

Specifically, the entry conditions for Case #3 should consider two aspects. First, when the UE needs to initiate neighbor cell measurements and fallback from Case #1/2 to Case #3, the entry condition for Case #3 is equivalent to the exit condition of Case #1/2. According to the "relaxed exit" principle, the entry condition for Case #3 in this scenario can be defined as: Serving cell quality measured by MR or LR is lower than SIntraSearchP but higher than the configured threshold, e.g. RSRP and/or RSRQ.

Secondly, for the scenario where the UE enters Case #3 from the cell edge, following the "strict entry" principle, the entry condition for Case #3 can be defined as: Both serving cell quality measured by MR and LR are above the configured thresholds, e.g. RSRP and/or RSRQ(LR threshold is mandatory). As shown in Fig 3.

Figure 3 Case #3 entry/exit condition

Proposal 4: For case #3, the entry condition for serving cell RRM relaxation can be defined as:

- Serving cell quality measured by MR or LR is lower than SIntraSearchP but higher than the configured threshold, e.g. RSRP and/or RSRQ; or

- Both serving cell quality measured by MR and LR are above the configured thresholds, e.g. RSRP and/or RSRQ(LR threshold is mandatory).

As for the exit conditions, there is no need to define separate conditions, just define it as not meeting the entry conditions.

Proposal 5: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

Proposal 6: For case #3, the exit condition for serving cell RRM relaxation can be defined as:

- Both serving cell quality measured by MR and LR are higher than SIntraSearchP, e.g. RSRP and/or RSRQ; or

- Serving cell quality measured by MR or LR is lower than the configured threshold, e.g. RSRP and/or RSRQ.

neighboring cells issue

In RAN2#126, we agreed that the RRM measurement of the neighboring cell can only be performed by MR. In RAN2#127bis and #128, we reached the following agreements:

RAN2#127bis Agreements Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 Agreements - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

For the ‘UE not at cell edge’ criterion, if the serving cell applies measurement relaxation while neighboring cells do not, it will significantly impact power-saving gains. Moreover, it introduces more complex procedures without providing sufficient positive benefits.

Proposal 7: The entry condition for R19 neighbour cell RRM measurement relaxation is the same as R19 serving cell RRM measurement relaxation.

For the ‘UE with low mobility’ criterion, we think it is necessary. Usually we think that a high-speed mobile UE has a higher handover probability, so the UE needs to perform accurate neighbor cell measurement. However, the relaxation factor currently discussed by RAN4 is very large. If a high-speed UE performs R19 neighbor cell measurement relaxation, it will bring risks in terms of mobility.

Proposal 8: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

- Conclusions

In this contribution, we discussed the LP-WUS measurement relaxation/offloading in RRC_IDLE/INACTIVE mode, and following are our observations and proposals.

Observations:

Observation 1: Case #2 is a possible case depending on network configuration for various scenarios, e.g. urban environments and rural environments.

Proposals:

Proposal 1: For Case #1 or #2, the entry threshold is equal to or higher than SIntraSearchP.

Proposal 2: The threshold in the entry condition of R19 neighboring cell measurement relaxation should be higher than SSearchThresholdP in R16, and lower than SIntraSearchP.

Proposal 3: UEs with high-mobility do not perform serving cell measurement offloading.

Proposal 4: For case #3, the entry condition for serving cell RRM relaxation can be defined as:

- Serving cell quality measured by MR or LR is lower than SIntraSearchP but higher than the configured threshold, e.g. RSRP and/or RSRQ; or

- Both serving cell quality measured by MR and LR are above the configured thresholds, e.g. RSRP and/or RSRQ(LR threshold is mandatory).

Proposal 5: No separate exit condition is needed, and the exit condition can be defined as failing to meet the entry condition.

Proposal 6: For case #3, the exit condition for serving cell RRM relaxation can be defined as:

- Both serving cell quality measured by MR and LR are higher than SIntraSearchP, e.g. RSRP and/or RSRQ; or

- Serving cell quality measured by MR or LR is lower than the configured threshold, e.g. RSRP and/or RSRQ.

Proposal 7: The entry condition for R19 neighbour cell RRM measurement relaxation is the same as R19 serving cell RRM measurement relaxation.

Proposal 8: For measurement relaxation of neighboring cell, to avoid mobility risks, the ‘UE with low mobility’ criterion is necessary.

- References

- draft R2-2501332, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- R2-2410912, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- draft R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD.

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD.