3GPP TSG-RAN WG2 Meeting #129	R2-2502726

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.4

Source:	CMCC

Title:	Discussion on Common signalling adaptation

Document for:	Discussion

Introduction

In the RAN#102 meeting, the new WI of enhancements of network energy savings for NR was set up [1], one of the objectives is to adapt the common signal/channel transmission, which has been discussed during R18 SI and proved to be beneficial for energy saving, the related objective is as following,

3. Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

And following are the agreements related to Paging adaptation in the last two RAN2 meetings:

RAN2#128 meeting Agreements on paging adaptation [2]

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

RAN2#129 Agreements on paging adaptation [3]

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

In this contribution, we provide our view on paging adaptation related issues.

Discussion

The legacy PEI can indicate up to 4 POs per PF, while for Rel-19 paging adaptation, the maximum number of PO is eight per PF. To support paging adaptation, RAN2 agreed to introduce a separate PEI configuration in last RAN2 meeting. However, it is not clear how to comprehend the “separate PEI”. In our understanding, there’re two interpretations as shown in Figure 1:

Alt1: Legacy PEI is used to indicate the legacy POs, and the R19 separate PEI is used to indicate the addition  POs. A NES UE monitor legacy PEI or R19 PEI based on its PO number according to PO/PF calculation formula;

Alt2: The R19 separate PEI is associated with all the POs, which is monitored by NES UE only and legacy PEI is monitored by legacy UE only.

Figure 1 Interpretations of separate PEI

Observation 1: There’s two interpretations of “separate PEI”

Alt1. Legacy PEI is used to indicate the legacy POs, and the R19 separate PEI is used to indicate the addition  POs. A NES UE monitor legacy PEI or R19 PEI based on its PO number according to PO/PF calculation formula;

Alt2. The R19 separate PEI is associated with all the POs, which is monitored by NES UE only and legacy PEI is monitored by legacy UE.

One open issue about separate PEI identified in the running CR discussion [4] is whether to introduce a separate pei-ConfigBWP for paging adaptation. In our opinion, it’s necessary to introduce a separate pei-ConfigBWP for R19 separate PEI, in this way, the collision of legacy PEI and R19 separate PEI which may introduce confusion for the UE, can be avoided easily.

In the pei-ConfigBWP, firstPDCCH-MonitoringOccasionOfPEI-O is indicated, and this has impact on the signalling overhead of PEI configuration. From signalling overhead point, Alt1 brings less signalling overhead than Alt2, in which firstPDCCH-MonitoringOccasionOfPEI-O is only used for the additional POs than legacy POs. hence, we prefer to adopt Alt1.

Proposal 1: For separate PEI, RAN2 down-select from the following alternatives:

Alt1 (preferred): Legacy PEI is used to indicate the legacy POs, and the R19 separate PEI is used to indicate the addition  POs. A NES UE monitor legacy PEI or R19 PEI based on its PO number according to PO/PF calculation formula;

Alt2: The R19 separate PEI is associated with all the POs, which is monitored by NES UE only and legacy PEI is monitored by legacy UE.

Proposal 2: A separate pei-ConfigBWP is introduced for the separate PEI.

Besides, since paging adaptation can also configure with legacy paging parameters, i.e., 4 POs per PF, RAN2 should discuss whether the Rel-19 separate PEI is only configured when the new values for paging adaptation is configured by the network.

Proposal 3: RAN2 should discuss whether the Rel-19 separate PEI is only configured when the new values for paging adaptation is configured by the network.

Conclusions

In this contribution, we analyse the paging adaptation related issues, and the following proposals and observation are made:

Observation 1: There’s two interpretations of “separate PEI”

Alt1: Legacy PEI is used to indicate the legacy POs, and the R19 separate PEI is used to indicate the addition  POs. A NES UE monitor legacy PEI or R19 PEI based on its PO number according to PO/PF calculation formula;

Alt2:The R19 separate PEI is associated with all the POs, which is monitored by NES UE only and legacy PEI is monitored by legacy UE.

Proposal 1: For separate PEI, RAN2 down-select from the following alternatives:

Alt1 (preferred): Legacy PEI is used to indicate the legacy POs, and the R19 separate PEI is used to indicate the addition  POs. A NES UE monitor legacy PEI or R19 PEI based on its PO number according to PO/PF calculation formula;

Alt2: The R19 separate PEI is associated with all the POs, which is monitored by NES UE only and legacy PEI is monitored by legacy UE.

Proposal 2: A separate pei-ConfigBWP is introduced for the separate PEI.

Proposal 3: RAN2 should discuss whether the Rel-19 separate PEI is only configured when the new values for paging adaptation is configured by the network.

References

- RP-240170 Revised WID for Enhancements of network energy savings for NR; Ericsson;

- Draft RAN2 128 meeting report;

- Draft RAN2 129 meeting report;

- Running RRC CR NES