3GPP TSG-RAN WG2 Meeting #129bis        	                       R2-2502731

Wuhan, China, 7th-11th April 2025

Agenda item:	8.1.4

Source:	CMCC, China Telecom

Title:	Discussion on UE side data collection

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed UE side data collection and achieved the following agreements.

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In this contribution, we will focus on the discussion on solution 2 and 3.

Solution 2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

Solution 3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

- Discussion

## 2.1 General

Firstly, we want to clarify the sub-options of solution 2 and 3 to be further discussed, which is similar to the options of model transfer/delivery. That is, UP-based and CP-based solution should be discussed separately. Furthermore, the analysis of current status and gaps, and potential RAN specification impact for different solutions of model transfer/delivery can be reused.

Therefore, the solution 2 and 3 can be split as follows:

Solution 2a (CP-based): UE collects training data and transfers it to Core Network via CP-based solution. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

Solution 2b (UP-based): UE collects training data and transfers it to Core Network via UP-based solution. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

Solution 3a (CP-based): UE collects training data and transfers it to OAM via CP-based solution. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Solution 3b (UP-based): UE collects training data and transfers it to OAM via UP-based solution. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Proposal 1: For UE side data collection, solution 2 and 3 can be split to CP-based and UP-based solution separately for further discussion.

## 2.2 Discussion on data transfer over UP

In RAN#106 plenary, RANP sent a LS [2] to ask SA/SA2/SA5 to study the UP-based solution. And in legacy UP solution, the session is terminated at UE and UPF, so RAN is transparent for user plane data transmission. For the moment, we don’t see the impacts on RAN related to data transfer over UP. More details can be further discussed after we receive SA inputs on UP-based solution.

Proposal 2: It is proposed to postpone the discussion of UP-based solution before further inputs are received from SA.

## 2.3 Discussion on NG-RAN involvement

In last meeting [3], during the discussion on LCM for UE-sided model for beam management use case, RAN2 agreed the following agreements on data collection for UE-side model training:

For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. The network can provide the data collection configuration (at any point in time), with or without UE request.

Regardless of UE request, the network can provide the data collection configuration for UE-sided model training. In our understanding, the network here refers to NG-RAN, since CSI feedback and beam management are PHY use cases and it is hard for CN to provide the configuration or understand the physical parameters. That is, NG-RAN is involved for data collection configuration for model training.

Observation 1: NG-RAN is involved to provide the data collection configuration for UE-sided model training.

For solution 2, since the data is terminated at CN, the data collection configuration can be initiated by CN. However, as we mentioned above, we don’t think CN can initiate the L1 data collection and send the configuration of physical use cases to RAN.

For solution 3, as legacy MDT, OAM can initiate the data collection procedure and send the configuration to RAN, and then RAN provide the configuration to the UE.

Therefore, we think solution 3a (CP-based) is preferred.

Proposal 3: Solution 3a (CP-based) is preferred since OAM can initiate the data collection procedure and send the configuration to RAN.

## 2.4 Discussion on scalability of CP

During the study phase, RAN1 reply LS [4] indicated that the typical data size per use case is provided for easy understanding as follows, and it is noted that the data size is per data sample:

- For CSI compression: The data size is about 10s of bits to ~ a few 1000bits even up to ~150Kbits (18.75kbyte) per data sample for different types of data content. RAN1 also notes that the data size is very depending on the configuration.

- For CSI prediction: Data size depends on the format of Target CSI, and RAN1 has no agreements on this. One example is given that the data size is up to around 1.5Mbits (equal to 192kbyte), assuming float 32 and 10 CSI-RS observation instances as input to predict one future CSI instance. However, during the RAN1 discussion, some companies also think 5 CSI-RS observation instances can be used as input to predict one future CSI instance, where the data size would be up to ~900Kbits (112kbyte).

- For beam management: RAN1 provides examples of the typical data size for BM Case 1, it would be up to hundreds of bits (e.g. ~500bits if Set A = 128, 7bits for the strongest beam and 4 bits for the remaining beams).

- For positioning: The data size depends on number of PRS/SRS resources, and the existing spec allows reporting up to 64 PRS/SRS resource per frequency layer. The typical data size per PRS/SRS resource may be 10s bits to 100bits for different data content, for 64 PRS/SRS, the typical data size would be 640bits to 6400bits.

Observation 2: The typical data size for model training per data sample would be about 10s of bits to a few 1000bits even ~150Kbits for CSI compression, beam management and positioning.

Observation 3: The typical data size for model training per data sample would be up to around 1.5Mbits (maybe ~hundreds of Kbits) for CSI prediction.

Among the existing framework, the max payload size per RRC signaling is up to 9kbyte, and max UL payload size is up to 144kByte considering RRC segmentation (i.e. up to 16 segmentation for UL). And for L1 measurement, the payload size is <1706bit in PUCCH and <3840bit in PUSCH. At first glance, the existing framework can support the data collection for model training per data sample except CSI prediction.

Observation 4: The existing framework (max payload size <9kbyte for RRC signaling) can support the data size requirements per data sample for model training for CSI compression, BM and positioning.

In our understanding, for CSI prediction or multiple samples for each use case, the requirements on data size can be satisfied if the number of RRC segmentation is extended.

On the other hand, RAN1 expects that NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling (i.e. over-the-air or other approaches) in LS [5]. If the number of RRC segmentation is extended, the RRC signaling can also be used for dataset or model parameters sharing.

Observation 5: If the number of RRC segmentation is extended, the data size requirements on CSI prediction or multiple samples for each use case will be satisfied, and the extended RRC signaling can also be used for dataset or model parameters sharing.

During the discussion on NW side data collection in last meeting, RAN2 agreed to follow the same principles as for logged MDT, i.e. the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. We think this option can also be effective for UE side data collection.

Observation 6: For NW side data collection, the same principles as logged MDT are reused, i.e. an available indication is included to indicate there are logged measurements available and multiple RRC message may be needed to report the collected data.

Based on the above analysis, we propose to discuss the following solutions to support the data collection for model training, dataset and model parameters sharing.

Proposal 4: RAN2 to discuss the following solutions to support the data collection for model training:

- Option 1: Extend the number of RRC segmentation, which also can be used for model parameters and dataset sharing

- Option 2: Reuse the mechanism of logged MDT, i.e. an available indication is included to indicate there are data available and multiple RRC message may be needed to transfer the data

# 3	Conclusion

Here are the observations and proposals for UE side data collection.

Observations

Observation 1: NG-RAN is involved to provide the data collection configuration for UE-sided model training.

Observation 2: The typical data size for model training per data sample would be about 10s of bits to a few 1000bits even ~150Kbits for CSI compression, beam management and positioning.

Observation 3: The typical data size for model training per data sample would be up to around 1.5Mbits (maybe ~hundreds of Kbits) for CSI prediction.

Observation 4: The existing framework (max payload size <9kbyte for RRC signaling) can support the data size requirements per data sample for model training for CSI compression, BM and positioning.

Observation 5: If the number of RRC segmentation is extended, the data size requirements on CSI prediction or multiple samples for each use case will be satisfied, and the extended RRC signaling can also be used for dataset or model parameters sharing.

Observation 6: For NW side data collection, the same principles as logged MDT are reused, i.e. an available indication is included to indicate there are logged measurements available and multiple RRC message may be needed to report the collected data.

Proposals

Proposal 1: For UE side data collection, solution 2 and 3 can be split to CP-based and UP-based solution separately for further discussion.

Proposal 2: It is proposed to postpone the discussion of UP-based solution before further inputs are received from SA.

Proposal 3: Solution 3a (CP-based) is preferred since OAM can initiate the data collection procedure and send the configuration to RAN.

Proposal 4: RAN2 to discuss the following solutions to support the data collection for model training, dataset and model parameters sharing:

- Option 1: Extend the number of RRC segmentation, which also can be used for model parameters and dataset sharing

- Option 2: Reuse the mechanism of logged MDT, i.e. an available indication is included to indicate there are data available and multiple RRC message may be needed to transfer the data

# 4	Reference

- RP-243307 Way forward on AIML UE sided data collection

- RP-243316 LS on AI/ML UE sided data collection

- Draft_RAN2_129_Meeting_Report_v3

- R1-2310681 Reply LS on Data Collection Requirements and Assumptions

- R1-2410922 LS on signalling feasibility of dataset and parameter sharing