# 3GPP TSG-WG2 Meeting #129bis                                            R2-2502734

Wuhan, China, 7th – 11th April 2025

Source: 			CMCC, ZTE, MediaTek, vivo, Huawei, CATT, Meta, Nokia, Nokia Shanghai Bell, xiaomi, 				Spreadtrum, UNISOC

Title:  		    Discussion on UE aggregation enhancement

Agenda item:	8.18

Document for: 	Discussion

Introduction

R17 SL relay is mainly focused on single path, where remote UE is connected to network via SL path. R18 SL relay enhancement extends the scenario to multi-path, one direct path (Uu link) and one indirect path (SL or Non-3GPP link). The direct path and indirect path are limited to the same gNB.

For TEI 19, we suggest to consider some enhancements on multi-path to meet the service requirements.

Discussion

## 2.1 The current specifications on Multi-Path Relay

3GPP RAN approved a work item “NR SL relay enhancements”[1] in R18 to address the enhancements and solutions required for supporting the UE-to-network Relay. The inclusion of multi-path with relay in R18, where a remote UE is connected to network via direct and indirect paths, holds the potential to enhance the reliability, robustness and throughput. This multi-path relay solutions are almost be utilized to for UE aggregation, a scenario where a UE is connected to the network via direct path and via another UE using a non-3GPP connection. UE aggregation is designed to cater applications requiring high UL bit rates on 5G terminals, in cases when normal UEs are too limited by UL UE transmission power to achieve required bit rates, especially at the edge of a cell. Moreover, UE aggregation can improve the reliability, stability and reduce delay of services as well, that is, if the channel condition of a terminal is deteriorating, another terminal can be used to make up for the traffic performance unsteadiness caused by channel condition variation.

However, due to time constraints, R18 only supports one direct path and one indirect path (PC5 or N3C) for UE aggregation, which may limit the full potential of multi-path relay solutions in enhancing network performance and service quality. The system gain from a single indirect path in UE aggregation is also constrained by the uplink transmission capability of the relay UE, as well as by mobility and radio link conditions.

Fig 1. The current multi-path relay

Observation 1: In current RAN2 specifications on multi-path relay, only a single indirect path (PC5 or N3C) for UE aggregation is supported.

## 2.2 The motivation on supporting multiple indirect paths

In application scenarios that demand UE aggregation, such as large-scale live broadcasting, there is a significant need for high uplink transmission rates and reliability. This makes it particularly important to increase the number of assisting relay UEs. For instance, in scenarios requiring ultra-long-range coverage, such as maritime fishing operations or offshore oil extraction platforms, terminals would ideally need to emit a transmission power of approximately 33dBm to cover the vast distances. To achieve this with typical terminals that have a transmission power of 23dBm, it would be necessary to aggregate signals from ten such terminals.

- Example of large-scale live broadcasting    2) Example of ultra-long-range coverage

- Fig 2. The example scenarios for UE aggregation (which are generated by AI)

This aggregation is crucial for meeting the high demands of uplink bitrates and ensuring the reliability of services, especially in environments where the transmission power of individual UEs may not be sufficient to achieve the required performance. By increasing the number of assisting relay UEs, the system can better accommodate the needs of high-data-rate applications and improve the overall robustness of the network.

Observation 2: One indirect path in UE aggregation cannot meet the requirements of some use cases, e.g. large-scale live broadcasting, maritime fishing operations or offshore oil extraction platforms.

Fig 3. The example for multiple indirect paths via N3C

Based on observation 1 and observation 2, we suggest to support multiple indirect paths for multi-path relay. Considering there may be more potential spec impacts for PC5 than N3C, and the remaining time is limited in Rel-19, we prefer to only support multi-path relay via N3C in TEI19.

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

In current specs, MP Remote UE and MP Relay UE are in the intra-gNB, and PCell is always on the direct path. Considering to avoid the potential impacts on other WGs (e.g. RAN3) and minimum the workload, we prefer that the multiple indirect paths are under the same gNB, and CU-DU split structure is not considered in TEI19.

Proposal 2：It is proposed to only consider the scenario where multiple indirect paths under the same gNB, and CU-DU split structure is not considered in TEI19.

## 2.3 The potential spec impacts for multiple indirect path

To support the multiple indirect paths via N3C, we think the following issues should be discussed, and the impacts on TS 38.300 [2], TS 38.331 [3] and TS 38.306 [4] may be considered. Moreover, we provide the analysis on these issues in the subsequent sections.

- Scenario

- Multi-path configuration, including split bearer and PDCP duplication

- Path failure

- UE capability

### 2.3.1 Description of scenario

- As we proposed in P1 and P2, the description of scenario should be captured in TS 38.300 [2], i.e. only consider the scenario where multiple indirect paths via N3C under the same gNB.

- In current spec, the UE can report up to 8 relay UEs’ information to the the network using UEAssistanceInformation message. To minimum the spec impacts, we prefer to reuse the current maximum number for N3C relay UE, i.e. 8, for multiple indirect paths via N3C. That is, the network can configure at most 8 N3C relay UEs which are reported by remote UE in UAI.

n3c-RelayUE-InfoList-r18 SEQUENCE (SIZE (0..8)) OF N3C-RelayUE-Info-r18

- Proposal 3: It is proposed to reuse the maximum number (i.e. 8) for multiple indirect paths via N3C.

### 2.3.2 Multi-Path configuration

Addition and Change of multiple indirect paths

- In the current mechanism, the IE N3C-IndirectPathAddChange in RRCReconfiguration message [3] indicates the N3C indirect path related configuration used by N3C remote UE in MP, and only one relay UE info is included.

- To support multiple indirect paths via N3C links, multiple configurations of relay UEs should be provided or released to the remote UE explicitly. In addition, the enhancements on the current IE n3c-IndirectPathAddChange should be avoid since it will introduce the impacts on legacy UEs. Therefore, the new IE n3c-ExtIndirectPathAddChangecan be introduced to provide or release the configuration of N3C relay UEs.

- Proposal 4: Introduce the new IE n3c-ExtIndirectPathAddChange to provide or release the configuration of N3C relay UEs.

Split Bearer

- As specified in TS 38.323 [5], the ul-DataSplitThreshold IE in PDCP-config [3] is used to decide which RLC entity or which path the PDCP PDU should be submitted to, when PDCP duplication is deactivated for the RB. If the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold, the PDCP PDU is submitted to either the Uu RLC entity or N3C, which is up to UE implementation.

- If the multiple indirect paths via N3C are supported, the legacy mechanism can be reused to avoid the additional spec impacts.

Observation 3: The legacy mechanism for data split for multiple indirect paths via N3C can be reused, and no spec impact is needed, i.e. it is up to UE implementation to select a path for the data split when the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold.

PDCP Duplication

- For PDCP duplication, as specified in TS 38.300 [2], if the split bearer is configured and the PDCP PDU duplication is activated on the PDCP entity, the duplicated PDCP PDUs are delivered via both direct path and indirect path.

- As specified in TS 38.331 [3], duplicationState IE indicates the uplink PDCP duplication state. And for multi-path, the index for the indication is determined by ascending order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path. That means the third bit of duplicationState IE indicates indirect path.

- Correspondingly, the Duplication RLC Activation/Deactivation MAC CE in TS 38.321 [6] is used to indicate the duplication RLC activation/deactivation. And for Multi-path, RLC entity i is counted for secondary RLC entities in order of direct path (where i is ascending order of logical channel ID of secondary RLC entities) and indirect path.

- For multiple indirect paths via N3C, to minimum the impacts on specifications, we prefer to up to UE implementation in this release.

- Proposal 5: For PDCP duplication, it can be up to UE implementation to decide which MP Relay UE is used when more than one MP Relay UEs are configured in N3C interface, without impacts on PDCP spec and MAC spec.

### 2.3.3 Path Failure

- As specified in TS 38.300 [2], when the L2 MP Remote UE using N3C indirect path detects N3C link failure and/or Uu link failure on the indirect path, the L2 MP Remote UE triggers path failure reporting through the direct path via a RRC message, if the direct path is not suspended. And for failure indirect path reporting, since there is only one indirect path in R18, the remote UE can report the available relay UE(s) info to the network, without indicating the indirect path failure.

- For multiple indirect paths via N3C, if the failure is detected in at least one indirect N3C path, the UE needs to report the failure info and indicates explicitly which path fails to the network. Therefore, a new IE should be introduced to indicate the failure info of one or more indirect N3C paths within FailureReportIndirectPath.

Proposal 6: A new IE should be introduced to indicate the failure info of one or more indirect N3C paths within FailureReportIndirectPath.

### 2.3.4 UE capability

- In TS 38.306 [4], the UE capability multipathRemoteUE-N3C-r18 is specified which indicates whether L2 multi-path remote UE operation using non-3GPP connection is supported by the UE.

To support the multiple indirect paths via N3C, we think a new capability should be introduced to differentiate with the legacy remote UE capability on N3C.

Proposal 7: A new remote UE capability to support multiple indirect paths via N3C should be introduced.

Based on the above proposals, we have prepared the following CRs to support multiple indirect paths via N3C:

- Draft CR to 38.300 [7]

- Draft CR to 38.331 [8]

- Draft CR to 38.306 [9]

Conclusion

According to the above discussion, the following observations and proposals are given:

Observations

Observation 1: In current RAN2 specifications on multi-path relay, only a single indirect path (PC5 or N3C) for UE aggregation is supported.

Observation 2: One indirect path in UE aggregation cannot meet the requirements of some use cases, e.g. large-scale live broadcasting, maritime fishing operations or offshore oil extraction platforms.

Observation 3: The legacy mechanism for data split for multiple indirect paths via N3C can be reused, and no spec impact is needed, i.e. it is up to UE implementation to select a path for the data split when the total amount of PDCP data volume is equal to or larger than ul-DataSplitThreshold.

Proposals

Proposal 1: RAN2 is suggested to support multiple indirect paths for multi-path relay via N3C.

Proposal 2：It is proposed to only consider the scenario where multiple indirect paths under the same gNB, and CU-DU split structure is not considered in TEI19.

- Proposal 3: It is proposed to reuse the maximum number (i.e. 8) for multiple indirect paths via N3C.

- Proposal 4: Introduce the new IE n3c-ExtIndirectPathAddChange to provide or release the configuration of N3C relay UEs.

- Proposal 5: For PDCP duplication, it can be up to UE implementation to decide which MP Relay UE is used when more than one MP Relay UEs are configured in N3C interface, without impacts on PDCP spec and MAC spec.

Proposal 6: A new IE should be introduced to indicate the failure info of one or more indirect N3C paths within FailureReportIndirectPath.

Proposal 7: A new remote UE capability to support multiple indirect paths via N3C should be introduced.

Reference

- RP-223501 Revised WID on NR sidelink relay enhancements

- TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TS 38.306 NR; User Equipment (UE) radio access capabilities

- TS 38.323 NR; Packet Data Convergence Protocol (PDCP) specification

- TS 38.321 NR; Medium Access Control (MAC) protocol specification

- R2-2502735	Corrections to TS 38.300 on multi-path relay enhancement

- R2-2502736	Corrections to TS 38.331 on multi-path relay enhancement

- R2-2502737	Corrections to TS 38.306 on multi-path relay enhancement