3GPP TSG-RAN WG2 Meeting #129bis                                 R2-2502749

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item	: 8.2.3 (Ambient_IoT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Contention resolution failure

As agreed in RAN2#126 meeting, the A-IOT device considers the contention resolution as successful when the Msg2 containing the same random number transmitted in Msg1 is received. In other words, if the A-IOT device receives the Msg2 including a random number different from the one transmitted in Msg1, the A-IOT device considers the contention resolution as failed.

In our view, as in NR, if the A-IOT device does not receive the Msg2 after transmitting the Msg1 for a certain duration, it should be considered as contention resolution failure as well. However, how to determine whether the A-IOT device does not receive the Msg2 after transmitting the Msg1 is not discussed.

In NR, the RAR is used to monitor the PDCCH for Msg2 reception. The purpose of the RAR window is to re-initiate the random access procedure if the Msg2 is not received while running the RAR. In addition, in TR 38.769 v19.0.0, the timing relationships is already captured as follows.

6.1.3 Timing relationships A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions. TR2D_R2D_min: Minimum time between two different consecutive R2D transmissions to the same A-IoT device. TD2R_D2R_min: Minimum time between two different consecutive D2R transmissions from the same A-IoT device. For the time interval between a R2D transmission and the corresponding D2R transmission following it, there are two options studied: Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D ≥ TR2D_min.

As shown above, TD2R_MAX indicates the expected maximum time to get the Msg2. It means that it is expected that the reader transmits the Msg2 within TD2R_MAX. Thus, similar to NR, a new timer should be introduced to determine the contention resolution failure. More specifically, after transmission of the Msg1, the A-IOT device starts a new timer for Msg2 reception. If the A-IOT device does not receive the Msg2 before the new timer for Msg2 reception expires, the A-IOT device considers the contention resolution as failed.

Proposal 1. A new timer for Msg2 reception is introduced to determine the contention resolution failure.

In RAN2#128 meeting, it was agreed that “RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID”. In addition, in TR 38.769 v19.0.0, how to request the Msg3 is captured as follows.

The handling of data transmission failure has been studied. It is understood that the subsequent R2D data transmission after the D2R data transmission does not need to be always sent. In case of D2R data transmission failure, the A-IoT device follows the reader subsequent R2D instruction, if any. - For instance, the reader can repeat the R2D upper layer "command" to trigger the A-IoT device to re-send the same D2R upper layer "response" (i.e., the A-IoT device just follows the received R2D to transmit following D2R). - For instance, the reader can re-send A-IoT Msg2 to specific device(s) to echo the random ID(s), in case of failure reception of corresponding D2R data transmission (i.e. "Msg3") after the initial A-IoT Msg2, together with the corresponding D2R resource scheduling. This can trigger the A-IoT device to re-send the same D2R data transmision (i.e. "Msg3"). If it is supported to include the echoed random IDs for multiple devices, the re-sent A-IoT Msg2 only includes the random IDs of the devices, whose "Msg3" is not successfully received.

Considering the above, the duration of the new timer for Msg2 reception should be sufficiently long because it should take into account the time for re-sending Msg2 multiple times.

Observation 1. The duration of the new timer for Msg2 reception should be sufficiently long because it should take into account the time for re-sending Msg2 multiple times.

Based on observation 1, since the duration of the new timer for Msg2 reception should be sufficiently long, there may be a case where the A-IOT device receives a new Msg1 resource indication before the new timer for Msg2 reception expires. This is because from the reader point of view, if the reader does not receive any Msg1 after transmitting the Msg1 resource indication, the reader considers that all A-IOT devices do not transmit the Msg1. In this case, the reader can transmit the next Msg1 resource indication.

Given the above, the A-IOT device can receive the Msg2 with the same random number transmitted in Msg1 after the reception of the Msg1 resource indication. For example, the A-IOT device#1 transmits the Msg1 containing the random number = 10, but the Msg1 is not transmitted successfully. After that, A-IOT device#1 and #2 receive the Msg1 resource indication. The A-IOT device#2 transmits the Msg1 containing the random number = 10. Then, A-IOT device#1 and #2 receive the Msg2 containing random number = 10. In this case, both A-IOT device#1 and #2 consider the contention resolution as successful.

Thus, in order to prevent the above case, the A-IOT device considers the contention resolution as failed and stops the new timer for Msg2 reception if the Msg1 resource indication is received while running the new timer for Msg2 reception. The following figure shows an example of this case.

Proposal 2. If the Msg1 resource indication is received while the new timer for Msg2 reception is running, the A-IOT device considers the contention resolution failed and stops the new timer for Msg2 reception.

  - NACK based indication

In the last meeting, the following agreement was made.

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback

As shown above, the NACK feedback is used after the contention resolution is completed, and the NACK feedback is used to indicate the re-access purpose. It means that the reader does not need to transmit the NACK feedback when the Msg3 is received. Thus, the absence of the NACK feedback is considered as a successful transmission of the Msg3.

Proposal 3. The absence of the NACK feedback is considered as successful transmission of the Msg3.

If the proposal 3 is agreeable, how to determine the absence of the NACK feedback should be discussed. As explained above, the TD2R_max is defined in the TR 38.769 v19.0.0. So, the NACK feedback should be indicated within the TD2R_max after transmitting the Msg3. Thus, we think that similar to the new timer for Msg2 reception, a new timer for NACK feedback reception should be introduced. For example, after completion of contention resolution, the A-IOT device transmits the Msg3. Then, the A-IOT device starts the new timer for NACK feedback reception. Then, if the new timer for NACK feedback reception expires without NACK feedback reception, the A-IOT device considers the Msg3 transmission as successful.

Proposal 4. Introduce a new timer for NACK feedback reception to determine whether the Msg3 is successfully transmitted or not.

Similar to the new timer for Msg2 reception, there is a case where the A-IOT device can receive the new Msg1 resource indication while running the new timer for NACK feedback reception. From the reader perspective, the reader transmits the Msg1 resource indication after successfully receiving the Msg3. Otherwise, the reader may transmit the NACK feedback if the Msg3 is not received. Therefore, if the A-IOT device receives the Msg1 resource indication while running the new timer for NACK feedback reception, the A-IOT device considers the Msg3 transmission successful and stops the new timer for NACK feedback reception.

Proposal 5. If the Msg1 resource indication is received while the new timer for NACK feedback reception is running, the A-IOT device considers the Msg3 transmission successful and stops the new timer for NACK feedback reception.

Next, RAN2 need to discuss whether the NACK based mechanism is used for the Msg5, i.e., D2R message for command response.

In order to handle the reception failure for the Msg5, we think that there are three options as follows.

- Option 1. The reader re-sends the Msg4, i.e., R2D command, to get the Msg5.

- Option 2. The reader transmits the dedicate A-IOT paging message to the A-IOT device after finishing the A-IOT paging round.

- Option 3. The reader indicates the NACK feedback to the A-IOT device for indicating the re-access.

In our view, at least Option 1 and Option 2 are feasible. However, the need for Option 3 is questionable. If the Msg3 transmission is received for the A-IOT device, the reader can know the A-IOT device’s identifier. In this case, there is no reason to indicate the re-access procedure to the A-IOT device. Since the re-access procedure means the reader indicates performing the contention-based random access procedure to the A-IOT device in the subsequent A-IOT paging round, Option 3 increases the collision ratio in the subsequent A-IOT paging round. Thus, considering that the high collision ratio causes a waste of radio resources, Option 3 is not needed for Msg5.

Proposal 6. NACK based mechanism is applied only to the Msg3.

Then, RAN2 need to discuss the NACK feedback format.

In the last meeting, the message type and contents for MAC PDU format were discussed, and the following agreements were made.

- Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

In our view, the NACK feedback should not be piggybacked in the Msg2 because the NACK feedback indicates the re-access procedure by terminating the Msg3 transmission in the current A-IOT paging round. Thus, since the NACK feedback is always used standalone, a new R2D MAC PDU format for NACK feedback should be defined.

Proposal 7. A new R2D MAC PDU format for NACK feedback should be defined.

Considering the FDMA, there might be a case where the reader indicates the NACK feedback to the multiple A-IOT devices because the multiple A-IOT devices can attempt the random access procedure in different frequencies. In our view, as proposed in R2-2501788, the frequency index may be contained in the A-IOT paging message. Thus, in the R2D MAC PDU for NACK feedback, only frequency indexes are needed to indicate which A-IOT device needs to perform the re-access procedure.

The following is an example of the NACK feedback format. For example, if the F0 field is set to 1, the A-IOT device that uses the frequency#0 performs the re-access procedure.

Example of NACK format

Proposal 8. In the R2D MAC PDU for NACK feedback, only frequency information is contained.

  - Msg2 format

In WID, it is stated that the multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. It means that since there is only one R2D frequency, the A-IOT device needs to receive the list of frequency information for D2R via the A-IOT paging message as proposed in R2-2501788. Based on this, the following scenario can be considered.

For example, after selecting the access occasion and frequency, the A-IOT device transmits the Msg1 at the selected frequency. Then, the A-IOT device should receive the Msg2 containing the random number transmitted on Msg1 to complete the contention resolution. However, from the reader point of view, the reader should transmit the multiple random numbers to complete the contention resolution for multiple A-IOT devices if the FDMA is supported.

In the last meeting, it was agreed that the multiplexing of information for multiple devices in R2D message for Msg2 is supported. In other words, in the Msg2, multiple random numbers can be included. However, if only random numbers are included in the Msg2, it is hard to identify which random number is associated with which frequency. Therefore, considering the FDMA operation, the information is required to indicate the association between the random number and the frequency. For this, the frequency index should be included in Msg2, and each frequency index is mapped with each random number. The following figure shows an example for Msg2 format. For example, if the F0 is set to 1, the F0 indicates the presence of the random number field for frequency#0.

Example of Msg2 format

Proposal 9. In Msg2, the frequency index should be included, and each frequency index is mapped with each random number.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. A new timer for Msg2 reception is introduced to determine the contention resolution failure.

Observation 1. The duration of the new timer for Msg2 reception should be sufficiently long because it should take into account the time for re-sending Msg2 multiple times.

Proposal 2. If the Msg1 resource indication is received while the new timer for Msg2 reception is running, the A-IOT device considers the contention resolution failed and stops the new timer for Msg2 reception.

Proposal 3. The absence of the NACK feedback is considered as successful transmission of the Msg3.

Proposal 4. Introduce a new timer for NACK feedback reception to determine whether the Msg3 is successfully transmitted or not.

Proposal 5. If the Msg1 resource indication is received while the new timer for NACK feedback reception is running, the A-IOT device considers the Msg3 transmission successful and stops the new timer for NACK feedback reception.

Proposal 6. NACK based mechanism is applied only to the Msg3.

Proposal 7. A new R2D MAC PDU format for NACK feedback should be defined.

Proposal 8. In the R2D MAC PDU for NACK feedback, only frequency information is contained.

Proposal 9. In Msg2, the frequency index should be included, and each frequency index is mapped with each random number.