3GPP TSG-RAN WG2 Meeting #129bis	R2-2502751

Wuhan, China, 7 – 11 April, 2025

Agenda item:	8.7.4.2

Source:	Samsung

Title:	Remaining Issues on DSR enhancements in Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2 meeting, RAN2 made the following agreements for DSR enhancements:

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

In this paper, we further discuss the essential aspects on the remaining issues of enhanced DSR.

Discussion

Configuration Constraint on Reporting Threshold

We agreed in RAN2#128:

Let the network configure the triggering and reporting thresholds without constraints.

while, we also agreed in the last meeting:

Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold)”,

that we have to consider the case that, if triggering threshold is configured larger than all the reporting thresholds for an LCG, a DSR can be triggered without data satisfying any reporting threshold(s). In this case, how the enhanced DSR MAC CE should be composed, as well as, its value, are unclear. Put differently, whether, in such a case, a DSR should be triggered is doubtable.

Observation 1: For an LCG, if triggering threshold is configured larger than all the reporting thresholds,  undesirably, enhanced DSR could be triggered without any data to report.

The simplest way to handle this issue, is imposing a constraint on the threshold configuration for enhanced DSR, e.g., at least one reporting threshold should be configured larger than or equal to the triggering threshold, or, the triggering threshold should be configured less than or equal to the largest reporting threshold, for an LCG. While since we also agreed not to introduce configuration constraint, a note could be introduced to remind that such undesirable DSR triggering should be prevented by proper network configuration.

Proposal 1. RAN2 to discuss how to prevent enhanced DSR from being triggered without any data to report, by considering following alternatives:

- Alt. 1: introduce a constraint that, for an LCG, at least one reporting threshold should be configured larger than or equal to triggering threshold;

- Alt. 2: introduce a constraint that, for an LCG, triggering threshold should be configured less than or equal to the largest reporting threshold; (same meaning as Alt. 1 with different expression);

- Alt. 3: introduce a note to remind such undesirable DSR triggering should be prevented by proper network configuration.

DSR Format Selection

In the last meeting, when to use enhanced DSR is left as FFS:

FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds.

Since it turns out that, during the MAC running CR review, no objection is noticed for the above rule of format selection, it is better to confirm and agree it officially.

Proposal 2. RAN2 to confirm that enhanced DSR MAC CE is used if at least one LCG is configured with dsr-ReportingThresList.

New DSR MAC CE Name

During the running CR review, the name of the new DSR MAC CE is not aligned, resulting in mainly two options: 1) Multiple Entry DSR MAC CE, 2) Enhanced DSR MAC CE. For the first option, it also entails the modification of the legacy DSR MAC CE to Single Entry DSR MAC CE. However, Single Entry here is misleading, since it can still report delay information for multiple LCGs. Also, here, the functionality and format changes are not that significant compared to that of single entry PHR and multiple entry PHR. Given that it is of further enhancement based on the legacy DSR MAC CE, Enhanced DSR MAC CE is more appropriate as the name of the new DSR MAC CE.

Proposal 3. RAN2 to agree on the name of the new DSR MAC CE as Enhanced DSR MAC CE.

LCGs Reported by Enhanced DSR

Given that the enhanced DSR is used when at least one LCG is configured with reporting threshold(s), one point needs to be clarified is, whether the LCG(s) not configured with reporting threshold(s), but with pending DSR, should be reported inclusively in the enhanced DSR or not. We think it is not clear, considering, in the latest version of running MAC CR:

- “Buffer Size: … In the Multiple Entry DSR MAC CE, the field Buffer Size i,j indicates the total amount of delay-reporting data associated with the reporting threshold j of LCG i, …” and “Remaining Time: … In the Multiple Entry DSR MAC CE, the field Remaining Time i,j indicates the shortest remaining time among the PDCP SDUs associated with the reporting threshold j of LCG i, …”, no implication for the LCG(s) not configured with reporting threshold is captured for enhanced DSR, while,

- “A DSR MAC CE shall include delay status information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built.” is inclusively embracing the LCG(s) not configured with reporting threshold,

leading to contradictory hypotheses.

Given that “a MAC PDU shall contain at most one DSR MAC CE”, we should confirm that, the LCG(s) not configured with reporting threshold, but with pending DSR, are inclusively served by enhanced DSR MAC CE, and this point should be reflected in field descriptions.

Proposal 4. RAN2 to confirm that, enhanced DSR MAC CE inclusively serves the LCG(s) not configured with reporting threshold, but with pending DSR, and its field descriptions should reflect it.

Conclusion

Based on the discussion above, we have the following observation and proposals:

Proposal 1. RAN2 to discuss how to prevent enhanced DSR from being triggered without any data to report, by considering following alternatives:

- Alt. 1: introduce a constraint that, for an LCG, at least one reporting threshold should be configured larger than or equal to triggering threshold;

- Alt. 2: introduce a constraint that, for an LCG, triggering threshold should be configured less than or equal to the largest reporting threshold; (same meaning as Alt. 1 with different expression);

- Alt. 3: introduce a note to remind such undesirable DSR triggering should be prevented by proper network configuration.

Proposal 2. RAN2 to confirm that enhanced DSR MAC CE is used if at least one LCG is configured with dsr-ReportingThresList.

Proposal 3. RAN2 to agree on the name of the new DSR MAC CE as Enhanced DSR MAC CE.

Proposal 4. RAN2 to confirm that, enhanced DSR MAC CE inclusively serves the LCG(s) not configured with reporting threshold, but with pending DSR, and its field descriptions should reflect it.