3GPP TSG RAN WG2 Meeting #129bis	 R2-2502756

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.6.4

Source: 	ITL

Title: 	Discussion on Conditional intra-CU LTM

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to conditional intra-CU LTM are as follows:

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this contribution, we discussed the conditional LTM for Intra-CU LTM.

Discussion

For Conditional LTM, the network can configure conditions for candidate cell(s). If the UE determines that any of these conditions are met, it initiates an LTM cell switch to the corresponding candidate cell.

In Conditional LTM, since the UE triggers an LTM Cell switch without an LTM Cell switch command MAC CE, the UE needs to receive or derive the information that would typically be included in the LTM Cell switch command MAC CE before performing the C-LTM.

Therefore, we consider it a key point of discussion to determine what information the UE needs to derive or receive, as well as how it should do so.

Figure 1. LTM Cell switch command MAC CE

In Figure 1, the LTM Cell Switch Command MAC CE contains information that enables the UE to perform either RACH-less or RACH-based LTM:

- Target Config ID: Indicates the candidate cell ID.

- Timing Advance Command: Specifies whether the timing advance (TA) is valid. Based on this, the UE determines whether to perform RACH-based or RACH-less LTM.

- TCI State ID: Indicates and activates the TCI state for the LTM target cell.

- Random Access Related Information (preamble index, SSB index, PRACH mask index, C, S/U, repetition number): Provides RACH-related information for CBRA and CFRA procedures.

On the below, we show our views on some aspects of the information that should be used and how it should be received by the UE to support both RACH-less and RACH-based Conditional LTM.

Timing advance command (TAC)

In R18 LTM, if the Timing Advance Command (TAC) indicated in the LTM cell switch command MAC CE is valid (i.e., not FFF), the UE can perform RACH-less LTM. Therefore, we think that the UE should obtain TAC information for candidate cell(s) to determine whether to proceed with RACH-less or RACH-based conditional LTM.

In RAN2#128, it was agreed that a new MAC CE is introduced to indicate TA value of candidate cell(s).

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

If the UE has a valid TA value for the C-LTM candidate cell(s), it can perform RACH-less C-LTM to the target cell when the L1 event conditions are met.

We think that at least following contents can be considered for new MAC CE.

- C-LTM candidate cell ID(s)

- TA value(s)

We think that the C-LTM candidate cell ID can either be the same as the existing LTM candidate cell ID (LTM-CandidateId) or serve as a separate identifier. In other words, if LTM and C-LTM coexist within the LTM configuration, the existing LTM candidate cell ID can be used. Conversely, if they do not coexist, a new identifier will be used.

Proposal 1: A new MAC CE for the TA value of C-LTM candidate cells includes the following information:

- C-LTM candidate cell ID(s)

- TA value(s)

Conditional LTM failure recovery

In TS 38.300, for the Handover failure recovery, methods for not performing the RRC re-establishment procedure in certain cases are defined as follows:

- When DAPS handover fails, the UE falls back to the source cell configuration, resumes the connection with the source cell, and reports DAPS handover failure via the source without triggering RRC connection re-establishment if the source link has not been released.

- When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

- When LTM execution attempt triggered by LTM cell switch command MAC CE fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed.

In other words, after an LTM failure, if attemptLTM-Switch is configured and the selected cell is one of the LTM candidate cells, the UE can attempt the LTM cell switch again. And if attemptLTM-Switch is not configured and the selected cell is not one of the LTM candidate cells, the UE perform the RRC re-establishment procedure.

For the C-LTM failure case, if the UE selects an Intra-CU cell, we think that the above scheme for attempting LTM can be reused. Additionally, if C-LTM is configured for the selected Intra-CU cell and the TA is valid, we think that C-LTM attempt can also be performed.

In RAN2#129, RAN2 agreed to support C-LTM fast recovery as follow:

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

According to TS 38.300, the UE may skip the RRC re-establishment procedure after HO or LTM failure under specific conditions. In Release 18, LTM failure recovery was limited to RACH-based LTM reattempts. In contrast, C-LTM in Release 19 introduces early TA for candidate cells, which can enable RACH-less C-LTM. Due to TA knowledge available in C-LTM candidate cells, RACH-less C-LTM can also be supported, improving recovery time and reducing signaling overhead.

Furthermore, it is our understanding that the C-LTM candidate cell where the C-LTM was initially triggered and the cell selected by the UE after the C-LTM failure may be different. This implies that the TA value used in the newly selected cell could differ from the TA associated with the originally attempted C-LTM candidate cell.

Proposal 2: RAN2 is kindly asked to discuss introducing RACH-less based C-LTM attempt after C-LTM failure

Conclusion

In this contribution, we discussed the conditional LTM for Intra-CU LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: A new MAC CE for the TA value of C-LTM candidate cells includes the following information:

- C-LTM candidate cell ID(s)

- TA value(s)

Proposal 2: RAN2 is kindly asked to discuss introducing RACH-less based C-LTM attempt after C-LTM failure

Reference

RAN2#129 meeting report