3GPP TSG-RAN WG2 Meeting #129bis												                         R2-2502769

Wuhan, China, April 7th – 11th, 2025                                                           Revision of R2-2501159

Agenda item:	8.9.2

Source:	MediaTek Inc.

Title:	RAN2 impact on S&F mode

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1], one of the objectives is to support S&F operation, as below:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

A study report, 23.700-29 [2], on the integration of satellite components in the 5G architecture Phase 3 has been released by 3GPP SA2. The conclusion for Key Issue #2: Support of Store and Forward Satellite operation has been made. Two types of system architecture can be adopted by operation deployment: a full CN onboard the satellite or a Split MME architecture.

In this contribution, we share our view on S&F mode, incorporating input from SA2.

# Discussion

S&F operation indication

In the RAN2#127bis, RAN2 made the following agreements regarding S&F operation indication. This indication has three possible meanings:

- Absent: this NTN cell is operating in real-time mode.

- ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

- ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

SA2 has also mentioned this indication in the latest TS 23.401[3], the NAS needs to know if the satellite is operating in the S&F mode to indicate the S&F capability during Attach and TAU.

### 4.13.9 Support of Store and Forward Satellite Operation #### 4.13.9.1 General --omit-- When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode, and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

Thus, the AS shall forward this S&F operation mode indication to NAS. Regarding the three possible meanings, the expected AS behaviour is summarized as following.

- If this indication is absent, the cell is operating in normal mode, AS should not forward this indication to NAS.

- If this indication is present and the value equals ‘1’, the NTN cell is operating in S&F mode for all UEs. In this case, AS should notify NAS.

- If this indication is present and the value equals ‘0’, idle Rel-19 UEs supporting S&F are barred and the cell is operating in S&F mode for UEs in Connected mode. For UE in connected mode, AS should notify NAS if SIB1 is updated. For UE in idle mode, if the cell is barred, this information is no needed for NAS.

Proposal 1: For Rel-19 UEs supporting S&F, if the S&F operation indication is present in SIB1, RRC will notify upper layer that this NTN cell is operating in S&F mode when camping on the cell or receiving updated SIB1 in Connected mode.

Time information

In the RAN2#128, RAN2 made the following agreements regarding S&F time information.

Agreements:

- Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

The information of remaining time of S&F mode is broadcasted. The intention of this time information is to delay some NAS procedure. The Wait Timer which is also used to delay some NAS procedure is notified by NAS signalling. To control the NAS procedure combined with Wait Timer and remaining time of S&F mode, the NAS should be the controller. Therefore, the information of remaining time of S&F mode should be forwarded from RRC to NAS when it is first received or updated thereafter.

Proposal 2: For Rel-19 UEs supporting S&F, the RRC will forward the remaining time of S&F mode to the upper layer upon reception and when updated.

Besides remaining time of S&F mode, the time information when the cell will enter the S&F mode can also be helpful. UE can trigger cell reselection despite other conditions when it is about to enter the S&F mode to look for a normal cell. Additionally, the device user can be notified of how long the feeder link will maintain available, allowing them to take the opportunity to start delay-sensitive transmissions.

Proposal 3: The time information when the cell will switch to S&F mode can be broadcasted in system information.

Release Cause

It has been agreed that the SIB1 contains the S&F operation indication. The UE can be aware of the S&F mode change due to the SIB1 modification notification. It doesn’t appear necessary to inform the UE that the RRC connection release is because of switching to S&F mode, UE will know it anyway.

According to TS 24.301[3], if NAS procedure is blocked by the S&F mode, the NAS procedure will be rejected. There will be a new reject cause in NAS.

### 4.13.9 Support of Store and Forward Satellite Operation #### 4.13.9.1 General --omit-- When an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

Therefore, a new RRC release cause is not needed.

Proposal 4: New RRC release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not needed.

Satellite ID

According to the SA2 CR [4], a S&F monitoring list of satellites IDs will be optionally provided to the UE during the attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. Note that how UE handle the S&F Monitoring List is up to UE implementation.

S2-2502513 When an on-board MME supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update. NOTE 2: Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O. The MME may adapt periodic update timer, mobile reachable timer and Implicit Detach timer to the fact that the UE is served by an MME operating in S&F Mode. For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause, with asatellite of the same PLMN that is operating in S&F Mode. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. - The MME may indicate to the UE that it should delete any previously provided S&F Monitoring List for the current PLMN. When the S&F Monitoring List is deleted then the UE may use any satellite(s). NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode.NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one. NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

To facilitate the UE prioritization of the cell on Satellite ID list, the satellite ID in the neighbour cell list can be used. For example, UE can search a cell on the frequency where the corresponding satellite ID is in the satellite ID list provided by NAS.

Currently, SIB31, SIB32, and SIB33 contain satellite IDs. The satellite ID in SIB31 belongs to serving cell. The satellite ID list in SIB32 and SIB33 refer to the neighbour cells.  To be able to use the satellite ID list, the satellite IDs in the SIB31, SIB32 and SIB33 must in a same name space, which is not clear according to current specification. And the satellite ID list in NAS is provided by MME belong to the same PLMN according to the CR [4], hence the satellite IDs provided in SIB31, SIB32, and SIB33 are globally unique within the current PLMN.

Proposal 5: Network ensures that the satellite IDs provided in SIB31(-NB), SIB32(-NB), and SIB33(-NB) are globally unique within PLMN.

# Conclusion

S&F operation indication

Proposal 1: For Rel-19 UEs supporting S&F, if the S&F operation indication is present in SIB1, RRC will notify upper layer that this NTN cell is operating in S&F mode when camping on the cell or receiving updated SIB1 in Connected mode.

Time information

Proposal 2: For Rel-19 UEs supporting S&F, the RRC will forward the remaining time of S&F mode to the upper layer upon reception and when updated.

Proposal 3: The time information when the cell will switch to S&F mode can be broadcasted in system information.

Release Cause

Proposal 4: New RRC release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not needed.

Satellite ID

Proposal 5: Network ensures that the satellite IDs provided in SIB31(-NB), SIB32(-NB), and SIB33(-NB) are globally unique within PLMN.

# References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

TR 23.700-29 v.1.0.0. Study on integration of satellite components in the 5G architecture Phase 3

TS 23.401 v19.1.0 General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access

S2-2502513, “Support of S&F Monitoring List in S&F Mode”, Huawei, HiSilicon, Samsung CR	Rel-19	23.401	5GSAT_Ph3-ARC