3GPP TSG-RAN WG2 Meeting #129bis   								 R2-2502795

Wuhan, China, 7 – 11 April, 2025

Agenda Item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on LCM for positioning use case

Document for:	Discussion and decision

Introduction

At the RAN#102 meeting, a new R19 WI on AI for air interface [1] was approved and further revised in the RAN#106 meeting [2]. One objective is specification of support for positioning accuracy enhancements. At RAN2#129 meeting, RAN2 agreed on the basic procedure for LCM for case 1, and some FFSes are left for further discussion.

Agreements

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

In this paper, we mainly discuss possible RAN2 impacts for UE-sided and NW-sided model for positioning.

Discussion

UE-sided model for positioning

Discussion on UE functionality reporting for positioning

For UE autonomous switching between AI/ML and non-AI/ML methods, there are two scenarios:

Scenario A: the UE only has one positioning method and this method is based on AI/ML

Scenario B: the UE has multiple positioning methods, and at least one method is a non-AI/ML method and one method is an AI/ML based method

For scenario A, due to the previous email discussion, majority of  companies preferred to let LMF control the switching, i.e. LMF can reconfigure the UE after an updated LPP ProvideCapabilities message or a failure message is received, and UE autonomous switching is not needed.

Currently the UE is allowed to have multiple positioning methods simultaneously, and LMF can control them separately. For scenario B, our views are as below:

- Assumed that the UE is configured with positioning method A based on AI/ML, and also configured with positioning B which is a legacy method. Initially, both methods are just configured and the UE waits for NW command on the reporting

- Later, LMF lets UE send positioning results for method A, and then it can not work well due to some reasons

- In this case, we think LMF can let UE stop method A and allow UE to send positioning results for method B, which is LMF-controlled switching in our view. We understand that for UE autonomous switching, it may mean that the UE can autonomously stop method A and start to send positioning results for method B. For this UE autonomous switching behavior, we do not see benefits compared with LMF-controlled switching, and there are more specification impacts

In summary, no matter how many positioning methods are available on the UE side, LMF should have control over each method, i.e. UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.

Proposal 1: UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.

At RAN2#125bis meeting, RAN2 made the following agreements, and we think previous RAN2 agreements on applicability reporting should be able to cover both BM and Positioning use cases. And then, we think the CR rapporteur can take this into account and then implement these agreements into the relevant specifications. In section 7 Annex, we have listed all RAN2 agreements on applicability reporting. It is noted that some of them are specific to BM so that they should not be applicable to Positioning.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

- FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Among all aspects, we think the following two aspects may need some discussions for positioning:

(1) For BM, RAN2 is discussing Option A and Option B, based on RAN1 LS R2-2500011. For positioning, we think there may be similar options:

Option A: in step 3, LMF can provide full inference configuration to the UE, and the UE can activate applicable functionality after step 4

Option B: in step 3, LMF can provide inference related parameters for applicability report only (not for inference). In step 4, the UE reports applicability, and LMF can provide inference configuration to the UE to activate indicated functionality

We think RAN2 can discuss both option A and option B, and then make some conclusions.

(2) The following FFS is about LMF control over applicability reporting.

UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

As we mentioned above, the previous RAN2 agreements on applicability reporting can be re-used for Positioning. For BM, UAI can be used for reporting and prohibit timer may be needed to restrict such reporting. For positioning, based on the email report [7], majority companies saw no issues to support unsolicited LPP ProvideCapabilities without explicit LMF control as in legacy, so they did not want to have any LMF control on applicability reporting.

Our views are:

- For capability reporting, "unsolicited messages" is purely up to UE, and there is no need to have network control. For example, in legacy, it may happen that the UE supports positioning method 1, but the NW does not support it, and such UE reporting behavior has been allowed based on current specs.

- For applicability reporting, we understand that if step 4 is out of network control, the UE may unnecessarily report its applicable functionalities (also in case they change). Similar to BM case where where the network configures whether the UE should report its applicable functionality, we suggest to discuss the necessity of NW configuration whether to allow step 4 or not.

Proposal 2: It is proposed RAN2 to discuss the following options for the applicability report:

Option A: in step 3, LMF can provide full inference configuration to the UE, and the UE can activate applicable functionality after step 4

Option B: in step 3, LMF can provide inference related parameters for applicability report only (not for inference). In step 4, the UE reports applicability, and LMF can provide inference configuration to the UE to activate indicated functionality

Proposal 3: It is proposed RAN2 to discuss the necessity of NW configuration whether to allow step 4 or not, e.g. a prohibit timer.

Discussion on monitoring

For the UE-sided model positioning, in the RAN1 116b meeting, the following agreements have been achieved.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

We can observe that for the UE-sided model positioning case, the monitoring related information exchange is between UE and LMF. Meanwhile, the functionality management triggers and commands are also between UE and LMF. The UE and LMF communicate via LPP signaling. The main procedures should include the following.

- Report configurations from LMF to UE. According to the RAN1 116b meeting, in option A, it is the UE to calculate the monitoring metrics while in option B, it is the LMF to perform the calculation based on UE reported inference result. Whichever option is chosen, the LMF should configure the UE about how to acquire calculation assistance data (option A) or what inference results to report (option B).

- Report from UE to LMF. Based on the configuration from LMF, the UE reports to LMF via LPP signaling. The UE may be configured to report with trigger conditions or periodically. After receiving reports from UE, the LMF may further send another configuration to the UE, to modify the previous configuration or cancel the UE reporting.

For now, we see that RAN1 is still discussing options and they may do down-selection of some options. Hence, we think RAN2 can wait for more RAN1 progress on monitoring.

Proposal 4: For performance monitoring for positioning Case 1, RAN1 is still discussing and analyzing details of different options and they may do down-selection in future. RAN2 can wait for more RAN1 progress before discussing this aspect.

Conclusions

In this paper, we discuss UE functionality reporting for positioning and monitoring. We have the following proposals:

Proposal 1: UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.

Proposal 2: It is proposed RAN2 to discuss the following options for the applicability report:

Option A: in step 3, LMF can provide full inference configuration to the UE, and the UE can activate applicable functionality after step 4

Option B: in step 3, LMF can provide inference related parameters for applicability report only (not for inference). In step 4, the UE reports applicability, and LMF can provide inference configuration to the UE to activate indicated functionality

Proposal 3: It is proposed RAN2 to discuss the necessity of NW configuration whether to allow step 4 or not, e.g. a prohibit timer.

Proposal 4: For performance monitoring for positioning Case 1, RAN1 is still discussing and analyzing details of different options and they may do down-selection in future. RAN2 can wait for more RAN1 progress before discussing this aspect.

References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- TR 38.843 v18.0.0

- [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)

- S2-2501133, LS on LMF-based AI/ML Positioning for Case 2b, Source: SA2, To: RAN1, RAN2, Cc: RAN3

- S2-2501341, LS on LMF-based AI/ML Positioning for case 3b, Source: SA2, To: RAN3, RAN1, Cc: RAN2

- R2-2500968	Report of [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)	Ericsson	discussion

Annex: RAN2 agreements on applicability reporting

RAN2#125bis

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

- FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

RAN2#126

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

RAN2#127

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

RAN2#127bis

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

RAN2#128

Agreements

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

4.	Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

5.	Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

6.	For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

RAN2#129

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI