3GPP TSG-RAN WG2 Meeting #129bis					    R2-2502797

Wuhan, China, 7 - 11 April, 2025

Title: 	Discussion on signalling feasibility of dataset and parameter sharing for CSI compression

Source: 	Huawei, HiSilicon

Agenda item:	8.1.5

Document for:	Discussion/Decision

- Introduction

For two-sided model, RAN1 has sent a LS [1] to RAN2, and the action is listed as below:

To: RAN2 CC: SA2/SA3/SA5 Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

During the email discussion [3], some options have been identified.

In this contribution, we provide our analysis on feasibility of RAN1 identified solutions on two-sided model.

- Discussion

Email discussion progress on OTA solution 1a

In [3], OTA solution 1a is considered as one candidate OTA solution. The summary is copied as below:

| Yes (8/14) | Vivo (small size), HW (split to multiple UEs), ZTE (same as HW), QC (but significant spec impact and implementation complexity), Oppo (same as HW), CATT (hard RRC segment), FW, SS (concern about direct server-to-server transfer) |
| --- | --- |
| No (7/14) | Apple, Lenovo, Ericsson, Nokia, MTK, LG, NEC |

Challenges listed by companies:

- Not feasible to have RRC buffer >200Mbyte for UE in 5G (Apple, Lenovo, QC, Ericsson, Lenovo, MTK, LG)

- Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, which is not realistic in late 5G, e.g. handling segmentation, retransmission, etc (Apple, Nokia, LG)

- UE selection may have significant specification impact (QC)

- Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition (QC, Ericsson, LG)

- Uu overhead for data collection from UE and dataset/model parameter transfer to UE (Ericsson)

- overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) (MTK)

On issue about size of dataset/model parameter, some feedback from companies to address the issue:

- OTA solution to start with model parameters which has a smaller size, e.g. 11.6MB (Vivo, HW)

- NW can split data into pieces in either RRC layer or higher layer, then selects proper UE(s) at proper time for transmission. UE training entity to merge different pieces from multiple UEs (HW, ZTE, Oppo, CATT)

Rapporteurs understand the intention to start from smaller size, while as mentioned earlier, there’s no intention from RAN2 point of view to down-select between whether to transfer dataset and model parameter, considering the later one may also require significant standardization effort, which is RAN1 scope to discuss further. Also, even for 11.6MB, it still needs around 1288 RRC segments, which is significantly larger than existing supported RRC segment (which is 9 in DL). Based on the feedback above, there’s also no consensus on whether thousands of RRC segments are feasible or not.

Regarding to split dataset/model parameter into different pieces, rapporteur understands it could be feasible, but there are some remaining open issues need to be further clarified:

- what is the suitable size of each piece

- who and how to select UE

- a new protocol layer on top of RRC for segmentation

There’s also one company questions what is the size threshold where required RRC segment number increasement are no longer viable. As replied by some companies, RRC is designed to transmit core control messages to maintain communication reliability, rather than for data transmission.

In summary, considering split views based on companies’ feedback, there’s no consensus on the feasibility of OTA solution 1a. However, since RAN1 is waiting for RAN2 reply to conclude their Rel-19 SI in next meeting, based on discussion from phase 1, at least all companies agree this is a candidate solution studied from RAN2 point of view, but no consensus on feasibility. Furthermore, as the conclusion of this email discussion and study of OTA solution 1a, rapporteurs further provide below table to conclude challenges of OTA solution 1a. We will not capture any solution as part of the study conclusion, as details and solutions can be discussed during normative phase.

| Challenges | - Not feasible to have RRC buffer >200Mbyte for UE in 5G - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) |
| --- | --- |
| Potential suitable scenario | - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs. |

Solution details on OTA solution 1a

For Solution 1a for model transfer/delivery defined in [2], the analysis from TR 38.843 is copied as below and we think it can be used as a starting point for analyzing OTA solution 1a of transfer of dataset/parameters.

Table 7.3.1.4-1 Analysis of current status and gaps, and potential 
RAN specification impact for Solution 1a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | Model size >45kBytes is not supported based on existing number of RRC segments | Extension of the number of RRC segments is required to support models larger than 45kBytes |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Transmission is restarted upon mobility | - Requires service continuity support for SRBs with segmentations. - Xn/NGAP enhancement(s) for model transfer/delivery continuity |
| A3. Network controllability on model transfer/delivery and management at gNB | Management and interaction between UE and gNB is not supported | Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion indication, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | Procedure latency depends on model size and SRB priority | Impact on SRB in DL, e.g., a new SRB with configurable priority, etc. |

For dataset/parameter, if the size is less than 45kBytes, dataset/parameter transfer using RRC signalling is feasible.

Proposal 1: If the size of dataset/parameter is less than 45kBytes, OTA solution 1a is feasible based on existing RRC segmentation mechanism.

If the size is larger than 45KBytes, there may be some challenges. For example, if the size of dataset/parameter is 50MB, it means at least 5556 PDU SDU (as defined in TS 38.323, the maximum supported size of a PDCP SDU is 9000 bytes). To support the transfer of such a large amount of information, RAN2 may discuss the following enhancements:

(A) If NG-RAN is to do the segmentation of information in RRC layer, extension of the number of RRC segments is required to support dataset/parameter larger than 45kBytes

(B) If NG-RAN is to do the segmentation of dataset/parameter above RRC layer (e.g. a new AS protocol layer), there may be few impacts to RRC

In the RAN1 LS, Option 3a-1 and 4 require offline engineering and hence OTT server is needed. The dataset/parameter can be transferred to UEs via two methods:

Method A): "Point to point (P2P)". NG-RAN can transfer all dataset/parameter to one UE, and then the UE transfers the information to OTT server

Method B): "Point to multipoint (P2M)". NG-RAN can transfer dataset/parameter to some UEs, and each UE may get part of the information. And then these UEs can transfer the information to OTT server. At OTT server, all received information can be combined into a whole one

In summary, there is a combination of solutions:

Table 1: Combination of solutions for OTA solution 1a

|  | New AS protocol layer to do segmentation | RRC layer to do segmentation |
| --- | --- | --- |
| P2P | There may be a large amount of RRC messages for one UE, e.g. one RRC message carries a segment from the upper layer. Transparent to RRC layer. | There may be a large amount of RRC message segmentations (e.g. the DLDedicatedMessageSegment message) for one UE. Non-transparent to RRC layer. |
| P2M | There may be a small amount of information for one UE, e.g. one RRC message. Transparent to RRC layer. | There may be a small amount of information for one UE, e.g. one RRC message. Non-transparent to RRC layer. |

For P2M transmission, the data toward one UE is not large, so the impact at UE side is small. However, if a new protocol layer is to do segmentation, there may be some RAN2 impacts.

For P2P transmission with a large amount of data, according to the analysis in section 2.1, the transfer of dataset/parameter is "relaxed latency (e.g., days/weeks) and infrequent", so we think P2P transmission is feasible due to the following two reasons:

(a) the network does not require to transfer a large amount of data to the UE in a given short period of time

(b) the network can select a proper time and a proper UE for the transmission, e.g. when the network is in lightly loaded and the UE has good signal quality. In this case, data transmission is not a big problem

Proposal 2: For OTA solution 1a, if the size is larger than 45KBytes, the following enhancements can be considered:

- NG-RAN can do the segmentation of dataset/parameter above RRC layer. This is transparent to RRC layer and a new AS protocol layer may have some RAN2 impacts

- NG-RAN can do the segmentation of dataset/parameter in RRC layer. This is non-transparent to RRC layer, and this may have some RAN2 impacts

Proposal 3: Regarding how NG-RAN transfers dataset/parameter to UE, the following two ways can be considered:

- "Point to point (P2P)". NG-RAN can transfer all dataset/parameter to one UE, and then the UE can directly use them or the UE transfers the information to OTT server

- "Point to multipoint (P2M)". NG-RAN can transfer dataset/parameter to some UEs, and each UE may get part of the information. And then these UEs transfer the information to OTT server. At OTT server, all received information can be combined into a whole one

As we mentioned above, if the size of dataset/parameter is larger than 45KBytes, it may need to divided to several segments (either segments in the layer above RRC or RRC segments), and then NW can transfer these segments to UE. In our understanding, one or more segments may form a subset, so the NW may send the subsets to the UE. It is expected that all segments should be transferred to UE side successfully, however, it may happen that some segments may not be received by the UE. In this case, even if the transfer of dataset/parameter is triggered by NW, it is beneficial for UE to request the delivery of some missing segments/subsets to the network.

An alternative is that the NW can inform UEs of segments/subsets information in advance, such as total number of segments/subsets and identifiers. Then, UEs can request specific segments/subsets from the NW.

Proposal 4: For OTA solution 1a, the NW can transfer segments/subsets of dataset/parameter to UE, and then the UE can request missing segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Proposal 5: For OTA solution 1a, the NW can inform UEs of segments/subsets information to UE in advance, such as total number of segments/subsets and identifiers. Then UEs can request specific segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Address challenges due to email discussion

As we shown in section 2.1, the email rapporteur has summarized challenges for OTA solution 1a, i.e.:

(1) Not feasible to have RRC buffer >200Mbyte for UE in 5G

(2) Significant specification impact:

- Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc

- UE selection

(3) Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition

(4) Uu overhead for data collection from UE and dataset/model parameter transfer to UE

(5) Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc)

For P2P

For challenge (1), as we explained in section 2.2, we think P2P is feasible.

For challenge (2), there may be the following specification impacts:

- a) For segmentation, the maximum segmentation number needs to be extended

- b) Spec impacts due to information on segments/subsets info (such as total number and identifiers) exchanged between UE and NG-RAN

- c) There may be RAN2 impacts if new AS protocol layer is considered

For UE selection, i.e. how UE selection for sharing of dataset/parameters are performed, as we explained in section 2.2, the UE can NW can exchange some meta data of dataset/parameter, and then the transfer of dataset/parameter can be performed. During the email discussion, some companies think that the UE selection means UE vendor information needs to be exchanged between UE and NW. We are not sure about this requirement, as RAN1 has not mentioned anything about vendor specification information due to RAN1 LS.

For challenge (3), we understand that E2E reliability is how UE-side server can completely get the required dataset/parameter from NW, and it is not about specific UE(s). In other words, with OTA solution 1a, even if some pieces may not be successfully got by one UE or some UE(s) due to HO or state transition, there are still means to let UEs request the missing pieces from NW. For example, either the NW or UE can record the transfer progress, and when UE returns to RRC_CONNECTED or HOs to the target cell, the dataset/model parameter transfer can be continued based on the recorded progress. So, we think challenge (3) has been addressed by our solutions.

For challenge (4), RAN1 indicates that the transmission is infrequent, so Uu overhead is not a problem.

For challenge (5), the issue can be solved if low-priority SRB is used, e.g. like SRB4 for QoE collection.

In addition, for P2P transmission, the impacts on UE power consumption and memory can be minimized. Firstly, following relaxed latency and infrequent transmission, UE power consumption increases only in a small time dimension in a long time period, so the UE power consumption is not a problem. Secondly, for OTA solution 1a, the UE is a relay and the UE should check both OTT and OTA parts before starting dataset/parameter transfer, and it means that after receiving data from the network, the UE should transfer the data to the OTT server as soon as possible. As a result, there should not be significant impacts on UE memory. For NW-side data collection, there is also discussion on UE memory, and we can follow the same principle here.

For P2M

For challenge (1), as we explained in section 2.2, we think P2M is feasible.

For challenge (2), there may be the following specification impacts:

- a) Spec impacts due to information on segments/subsets info (such as total number and identifiers) exchanged between UE and NG-RAN.

- b) There may be RAN2 impacts if new AS protocol layer is considered.

For UE selection, similar as our analysis for P2P, we observe no issues.

For challenge (3), (4), (5), similar as our analysis for P2P, we observe no issues.

In addition, for P2M transmission, as the data volume of dataset/parameter for one specific UE is quite small, e.g. may be just one RRC segmentation or a small number of RRC segmentations. Then the impacts on UE power consumption and memory are minimal.

Here is a summary:

Table 2: Technical analysis on challenges for OTA solution 1a (P2P and P2M)

| Challenges | P2P | P2M |
| --- | --- | --- |
| (1) Not feasible to have RRC buffer >200Mbyte for UE in 5G | Feasible | Feasible |
| (2) Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection | Possible specification impacts: a) For segmentation, the maximum segmentation number needs to be extended. b) Spec impacts due to information on segments/subsets info (such as total number and identifiers) exchanged between UE and NG-RAN. c) There may be RAN2 impacts if new AS protocol layer is considered. The UE can NW can exchange some meta data of dataset/parameter, and then the transfer of dataset/parameter can be performed. | Possible specification impacts: a) Spec impacts due to information on segments/subsets info (such as total number and identifiers) exchanged between UE and NG-RAN. b) There may be RAN2 impacts if new AS protocol layer is considered. The UE can NW can exchange some meta data of dataset/parameter, and then the transfer of dataset/parameter can be performed. |
| (3) Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition | This challenge can be solved. We understand that E2E reliability is how UE-side server can completely get the required dataset/parameter from NW, and it is not about specific UE(s). The UE can request the missing pieces from NW if needed. | Similar as for P2P |
| (4) Uu overhead for data collection from UE and dataset/model parameter transfer to UE | RAN1 indicates that the transmission is infrequent, so Uu overhead is not a problem. | Similar as for P2P |
| (5) Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) | This challenge can be solved. The issue can be solved if low-priority SRB is used, e.g. like SRB4 for QoE collection. | Similar as for P2P |
| Others, e.g. the impacts on UE power consumption and memory | The impacts can be minimized. | The impacts can be minimized. |

Proposal 6: It is proposed RAN2 to agree on table 2, which provides technical analysis on challenges for OTA solution 1a.

Proposal 7: OTA solution 1a (including P2P and P2M) is feasible even in large dataset/parameter.

- Conclusion

In this contribution, we have the following proposals regarding OTA solution 1a:

Proposal 1: If the size of dataset/parameter is less than 45kBytes, OTA solution 1a is feasible based on existing RRC segmentation mechanism.

Proposal 2: For OTA solution 1a, if the size is larger than 45KBytes, the following enhancements can be considered:

- NG-RAN can do the segmentation of dataset/parameter above RRC layer. This is transparent to RRC layer and a new AS protocol layer may have some RAN2 impacts

- NG-RAN can do the segmentation of dataset/parameter in RRC layer. This is non-transparent to RRC layer, and this may have some RAN2 impacts

Proposal 3: Regarding how NG-RAN transfers dataset/parameter to UE, the following two ways can be considered:

- "Point to point (P2P)". NG-RAN can transfer all dataset/parameter to one UE, and then the UE can directly use them or the UE transfers the information to OTT server

- "Point to multipoint (P2M)". NG-RAN can transfer dataset/parameter to some UEs, and each UE may get part of the information. And then these UEs transfer the information to OTT server. At OTT server, all received information can be combined into a whole one

Proposal 4: For OTA solution 1a, the NW can transfer segments/subsets of dataset/parameter to UE, and then the UE can request missing segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Proposal 5: For OTA solution 1a, the NW can inform UEs of segments/subsets information to UE in advance, such as total number of segments/subsets and identifiers. Then UEs can request specific segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Proposal 6: It is proposed RAN2 to agree on table 2, which provides technical analysis on challenges for OTA solution 1a.

Proposal 7: OTA solution 1a (including P2P and P2M) is feasible even in large dataset/parameter.

- References

R1-2410922, LS on signalling feasibility of dataset and parameter sharing, Source: RAN1, To: RAN2, Cc: SA2, SA3, SA5

TR 38.843 v18.0.0

R2-250xxxx_[POST129][029][AI Phy] Model transfer (XiaomiEricsson)_P2_v18_Rapp, Xiaomi, Ericsson

- Annex - Summary proposals from R2-250xxxx_[POST129][029][AI Phy] Model transfer (XiaomiEricsson)_P2_v18_Rapp

Principle (new principles proposed in phase 1 to be discussed in Q3-2)

Proposal 1 [easy agreeable, phase 2 12/14]: Dataset/model parameter transfer solution from NW to UE training entity shall follow below principles:

- A1 - Size: From RAN2 point of view, RAN2 aims to define a unified solution (e.g. OTA, non-OTA, or a combined) to support various sizes of dataset/model parameter transfer (dataset and/or parameter sharing size could range from tens of KBs to hundreds of MBs, but in average around hundreds of MBs);

- A2 - Continuity: Service continuity of dataset and/or parameter transfer/delivery during UE mobility needs to be supported;

- A3 - Controllability: NW decides on if and when to transfer/delivery the dataset and/or model parameter from NW to UE or UE training entity (a server inside MNO or an OTT server);

- A4 - Latency: Relaxed latency requirement and infrequent update;

- A5 - Visibility: dataset and model parameter to be understandable by UE/UE-side training entity (a server inside MNO or an OTT server).

- A6: Proprietary information of the network should be respected and not disclosed.

Alternatives for dataset/model parameter transfer (non-OTA, OTA):

Proposal 2 [phase 2 10/14]: RAN2 to conclude following approaches for dataset/model parameter transfer Direction A:

Alternative 1 (non-OTA approach): gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server) data RAN2 analyzed area dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training UE-side training entity for two-sided UE part model training. Alternative 2 (OTA approach): gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server) RAN2 analyzed area UE-side training entity for two-sided UE part model training dataset/model parameter for training dataset/model parameter transfer CSI compression data collection at gNB NW-side dataset/model parameters collection entity (gNB/CN/OAM) for two-sided UE-part model training data. NOTE: The data transfer (e.g. raw data, dataset and/or model parameter, etc, up to RAN1 further details on what to transfer) between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

non-OTA solution

Proposal 3 [phase 2 10/14]: For non-OTA solutions ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidates and did not identify any showstopper from RAN2 point of view. Feasibility analysis of non-OTA solutions is required to be evaluated by RAN3, SA2, and SA5. It does not preclude RAN3/SA2/SA5 to identify other candidate solutions beyond options listed below.

| Option | Impacted WG | Specification impact/Implementation impact |
| --- | --- | --- |
| - OAM -> UE-side training entity (a server inside MNO or an OTT server), where OAM is NW-side dataset/model parameter collection entity | SA5 | Up to SA5 (any intermediate node, if any, between OAM and UE-side OTT server is up to SA5; CN involvement if needed is up to SA2/SA5 discussion) |
| - CN -> UE-side training entity (a server inside MNO or an OTT server), where CN is NW-side dataset/model parameter collection entity | SA2 | Up to SA2 (any intermediate node, if any, between CN and UE-side OTT server is up to SA2) |
| - gNB -> OAM/CN -> UE-side training entity (a server inside MNO or an OTT server), where gNB is NW-side dataset/model parameter collection entity | RAN3, SA2, SA5 | Up to RAN3, SA2, SA5 (any intermediate node, if any, between gNB/OAM, OAM/UE-side OTT server, CN/UE-side OTT server is up to RAN3/SA2/SA5) |

OTA solution

Proposal 4 [easy agreeable, phase 1 14/14]: From RAN2 point of view, when gNB is the NW dataset/model parameter collection entity, solution 1b (i.e. gNB -> UE via UP) is not considered as a candidate solution in 5GA.

Proposal 5 [easy agreeable, phase 1 14/14]: For OTA solutions ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions but there’s no consensus on feasibility:

- OTA solution 1a: gNB -> UE via CP

- OTA solution 2: CN -> UE via gNB

- OTA solution 3: OAM -> UE via gNB

Proposal 6 (to be discussed online): RAN2 to conclude following challenges and potential suitable scenario for OTA solutions:

| OTA Solution 1a | Challenges | - Not feasible to have RRC buffer >200Mbyte for UE in 5G - Significant specification impact: - Other segmentation beyond RRC layer requires a new SRB protocol stack to perform segmentation, including functions such as handling segmentation, retransmission, etc - UE selection - Challenges to support E2E reliability, considering dataset/model parameter transfer is shared by different gNB/vendors during UE mobility and different RRC state transition - Uu overhead for data collection from UE and dataset/model parameter transfer to UE - Overloading CP with large datasets would disrupt core control message transmission (e.g. service degradation, reliability, etc) |
| --- | --- | --- |
|  | Potential suitable scenario | - Small dataset/model parameter size. However, the maximum RRC segment needs to be further studied - Split large dataset/model parameter into small pieces, and potentially send to multiple UEs, then gather by UE training entity. RAN2 has not study the feasibility of split dataset/model parameter to multiple UEs. |
| OTA Solution 2 | Challenges | - Same challenges as OTA solution 1a, if OTA solution 2 via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to CN, then transmit back to gNB. Relaying dataset/model parameter via gNB to UE then back to UE training entity is not desireable. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure if gNB and CN are not from the same NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution 2 via CP is the same as OTA solution 1a. - OTA solution 2 and its feasibility is required to be evaluated by RAN3 and SA2. |
| OTA Solution 3 | Challenges | - Same challenges as OTA solution 1a, if OTA solution 2 via CP - No benefit over non-OTA solution, as dataset/model parameter needs to transmit to OAM, then transmit back to gNB. - Unclear how to guarantee E2E reliability across multiple hops - A risk of proprietary information exposure that OAM may share to a second NW vendor |
|  | Potential suitable scenario | - Feasibility analysis of OTA solution 3 via CP is the same as OTA solution 1a. - OTA solution 3 and its feasibility is required to be evaluated by RAN3 and SA5. |

Proposal 7 [easy agreeable, phase 1 13/14]: In OTA approach, UE transfers the received dataset/model parameter to UE training entity (an OTT server) transparently to 3GPP network.

Proposal 8: RAN2 reply RAN1 LS based on based on outcome of Proposal 2/3/5/6 and to RAN3, SA2 and SA5 for further action on feasibility analysis of non-OTA solution and OTA solution 2 and 3.