3GPP TSG RAN WG2 #129-bis				R2-2502820

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.2.3

Source:		ASUSTeK

Title:				Discussion on Ambient IoT Msg2 design

Document for: Discussion and Decision

- Introduction

In RAN2#129 meeting [1], the following agreements related to ambient IoT random access were made:

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

In RAN1#120 meeting [2], the following agreements related to ambient IoT random access were also made:

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2. Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure.

In this contribution, we further discuss the detailed contents of A-IoT Msg2.

- Discussion

2.1  Msg2 content

2.1.1  ID

In CBRA, a random ID is sent in Msg1 from the device to the reader. In response to a successful reception of Msg1, the reader should send Msg2 which echoes the random ID, at least for the purpose of contention resolution. As for CFRA, Msg2 could include the same ID as in the paging message or in Msg1 for further transmission (e.g. Msg3).

Moreover, according to the agreements in [2], it is supported that:

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

It is clearly feasible for Msg2 to include one ID field corresponded to one Msg1 from one device. On the other hand, given that TDMA and/or FDMA could be supported for Msg1 transmission, there may be X*Y occasions for Msg1 triggered by the same R2D transmission. When multiple devices perform Msg1 transmission on different occasions, and if Msg2 only includes one ID field from a single device, the number of Msg2 would be up to X*Y, resulting in intolerable latency and signalling overhead. Therefore, we propose that Msg2 is supported to include multiple ID fields corresponded to multiple Msg1 from different devices. Moreover, it would be more flexible to support multiple ID fields in Msg2. It allows the reader to include one or multiple IDs, depending on the reader.

Proposal 1:  Msg2 is supported to include one or multiple ID fields.

2.1.2  D2R resource information

For 3-step RA procedure, Msg3 is supposed to include the device ID. In TR 38.769 [3] as below, the information related to D2R resource is provided via corresponding PRDCH. Nevertheless, how to provide D2R resource for Msg3 still needs further study by RAN1. Design of this part could be postponed till RAN1 agree to use higher-layer signalling to carry D2R resource.

#### 6.1.2.8 Scheduling of D2R For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via the corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions - Midamble (if supported) related information For each information, it is for further study whether higher-layer signalling and/or L1 R2D control signalling is used - see Clause 6.1.1.10.

Proposal 2:  The design of D2R resource information included in Msg2 could wait for RAN1’s progress.

2.2  Msg2 for re-access

According to TR 38.769 [3] as below:

6.3.5	A-IoT data transmission

…

the reader can re-send A-IoT Msg2 to specific device(s) to echo the random ID(s), in case of failure reception of corresponding D2R data transmission (i.e. "Msg3") after the initial A-IoT Msg2, together with the corresponding D2R resource scheduling. This can trigger the A-IoT device to re-send the same D2R data transmision (i.e. "Msg3"). If it is supported to include the echoed random IDs for multiple devices, the re-sent A-IoT Msg2 only includes the random IDs of the devices, whose "Msg3" is not successfully received.

To solve failure reception of Msg3, the reader can re-send Msg2 to trigger the device to re-send the same Msg3. For clarification, the format of the re-sent Msg2 should be the same as format of the initial Msg2, except for the re-sent Msg2 only includes the random ID of the devices whose Msg3 is not successfully received.

Proposal 3:  In the case of solving failure reception of Msg3, the format of the re-sent Msg2 should be the same as the format of the initial Msg2.

- Conclusion

Proposal 1:  Msg2 is supported to include one or multiple ID fields.

Proposal 2:  The design of D2R resource information included in Msg2 could wait for RAN1’s progress.

Proposal 3:  In the case of solving failure reception of Msg3, the format of the re-sent Msg2 should be the same as the format of the initial Msg2.

- Reference

[1] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece

[2] Draft Report of 3GPP TSG RAN WG1 #120 v0.3.0 (Athens, Greece, February 17th – 21st, 2025)

[3] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”