3GPP TSG-RAN WG2 Meeting #129bis	R2-2502839

Wuhan, China, 7 - 11 Apr, 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on DL coverage enhancements

Agenda Item:	8.8.2

Document for:	Discussion and decision

- Introduction

In the past meetings, RAN2 discussion on DL coverage enhancements is focused on SMTC enhancements, DTX/DRX, and cell barring:

- On SMTC enhancements, RAN2 made assumptions on the scenarios and in some scenario the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

- On DTX/DRX, RAN2 has agreed only two states (on/off) will be considered, further details are pending on RAN1 replies on the RAN2 LS [2] (RAN1 has provided answers on Q1 and Q4 in [3]).

- On cell barring, RAN2 has agreed to introduce new barring bit in SIB1 to bar UEs supporting DL coverage enhancement if it will be justified to use existing barring bit to bar UEs not supporting DL coverage enhancement.

In this contribution, we will provide further analysis on the remaining issues.

- Discussion

  - SMTC adjustment

- Enable configuring different SMTC periodicities

In RAN2#129 meeting, the following was agreed:

Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

It is still FFS whether the existing signalling for SMTC configuration needs to be enhanced.

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

<unrelated parts omitted>

smtc4list-r17                       SSB-MTC4List-r17                                                OPTIONAL,   -- Need R

<unrelated parts omitted>

}

SSB-MTC4List-r17::=                 SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC4-r17 ::=             SEQUENCE {

pci-List-r17                 SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId                          OPTIONAL,  -- Need M

offset-r17                   INTEGER (0..159)

}

In the existing signalling, SSB-MTC4 does not provide periodicity configuration, and the periodicity in SSB-MTC will be reused for SSB-MTC4. Also note that only one MO can be configured for the corresponding frequency, it is not feasible to use multiple MOs on a single frequency to configure different SMTCs with different frequencies:

--- cited from TS 38.331 clause 5.5.2.1 --- - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency; - an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value and that an smtc4list included in any measurement object with the same ssbFrequency has the same value;

It was also raised in RAN2#129 online discussion that the SMTC periodicity can be configured as the largest periodicity among neighbour cells. However, if one neighbour cell is of 160ms SSB periodicity and another neighbour cell is of 20ms SSB periodicity, using 160ms for SMTC periodicity will largely increase the measurement delay even for the neighbour cells of short SSB periodicity.

Take inter-frequency measurement as an example, the baseline is that the measurement result requires 8 samples, and the time taken for one sample needs to consider the occasion when DRX active duration, SMTC duration and gap duration overlap with each other. Therefore, if we use the largest periodicity for all the neighbour cells, the measurement delay will be unnecessarily increased to a large extent.

--- cited from TS 38.133 clause 9.3.5 --- Table 9.3.5-1: Measurement period for inter-frequency measurements with gaps (Frequency FR1) Condition NOTE1,2; T SSB_measurement_period_inter. No DRX; Max(200ms, Ceil(8 * Kgap) Max(MGRP , SMTC period)) CSSFinter. DRX cycle ≤ 320ms; Max(200ms, Ceil(8 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter. DRX cycle > 320ms; Ceil(8 * Kgap) DRX cycle CSSFinter. NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: For a UE supporting concurrent measurement GAPs, the MGRP above is the MGRP of the activated Pre-MG or the measurement gap associated with the target frequency layer to be measured if concurrent GAPs are configured..

Observation 1: At most one MO can be configured for a single NTN frequency, and configuring the SMTC periodicity to the largest SSB periodicities among neighbour cells on this frequency will largely increase the measurement delay.

To allow configuring different SMTC periodicities on the same frequency, RAN2 can consider using multiple smtc4lists in MO, each having its own SSB periodicity and duration, or introducing a separate SMTC periodicity list to correspond with the existing SMTCs in smtc4list.

Proposal 1: The network should be allowed to configure different periodicities for the multiple SMTCs on a single frequency.

- Enable configuring multiple SMTCs

Another issue is whether to enhance the existing SMTC mechanism to accommodate the differential arriving time of neighbour cell SSBs due to beam hopping. The following has been agreed in the last meeting:

From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1.

RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well).

RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

Agreements:

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

There are several options for SMTC enhancements:

- UEs in RRC_CONNECTED

Option 1: location-based SMTC selection

For connected UEs, it is important to ensure that both the UE and the NW have the same understanding of SMTC, because SMTCs not only impact gap configuration (for inter-frequency measurements that require gaps) but may also impose scheduling restriction. Therefore, if Option 1 is adopted, it needs to be further discussed whether the UE reports the selected SMTC to the network, and how the network confirms the selected SMTC (e.g. NW reconfigures the SMTC/gap configuration or some other automatic SMTC selection mechanism).

Option 2: SSB-based SMTC selection

In this option, the network configures the SMTCs (and corresponding gaps for inter-frequency measurements that require gaps) based on the reported SSB measurement results from the UE. This option largely relies on beam-level measurement. As illustrated in the picture, if the UE is at the overlapping area of SSB1 and SSB4, it is possible that SSB4 is reported as the best beam and the network configures SMTCs corresponding to Cell 5 and Cell 6 (especially considering some UEs support a maximum of two SMTCs per frequency), while Cell 1 is the closest neighbour cell to the UE.

Option 3: UE assistance information reporting

This option is quite similar to Option 1, but it decouples the UE reporting procedure from SMTC/gap configuration. The network configures SMTC and gap as in legacy, and a separate procedure is used for UE assistance reporting. It is still FFS what exactly the assistance information is, for instance, UEAssistanceInformation message can be reused, or MeasurementReport message can be reused if location-based events are defined.

Option 4: alternate SMTC occasions

In this option, one SMTC is configured with a small periodicity (much smaller than the SSB periodicity), and UE takes turns to measure different neighbour cells in each SMTC period. UE will be indicated which PCI to measure in each period. This is basically another form of “multiple SMTC configuration” but it reduces the signalling overhead to one SMTC configuration, leveraging the characteristics of SSBs of neighbour cells. This option requires the SSBs of neighbour cells appear in a same SSB periodicity and the offset is also in an evenly increasing pattern.

Proposal 2: RAN2 to consider the following options for configuring SMTCs for UEs in RRC_CONNECTED:

Option 1: location-based SMTC selection, FFS UE/NW alignment on SMTC/gaps

Option 2: SSB-based SMTC selection

Option 3: UE assistance information reporting

Option 4: alternate SMTC occasions

- UEs in RRC_IDLE/RRC_INACTIVE

Option 1: location-based SMTC selection

This is quite similar to Option 1 for Connected UE, but the SMTC selection result does not need to be informed to the network.

Option 2: per-SSB SMTC configuration in SIB

Unlike Option 2 for Connected UEs (which mainly depends on network implementation), the network needs to broadcast the SMTC configuration per SSB in the system information.

Option 3: alternate SMTC occasions

This option is quite similar to Option 4 for Connected UEs, but gaps don’t need to be configured to the UE.

Proposal 3: RAN2 to consider the following options for configuring SMTCs for UEs in RRC_IDLE/RRC_INACTIVE:

Option 1: location-based SMTC selection

Option 2: per-SSB SMTC configuration in SIB

Option 3: alternate SMTC occasions

2.2 DTX/DRX

In RAN#106, the revised WID was approved [1]. The system level DL coverage enhancement now only contains SSB periodicity extension, and the description related to R18 NES has been removed:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

Based on the WID, cell/beam DTX/DRX is no longer in the WID scope, and RAN2 discussion should only be initiated if DTX/DRX enhancement is proven to be essential as a result of SSB periodicity extension.

Proposal 4: RAN2 should no longer discuss DTX/DRX enhancement unless it is proven to be essential due to the extension of SSB periodicity.

2.3 Cell Barring

In previous meetings, RAN2 has achieved some agreements on cell barring, with the pre-requisites of “If we need to bar UEs…” “If it turns out that there is a need to bar UEs …”

Agreements @RAN2#128: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.) Agreements @RAN2#127bis: 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) Agreements @RAN2#127: 1. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In RAN1#119 meeting, the following conclusion has been made:

Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

RAN1 agreement is focused on the case of “periodicity larger than 160ms”. In RAN plenary #106 meeting, SSB periodicity larger than 160ms was ruled out.

When SSB periodicity is equal to 160ms, according to the above RAN1 conclusion, some UEs may be able to operate in the cell while some other UEs may not: “Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.”.

From RAN2 perspective, the safest approach is to introduce the signalling for barring legacy UEs and leave it to NW implementation to decide whether to enable the cell barring mechanism.

On the other hand, it was also commented by some companies that the agreement related to cell barring was made before RAN plenary decided to exclude 320ms SSB periodicity, the existing RAN2 spec is already compatible with SSB periodicity equal to 160ms (e.g. in terms of paging, random access, system information etc). If a legacy UE happens to receive the SSB of R19 NTN cell, no backward compatibility issue is foreseen.

Proposal 5: RAN2 to down-select from the following:

Option a) It is up to NW implementation to set cellBarredNTN-r17 to barred to bar UEs not supporting R19 extended SSB periodicity. A new field cellBarredNTN-r19 (value is set to barred or notBarred) is introduced for future use. UEs supporting extended SSB periodicity ignore cellBarredNTN-r17 if the new cellBarredNTN-r19 is present, and follow cellBarredNTN-r17 if the new cellBarredNTN-r19 is absent.

Option b) No cell barring mechanism is pursued in R19 NR NTN.

It is also noted that in the existing 38.331, SIB1 periodicity is either 20ms (multiplexing pattern 1) or the same as SSB periodicity (multiplexing pattern 2/3):

--- cited from TS 38.331 clause 5.2.1 --- - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

Since SSB periodicity is extended to 160ms due to beam hopping, SIB1 periodicity should also be extended. No agreement has been made in RAN1 yet, as RAN1 is now focusing on SIB1 repetition under link level enhancements. We think it can be discussed in RAN2 whether to change the corresponding spec regarding SIB1 periodicity for initial access.

Proposal 6: For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 160ms if SSB periodicity is 160ms.

- Conclusion

This contribution discussed the DL coverage enhancements in Rel-19 NR NTN. Proposals are summarized as follows:

Observation 1: At most one MO can be configured for a single NTN frequency, and configuring the SMTC periodicity to the largest SSB periodicities among neighbour cells on this frequency will largely increase the measurement delay.

Proposal 1: The network should be allowed to configure different periodicities for the multiple SMTCs on a single frequency.

Proposal 2: RAN2 to consider the following options for configuring SMTCs for UEs in RRC_CONNECTED:

Option 1: location-based SMTC selection, FFS UE/NW alignment on SMTC/gaps

Option 2: SSB-based SMTC selection

Option 3: UE assistance information reporting

Option 4: alternate SMTC occasions

Proposal 3: RAN2 to consider the following options for configuring SMTCs for UEs in RRC_IDLE/RRC_INACTIVE:

Option 1: location-based SMTC selection

Option 2: per-SSB SMTC configuration in SIB

Option 3: alternate SMTC occasions

Proposal 4: RAN2 should no longer discuss DTX/DRX enhancement unless it is proven to be essential due to the extension of SSB periodicity.

Proposal 5: RAN2 to down-select from the following:

Option a) It is up to NW implementation to set cellBarredNTN-r17 to barred to bar UEs not supporting R19 extended SSB periodicity. A new field cellBarredNTN-r19 (value is set to barred or notBarred) is introduced for future use. UEs supporting extended SSB periodicity ignore cellBarredNTN-r17 if the new cellBarredNTN-r19 is present, and follow cellBarredNTN-r17 if the new cellBarredNTN-r19 is absent.

Option b) No cell barring mechanism is pursued in R19 NR NTN.

Proposal 6: For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 160ms if SSB periodicity is 160ms.

- References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, THALES, CATT

- R2-2405762, LS on DL coverage enhancements, CMCC

- R1-2407538, Draft Reply LS on DL coverage enhancements, CMCC

- R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei