3GPP TSG-RAN WG2 Meeting #129bis	R2-2502840

Wuhan, China, 7 - 11 April, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the remaining issues of on-demand SSB SCell operation, including MAC CE design and L3 measurements.

Discussion

2.1 MAC CE design

On MAC CE design, in the last meeting RAN2 briefly discussed fixed size vs variable size, explicit SSB periodicity in MAC CE vs configuration index, but no conclusion was made.

Before going to the detailed design, we summarize the RAN1/RAN2 related progress below:

From RAN1 perspective (related RAN1 agreements provided in the Annex):

- OD-SSB activation by MAC CE, at least multiple periodicities can be configured by RRC and indicated by MAC CE. OD-SSB periodicity can be 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms.

- OD-SSB deactivation can be either explicitly indicated by MAC CE, or implicitly indicated by number N of OD-SSB bursts to be transmitted after activation, number N is configured by RRC.

- OD-SSB activated by RRC is expected to be transmitted after UE receives RRC config (based on time location of on-demand SSB burst i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex); OD-SSB activated by MAC CE is expected to be transmitted after a minimal time defined by RAN1, the actual time location still follows RRC configuration.

- RAN1 expects some parameters to be configured by RRC for OD-SSB SCell, including OD-SSB frequency, SSB positions in a burst, OD-SSB periodicity, OD-SSB SCS, PCI, time location of OD-SSB, DL transmit power of OD-SSB, number N for implicit deactivation.

From RAN2 perspective:

- The new MAC CE is not used for SCell activation/deactivation.

- The new MAC CE can indicate OD-SSB transmission for multiple SCells.

- OD-SSB activation/deactivation can be indicated by RRC, including initial status and reconfigured status.

Based on the above progress, we think it is preferred to have multiple set of OD-SSB configurations in RRC and the configuration index is included in MAC CE, to save signalling overhead.

On explicit serving cell index vs implicit serving cell index, we prefer to use implicit index (in the form of a bitmap) because each SCell would take up 5bits.

Also, including all the required information for OD-SSB transmission triggering of up to 31 SCells can make the new MAC CE very big. To save signalling overhead, we think only the SCells supporting OD-SSB should be included in the new MAC CE for OD-SSB transmission. Regarding how to inform the UEs of which SCells supporting OD-SSB, we think it can be indicated in the SCell configuration, or implicitly indicated by the initial status of OD-SSB transmission for SCell (regardless of whether the initial status of OD-SSB is transmitted or not transmitted, the presence of the field indicates the SCell supports OD-SSB).

The following design can be considered:

Variable size MAC CE, bitmap for SCells supporting OD-SSB in increasing order, configuration index is used

RRC signalling can be used to configure an additional index for the SCells supporting OD-SSB (i.e. enabling the OD-SSB feature from UE perspective), and the assumption is that there will not be more than 8 SCells supporting OD-SSB simultaneously. To be specific, a new field od-SSB-SCellIndex can be added to SCellConfig with the value range “INTEGER (1..8)”. And the MAC CE can be designed as follows:

-	Oi: This field indicates the activation/deactivation status of the on-demand SSB transmission of the SCell with Od-SSB-SCellIndex i as specified in TS 38.331 [5]. The Oi field is set to 1 to indicate that the on-demand SSB transmission of the corresponding SCell is activated. The Oi field is set to 0 to indicate that the on-demand SSB transmission of the corresponding SCell is deactivated;

-	Configuration index: This field indicates the configuration of the on-demand SSB of the SCell for which on-demand SSB transmission is activated. The length of the field is 4 bits. The configuration indices are included in ascending order based on the Oi set to 1.

Proposal 1: For OD-SSB MAC CE, RAN2 to consider a variable size MAC CE, in which a bitmap is used for SCells supporting OD-SSB in increasing order, and a configuration index is indicated for each SCell supporting OD-SSB.

The above design assumes for a single UE, all SCells supporting OD-SSB use explicitly deactivation, or all SCells supporting OD-SSB use implicit deactivation. If the mixed case is considered, the MAC CE design will be further complicated. RAN2 can first consider whether there is a need to support a mixed use of explicit deactivation and implicit deactivation for a single UE.

Proposal 2: RAN2 to discuss whether there is a need to support a mixed use of explicit deactivation and implicit deactivation for a single UE.

The new MAC CE requires an LCID or eLCID. Currently we only have 12 reserved values for LCID (values 35-46), while there are plenty of reserved values for one-octet eLCID (values 0 to 215) for DL-SCH. It is preferred to use eLCID for the OD-SSB transmission MAC CE.

Proposal 3: Use an eLCID for the OD-SSB transmission MAC CE.

2.2 L3 measurements

There are several open issues on L3 measurements:

Issue 1: Whether OD-SSB (on-demand SSB) is on the same frequency with AO-SSB (always-on SSB) or not

Issue 2: Whether OD-SSB measurement is based on SMTC or OD-SSB pattern

On Issue 1, RAN1 has started discussing both intra-frequency and inter-frequency cases, and an LS is sent to RAN4 cc RAN2 [1]:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB NOTE: AO-SSB periodicity is not adapted

RAN4 discuss has started from intra-frequency scenario in the past meetings. From RAN2 perspective, we would also like to prioritize intra-frequency case, because inter-frequency case would bring considerable RAN2 impacts:

1) Impacts on servingCellMO configuration: In existing spec, each serving cell can be configured with a servingCellMO for deriving serving cell measurements, and only one SSB frequency is configured per MO. If both OD-SSB and AO-SSB are used for serving cell measurements, either two servingCellMOs are considered, or two SSB frequencies are configured in the single servingCellMO. Also, the serving cell CSI-RS measurement is based on an associated SSB, it is also unclear whether it is based on OD-SSB or AO-SSB.

2) Impacts on neighbour cell measurements: In existing spec, whether an SSB based measurement is considered as intra-frequency or inter-frequency measurement depends on whether the neighbour cell SSB is of the same frequency with the SSB of serving cell, and whether the SCS is the same as serving cell SSB. If both OD-SSB and AO-SSB are configured for serving cell measurements, RAN2 needs to clarify whether OD-SSB or AO-SSB will be used as baseline to identify intra-frequency/inter-frequency measurements.

3) Impacts on gap configuration: In existing spec, other than the cases where UE reports gap requirement (e.g. via NeedForGapsInfoNR reporting), if any of the configured BWPs do not contain serving cell SSB (which refers to the SSB of servingCellMO) and are not configured with NCD-SSB for serving cell measurement (which was introduced in RedCap), the network should configure gaps to the UE for intra-frequency measurements. However, if the SCell can be configured with AO-SSB and an inter-frequency OD-SSB, it is unclear whether gaps are needed if a BWP contains AO-SSB but not OD-SSB.

- For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case: - Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement.

Proposal 4: RAN2 starts discussion on L3 measurement from intra-frequency scenario (OD-SSB and AO-SSB on the same SSB carrier frequency).

Proposal 5: For inter-frequency sceanrio, the following issues need to be addressed:

- Impacts on servingCellMO configuration (whether both OD-SSB and always-on SSB frequencies are configured)

- Impacts on neighbour cell measurements (whether intra-frequency/inter-frequency determination can be based on OD-SSB)

- Impacts on gap configuration for intra-frequency measurements (whether gaps are needed if a BWP contains always-on SSB but not OD-SSB)

On Issue 2 (Whether OD-SSB measurement is based on SMTC or OD-SSB pattern):

In legacy mechanism, L3 measurement is based on the measConfig, which uses measIds to link measObjects (which provide SMTC for UE to monitor SSB) and reportConfigs, and UE measurement reporting is also based on measIds. For OD-SSB, at least the OD-SSB pattern of the serving cell will be informed to the UE. Therefore, RAN2 needs to determine whether the L3 measurement is still based on SMTC or it is based on the OD-SSB pattern or both.

For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), as long as the OD-SSB pattern is informed to the UE, UE can perform serving cell measurements. There is no need to modify the legacy SMTC or make up a new dedicated SMTC for serving cell measurement purpose. If the specification forces the NW implementation to configure an SMTC for OD-SSB measurements, this configuration is anyway aligned with the OD-SSB pattern, which is already known to the UE, and an unnecessary step is defined at the NW side.

It was raised by some companies that reusing the existing SMTC framework minimizes RAN4 impacts. From our perspective, it is up to RAN4 to decide whether the spec impact is acceptable, for instance RAN4 is also discussing replacing the SMTC periodicity in the existing requirements with OD-SSB periodicity, so the measurements of OD-SSB can be based on OD-SSB pattern rather than SMTC. Besides, even if the OD-SSB measurement is based on SMTC, the existing framework also needs to be modified, considering the following open issues:

- If AO-SSB needs to be measured as well, the SMTC may not cover both OD-SSB and AO-SSB;

- If the configured SMTC window does not fit OD-SSB, it needs to be updated/reconfigured, additional signalling design is needed;

- The measurement reporting procedure anyway needs to be modified, as the OD-SSB can be denser than normal SSB and it is not appropriate to use legacy events like A3/A5 to compare the measurement results with neighbour cells with normal SSB.

Observation 1: The existing measurements framework needs to be modified anyway, regardless of whether OD-SSB pattern or SMTC is used for OD-SSB measurements.

Proposal 6: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC.

For neighbour cells measurements, we think on-demand SSB status of neighbour cells should be transparent to the UE (otherwise large signalling overhead in Uu interface and Xn interface are incurred), and it is up to gNB implementation how to handle the reported neighbour cell measurement results which could be degraded based on the intermittent SSB transmission status of neighbour cell.

Proposal 7: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

In RAN4#112bis, RAN4 agreed to introduce fast L3 measurements for deactivated SCell:

Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

In RAN4#113, RAN4 further agreed that in fast mode the measurement periodicity is based on OD-SSB periodicity rather than measCycleSCell:

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

However, the fast measurement of deactivated SCell cannot be guaranteed, if the UE is configured with multiple MOs. In legacy spec (earlier than R18), it is up to UE implementation which MO if measured first if multiple MOs are configured. In R18, the field measSequence was introduced to solve the unpredictable sequence of inter-frequency measurement.

To satisfy RAN4’s need of fast deactivated SCell measurement, the SCell’s servingCellMO needs to be prioritized. But setting the sequence as 1 in measSequence of the servingCellMO may not work. First, there may be multiple MOs configured with the same meaSequence (e.g. set to 1, the highest priority), it is still up to UE implementation to measure which one. Second, the measSequence is configured in the MO, while the MO of the deactivated may not always be prioritized (e.g. only prioritized when OD-SSB is triggered).

Proposal 8: RAN2 to discuss enabling the UE to prioritize deactivated SCell measurements when OD-SSB of that SCell is triggered.

2.3 Measurement reporting

In the existing spec, the report types of RRM measurements include the following:

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

Event triggered reporting and periodical reporting are widely used to trigger serving/neighbour cell measurement reports.

For OD-SSB based SCell measurements, considering OD-SSB is transmitted in a limited period (e.g. from time instance A to time instance B), and there could be no intra-frequency neighbour cell SSBs during the period or there are fewer SSB occasions of neighbour cells compared with serving cell OD-SSB, it is difficult to define a proper report interval.

Looking at the existing RRM measurement events, event A1 and A2 only concern serving cell, and can be used for triggering OD-SSB measurement reporting. For event A3 and A5, the neighbour cell measurements need to be compared with serving cell measurements to determine whether the event is triggered. Considering the different characteristics of serving cell OD-SSB (which is denser, in a confined period of time) and neighbour cell always-on SSB (which is sparser), it is unclear whether A3 and A5 can be triggered properly.

Proposal 9: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: For OD-SSB MAC CE, RAN2 to consider a variable size MAC CE, in which a bitmap is used for SCells supporting OD-SSB in increasing order, and a configuration index is indicated for each SCell supporting OD-SSB.

Proposal 2: RAN2 to discuss whether there is a need to support a mixed use of explicit deactivation and implicit deactivation for a single UE.

Proposal 3: Use an eLCID for the OD-SSB transmission MAC CE.

Proposal 4: RAN2 starts discussion on L3 measurement from intra-frequency scenario (OD-SSB and AO-SSB on the same SSB carrier frequency).

Proposal 5: For inter-frequency sceanrio, the following issue needs to be addressed:

- Impacts on servingCellMO configuration (whether both OD-SSB and always-on SSB frequencies are configured)

- Impacts on neighbour cell measurements (whether intra-frequency/inter-frequency determination can be based on OD-SSB)

- Impacts on gap configuration for intra-frequency measurements (whether gaps are needed if a BWP contains always-on SSB but not OD-SSB)

Observation 1: The existing measurements framework needs to be modified anyway, regardless of whether OD-SSB pattern or SMTC is used for OD-SSB measurements.

Proposal 6: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC.

Proposal 7: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

Proposal 8: RAN2 to discuss enabling the UE to prioritize deactivated SCell measurements when OD-SSB of that SCell is triggered.

Proposal 9: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

References

- R1-2501633, LS on time location of on-demand SSB for SCell, RAN1 (contact company: LG)

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

Annex – related RAN1 agreements

@RAN1 118:

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB.

- FFS: Any other relevant parameters.

@RAN1 118bis:

Agreement

The previous RAN1 agreement is partly confirmed and further revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

      - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - (Working assumption) T=T_min

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

@RAN1 119:

Agreement

- New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

@RAN1 120:

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list